3GPP TSG-RAN WG4 Meeting #112	R4-2412352
Maastricht, Netherlands, 19th – 23rd August, 2024

Title:	R19 Simulation results of 6Layer performance

Source:	OPPO

Agenda item:	8.1.1.3.2

Document for:	Approval

- Introduction

6Layer was discussed in last meeting and captured in WF [1]. This paper studies the 6Layer performance in handheld UE with real antenna parameters.

- Discussion

2.1 Ant correlations

ECC is evaluated in a commercial foldable handheld UE with antennas located as shown in figure 1. And the ECC values are as shown in table 1.

Figure 1 power boosting by changing ACLR/SEM to IBE

Table 1 ECC values of the 6 antennas

2.2 Performance comparison of 6Rx and 4Rx

Based on these ECC values, link level simulations have been done to evaluate the performance of 6Layers. And two scenarios have been evaluated, one is TDL channel model, and the other is CDL channel model. Detailed simulation assumptions are in Table 2 and 3.

Table 2 TDL LLS assumptions

| Parameters | Value |
| --- | --- |
| Duplexing | TDD |
| Carrier frequency | 3.5GHz |
| Subcarrier spacing | 30kHz |
| Propagation condition | TDL-A with DS=30 in TR 38.901 |
| gNB antenna parameter | 8Tx with ideal non-correlation (beta=0) |
| UE antenna parameter | 4Rx/6Rx |
| Digital precoding | Type I codebook, PRG=4 |
| CSI-RS periodicity | 40ms |
| DMRS type | Type 2 |
| Number of DMRS symbols | 1 in symbol index 2 |
| PDSCH mapping | Type A, Starting symbol index 3, Duration 11 |
| Bandwidth | 8RBs |
| MCS | Link adaptation (up to 64QAM) |
| Rank | Rank adaptation 1-4 for 4Rx, Rank adaptation 1-6 for 6Rx |
| UE speed | 3km/h |

The throughput comparisons of 4Rx and 6Rx in TDL channel model can be found in figure 2.

Figure 2 TDL channel model: 4Rx vs 6Rx

From the results it can be seen that 6Rx performance is better than 4Rx in almost all the SNR range, and below 20dB the MIMO layer is 4Layer, then above 20dB 5/6Layers are enabled and its performance is better than 4Layer+2Diversity.

Table 2 CDL LLS assumptions

| Parameters | Value |
| --- | --- |
| Duplexing | TDD |
| Carrier frequency | 3.5GHz |
| Subcarrier spacing | 30kHz |
| Propagation condition | CDL-C with DS=100 in TR 38.901 |
| gNB antenna parameter | 32Tx: (M, N, P, Mg, Ng, Mp, Np) = (2,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| UE antenna parameter | 4Rx: (M, N, P, Mg, Ng, Mp, Np) = (1,2,2,1,1,1,2), 6Rx: (M, N, P, Mg, Ng, Mp, Np) = (1,3,2,1,1,1,3), |
| Digital precoding | Type I codebook, PRG=4 |
| CSI-RS periodicity | 40ms |
| DMRS type | Type 2 |
| Number of DMRS symbols | 1 in symbol index 2 |
| PDSCH mapping | Type A, Starting symbol index 3, Duration 11 |
| Bandwidth | 8RBs |
| MCS | Link adaptation (up to 256QAM) |
| Rank | Rank adaptation 1-4 for 4Rx, Rank adaptation 1-6 for 6Rx |
| UE speed | 3km/h |

The throughput comparison of 4Rx and 6Rx in CDL channel model can be found in figure 3.

Figure 3 CDL channel model: 4Rx vs 6Rx

Similar as TDL channel model, 6Rx has better performance than 4Rx in almost all SNR ranges, and 6Layer has better performance than 4Layer+2Div when SNR is higher than 30dB in CDL channel model.

Observation 1:   6Layer have better performance than 4Layer+2Div when SNR is higher than 20dB in TDL channel model and 30dB in CDL channel model.

Proposal 1:         Introduce 6Layers as optional feature for UE which supports 6Rx antennas.

- Conclusion

This paper analyzed the 6Layer throughput performances with real handheld UE antenna correlations in both TDL and CDL channels, and got observations and proposals as below.

Observation 1:   6Layer have better performance than 4Layer+2Div when SNR is higher than 20dB in TDL channel model and 30dB in CDL channel model.

Proposal 1:         Introduce 6Layers as optional feature for UE which supports 6Rx antennas.

- Reference

[1] R4-2410751 WF on requirements for 6Rx, AT&T, RAN4#111 2024-05