3GPP TSG-RAN WG4 Meeting #112	R4-2412354
Maastricht, Netherlands, 19th – 23rd August, 2024

Title:	RAN task R17 power class applicability related

Source:	OPPO

Agenda item:	10.2

Document for:	Approval

- Introduction

The power class ambiguity issue has been discussed for a long time and in RAN4#111 meeting the group decided to stop the discussion unless there is clear offline consensus. And then in RAN#104 meeting, a company paper was submitted to further pursue this topic, especially for the DL CA only case and would like to enable HPUE in DL CA with single UL CC case. And RAN task is given to RAN4 as below [1].

Way Forward - RAN tasks RAN4 to investigate how to enable HPUE maximum transmit power in downlink CA with a single configured UL. - RAN task shall conclude by RAN#106 - RAN4 online meeting time should be limited to <0.1 TUs

This paper further discusses this issue.

- Band in a BC is capped by powerClass of the BC

Although it has been discussed a lot in the past, it is clear that any band in a band combination shall be capped by the powerClass capability of the band combination.

Observation 1:  Band in a BC is capped by powerClass capability of the BC

- How to enable HPUE for DL CA with UL single CC

3.1 UE hardware capability

This is known as DL-only CA scenario in the power class discussions. And as we pointed before, from PA ability perspective there is no difficulty to keep same Tx power as the single band power class. The only issue with the added CCs in DL is whether MSD is needed to keep the Tx power. However, this will not change the Tx power but only impact the REFSENS with MSD as long as it has specified and supported. This can be covered by the agreed CR [2], i.e. if the MSD has not been specified, the higher power class is not supported by spec for certain band combination.

Observation 2:   From PA ability perspective, there is no difficulty to keep same Tx power as the single band power class as long as the MSD if any has been introduced in the spec.

3.2 Signalling aspects

From signalling perspective, the powerClass IE is used to indicate the power capability of a band combination, and the DL-only CA case is also covered by this IE.

If UE in single band is HPUE, e.g. PC2, then when DL CA is configured, this UE can indicate PC2 capability also in powerClass IE. This will allow UE to transmit HPUE in both single band and CA.

Therefore, to solve the RAN task issue, actually current capability signalling also support it. No additional solution is needed as long as the MSD is specified in the spec and UE indicate HPUE in the powerClass IE for this band combination.

Observation 3:   From signalling perspective, powerClass IE can be used to indicate HPUE for DL-only CA case, no new signalling is needed to support the RAN task targeted scenario.

Proposal:   	Reply to RAN plenary that, for DL-only CA case, as long as UE indicate HPUE power class in both powerClass IE and uepowerClass IE, and the corresponding MSD requirements have been specified in RAN4 spec, then it can support HPUE. No new solution is needed.

- References

- RP-241687, Moderator's summary for offline discussion on CA Power Class Support, RAN vice-chair, RAN#104 2024-06

- R4-2314915, CR for TS38101-1 Clarifying applicable power classes for NR CA, Huawei, RAN4#108, Aug 2023