3GPP TSG-RAN4 Meeting #112	R4-2412357

# Maastricht, NL, 19th Aug 2024 – 23rd Aug 2024

Source:	LG Electronics

Title:	Discussion on Tx requirements for NTN HPUE

Agenda item:	8.8.2.2

Document for:	Discussion

Introduction

We would like to share our perspective on the Tx requirements of Rel-19 NTN HPUE.[1]

Table 1 Objective of Rel-19 HPUE NTN [1]

High power UE (HPUE) for NTN - Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes - Evaluate the impact on the uplink performance of TN (Terrestrial network) - Example bands include bands n256 and 256, n255 and 255 - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18 - Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study. - Support other power classes based on the co-existence and feasibility study - Non-hand held UE is prioritized - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands - Necessary Tx requirements, at least including - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary - Specify the necessary signaling to support the above objectives

Discussion

In Rel-17, the requirements for PC3 NTN have been specified in TS 38.101-5. The NTN UE RF requirements defined in Rel-17 were specified assuming that NTN UE has a similar form factor to a TN  UE, therefore the legacy RF requirements were reused as much as possible to specify the NTN UE RF requirements. In Rel-17 NTN Tx requirement, the biggest difference between NTN UE and legacy UE is frequency error requirement, and frequency error pre-compensation has been added for NTN UE Tx requirement.

The Tx requirement for Rel-19 NTN HPUE are also expected to reuse the TN HPUE Tx requirement as much as possible. However, some requirements need technical discussion to be specified. The following are the Tx requirements for NTN HPUE that need to be discussed.

- ACLR – Up to the co-existence study

- MPR  – If the NTN HPUE ACLR value is different from FR1 HPUE, MPR simulation is needed to specify the MPR requirement for NTN HPUE

- A-MPR – Regardless of ACLR difference based on co-existence study, A-MPR is needed for NTN HPUE

Proposal 1: The ACLR, MPR, and A-MPR requirements for NTN HPUE need technical discussion

- ACLR – Up to the co-existence study

- MPR  – If the NTN HPUE ACLR value is different from FR1 HPUE, MPR simulation is needed to specify the MPR requirement for NTN HPUE

- A-MPR – Regardless of ACLR difference based on co-existence study, A-MPR is needed for NTN HPUE

The co-existence study objective related to NTN-HPUE's power class includes PC2, PC1.5, and PC1. However, the NTN-HPUE spec requirement objective only includes PC2. It needs to be checked whether the co-existence study objective include PC2, PC1.5, and PC1, while the requirement objective only includes PC2.

Proposal 2: It needs to be checked whether the co-existence study objective include PC2, PC1.5, and PC1, while the requirement objective only includes PC2.

Conclusion

Proposal 1: The technical discussion on ACL, MPR and A-MPR need to consider following aspects

- ACLR – Up to the co-existence study

- MPR  – If the NTN HPUE ACLR value is different from TN  HPUE, MPR simulation is needed to specify the MPR requirement for NTN HPUE

- A-MPR – Regardless of ACLR difference based on co-existence study, A-MPR is needed for NTN HPUE

Proposal 2: It needs to be checked whether the co-existence study objective include PC2, PC1.5, and PC1, while the requirement objective only includes PC2.

- References

# [1] RP-241281, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN (Samsung)