3GPP TSG-RAN WG4 Meeting #112	R4-2412365

Maastricht, Netherlands, 19th August – 23th August 2024

Source:	Ericsson, BT plc

Title:	TP for TR 38.792: 3Tx inter-band CA_n1-n78

Agenda item:			7.8.3

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for 38.792 adding 3Tx PC2 and PC1.5 UL CA_n1A-n78A to DL CA_n1A-n78A.

# 2. Text Proposal

# ---Start of changes---

## 5.x	CA_n1-n78

### 5.x.1	Operating bands for CA

Table 5.x.1-1:  CA band combination of band n1+n78

| NR CA Band combination | NR Band |
| --- | --- |
| CA_n1-n78 | n1, n78 |

### 5.x.2	Maximum output power for inter-band CA

Table 5.x.2-1: UE Power Class for 3Tx inter-band CA with UL MIMO

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n78A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC3 or PC2 with UL MIMO in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with UL MIMO in TDD band. NOTE 4: The UE supports PC2 with UL MIMO in either one of the TDD bands and PC2 in the other TDD band. |  |  |  |  |  |  |

Table 5.x.2-2: UE Power Class for 3Tx inter-band CA with TxD

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n78A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC2 with Tx Diversity in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with Tx Diversity in TDD band. |  |  |  |  |  |  |

### 5.x.3	MSD scenario studies

Table 5.x.3-1 and 5.x.3-2 summarizes frequency ranges where harmonics mixing, cross band leakage, IMD interferences occur for CA_n1-n78.

Table 5.x.3-1: MSD scenarios for CA_n1-n78 with total power class 2

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | N/A | N/A | N/A |
| Cross band leakage | N/A | N/A | N/A |
| IMD4 | 3 * n1 – n78 | n1 | Yes |

Table 5.x.3-2: MSD scenarios for CA_n1-n78 with total power class 1.5

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | N/A | N/A | N/A |
| Cross band leakage | N/A | N/A | N/A |
| IMD4 | 3 * f n1 – f n78 | n1 | No |

### 5.x.4	REFSENS requirements

#### 5.x.4.1	REFSENS requirements for total power class 2

- The 2Tx MSD due to IMD4 has already been specified, which is applicable for 3Tx.

#### 5.x.4.2	REFSENS requirements for total power class 1.5

- The MSD due to IMD4 for PC1.5 3Tx are specified in the following table. The IMD4 MSD values for CA_n1-n78 and CA_n2-n77 are the same. Therefore, the CA_n1-n78 PC1.5 MSD values are reused from CA_n2-n77 PC1.5 MSD values.

Table 5.x.4.2-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC1.5 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band |  | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n78 | n1 |  | 1950 | 5 | 25 | 2140 | 26.4 | FDD | IMD4 |
|  | n78 | 3710 |  | 10 | 50 | 3710 | N/A | TDD | N/A |

---End of changes---

# Reference

[1]		RP-241679, New WID: Rel-19 High power UE (power class 1.5 or 2) for NR intra-band Carrier Aggregation (CA) or NR inter-band CA/Dual connectivity (DC) band combinations with/without NR SUL (supplementary uplink)