3GPP TSG-RAN WG4 Meeting #112                                                                                        R4-2412381

Maastricht, NL, Aug. 19 – Aug. 23, 2024

Source: 	CATT

Title: 	Discussion on system parameters for NTN support spectrum less than 5 MHz

Agenda item:	8.8.3.1

Document for:	Approval

- Introduction

In RAN #104 meeting, the work item for enhanced requirements and test methodology for NR and IoT NTN was approved [1]. Part of the objective is shown as follows:

Less than 5MHz for NTN - Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands - Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Focus on power class 3 (PC3) for Tx RF requirements - Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary signaling to support the above objectives

In this contribution, we would like to share some initial views on system parameters for support spectrum less than 5 MHz.

- Discussion

2.1 Transmission bandwidth configuration

Per work item objectives, the intention of NTN support spectrum less than 5 MHz is to introduce the 3 MHz channel bandwidth designed in Rel-18 for NR NTN FR1 operating band n256, n255 and n254. Regarding to the Rel-18 3 MHz channel bandwidth design for NR TN network, the transmission bandwidth configuration for NTN support 3 MHz channel bandwidth should be defined as illustrated in the table below.

Table 2.1-1: Maximum transmission bandwidth configuration NRB for FR1-NTN

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 30 MHz |
| --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 15 | 25 | 52 | 79 | 106 | 160 |
| 30 | N/A | 11 | 24 | 38 | 51 | 78 |
| 60 | N/A | N/A | 11 | 18 | 24 | 38 |

Proposal 1: For NTN support 3 MHz channel bandwidth in FR1, the transmission bandwidth configuration should be defined as Table 2.1-1.

2.2 Minimum guardband

Regarding to the Rel-18 3 MHz channel bandwidth design for NR TN network, the minimum guardband for NTN support 3 MHz channel bandwidth should be defined as illustrated in the table below:

Table 2.2-1: Minimum guardband for each channel bandwidth and SCS (kHz) for FR1-NTN

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 30 MHz |
| --- | --- | --- | --- | --- | --- | --- |
| 15 | 142.5 | 242.5 | 312.5 | 382.5 | 452.5 | 592.5 |
| 30 | N/A | 505 | 665 | 645 | 805 | 945 |
| 60 | N/A | N/A | 1010 | 990 | 1330 | 1290 |

Proposal 2: For NTN support 3 MHz channel bandwidth in FR1, the minimum guardband should be defined as Table 2.2-1.

2.3 Channel bandwidth per operating band

According to the work item [1], the channel bandwidth per operating band should be updated as well. Following the work item description, the 3 MHz channel bandwidth could be introduced for band n256, n255 and n254. Therefore, the channel bandwidth per operating band should be defined as illustrated in the table below:

Table 2.3-1: Channel bandwidths and SCS per operating band in FR1

| NTN satellite Band | SCS (kHz) | Channel bandwidth (MHz) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 30 (NOTE) |
|  | 15 | 3 | 5 | 10 | 15 | 20 |  |
| n256 | 30 |  |  | 10 | 15 | 20 |  |
|  | 60 |  |  | 10 | 15 | 20 |  |
|  | 15 | 3 | 5 | 10 | 15 | 20 |  |
| n255 | 30 |  |  | 10 | 15 | 20 |  |
|  | 60 |  |  | 10 | 15 | 20 |  |
|  | 15 | 3 | 5 | 10 | 15 |  |  |
| n254 | 30 |  |  | 10 | 15 |  |  |
|  | 60 |  |  | 10 | 15 |  |  |
|  | NOTE: Deployment of 30 MHz channel bandwidth for NTN needs to be preceded by introduction of all applicable Tx RF, Rx RF, and demodulation requirements. |  |  |  |  |  |  |

Proposal 3: For NTN support 3 MHz channel bandwidth in FR1, the channel bandwidth and SCS per operating band should be defined as Table 2.3-1.

2.4 Synchronization raster

For NTN supports 3 MHz channel bandwidth, the same approach for defining synchronization raster as TN networks can be considered. Since the 3 MHz channel bandwidth will be introduced for the band n256, n255 and n254, the range of frequencies should be 0 – 3000 MHz. For SSB frequency position SSREF, the same method of calculation for TN 3 MHz channel bandwidth could be reused as well. Therefore, the sync raster of NTN support 3 MHz could be defined as shown in following table.

Table 2.4-1: GSCN parameters for the global frequency for 3 MHz channel bandwidth

| Range of frequencies (MHz) | SS block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 0 – 3000 | N * 600 kHz + M * 50 kHz + 300 kHz, N = 1:4999, M ϵ {1,3,5} (Note 1) | 26638+3N + (M-3)/2 | 26640 – 41636 |

Table 2.4-2: Applicable SS raster entries per operating band for 3 MHz channel raster (FR1-NTN)

| NTN satellite operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 15 kHz | Case A | 37492 – <1> – 37629 |
| n255 | 15 kHz | Case A | 34267 – <1> – 34424 |
| n254 | 15 kHz | Case A | 39060 – <1> – 39129 |
| NOTE : SS Block pattern is defined in clause 4.1 in 3GPP TS 38.213 [7]. |  |  |  |

Proposal 4: For NTN support 3 MHz channel bandwidth in FR1, the range of frequencies should be defined as 0 – 3000 MHz.

Proposal 5: For NTN support 3 MHz channel bandwidth in FR1, the synchronization raster and GSCN per operating band should be defined as Table 2.4-1 and Table 2.4-2.

- Conclusion

This contribution provides our initial views on system parameters for introduction of the NTN new S-band. The following proposals are concluded as follows:

Proposal 1: For NTN support 3 MHz channel bandwidth in FR1, the transmission bandwidth configuration should be defined as Table 2.1-1.

Proposal 2: For NTN support 3 MHz channel bandwidth in FR1, the minimum guardband should be defined as Table 2.2-1.

Proposal 3: For NTN support 3 MHz channel bandwidth in FR1, the channel bandwidth and SCS per operating band should be defined as Table 2.3-1.

Proposal 4: For NTN support 3 MHz channel bandwidth in FR1, the range of frequencies should be defined as 0 – 3000 MHz.

Proposal 5: For NTN support 3 MHz channel bandwidth in FR1, the synchronization raster and GSCN per operating band should be defined as Table 2.4-1 and Table 2.4-2.

- References

- RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN, RAN4 chair (Huawei)