3GPP TSG-RAN WG4 Meeting # 112                                R4-2412384

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	5.24.1

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on L1/L2 based inter-cell mobility

Document for:	Approval

Introduction

In this contribution we discuss the remaining open issues for mobility core part requirements based on the RAN4#111 way forward [1].

Discussion

In this contribution, we will provide our views on some issues based on the WF [1] which agreed in last meeting.

2.1 LTM - general aspects and scenarios

Issue 1-3-1: Whether to consider early TCI state activation for multiple cells at the same time - Proposals - Option 1 (Huawei, vivo, Nokia, MTK): - Early TCI state activation delay requirements to be defined for one or more TCI states for a single candidate cell, because one MAC-CE activates TCI states only for a single candidate cell. - Option 2 (Ericsson, QC): - If all the target LTM TCI states in the active TCI state list are known, if the UE receives TCI state activation command at slot n, UE shall have completed the LTM TCI state list update in slot n + THARQ +3Nslotsubframe,µ + TOk*(Tfirst-SSB_List + TSSB-proc) / NR slot length. - If any of the target TCI states in the active TCI state list are unknown, if the UE receives TCI state activation command at slot n, UE shall have completed the LTM TCI state list update in slot n+ THARQ +3Nslotsubframe,µ + (TL1-RSRP_list +TOuk*(Tfirst-SSB_List+ TSSB-proc)) / NR slot length. - In the TCI state activation requirements for LTM candidate cells, T L1-RSRP = 0 for FR1 - In the TCI state activation requirements for LTM candidate cells, Tfirst-SSB_List is given by - For FR1, Tfirst-SSB_List = max (Tfirst-SSB_LTM1, Tfirst-SSB_LTM2, .. , Tfirst-SSB_LTMn). - For FR2, - Tfirst-SSB_List = Tfirst-SSB_LTM1 + Tfirst-SSB_LTM2 + ... +Tfirst-SSB_LTMn, if the time to first SSB associated to LTM candidate TCI states are overlapped in FR2. - Tfirst-SSB_List = max (Tfirst-SSB_LTM1, Tfirst-SSB_LTM2, .. , Tfirst-SSB_LTMn.) if the time to first SSB associated to LTM candidate TCI states are not overlapped. - Where, the Tfirst-SSB_LTMn is the SSB periodicity of LTM candidate cell n. < Way Forward > - Early TCI state activation delay requirements are defined for one or more TCI states for a single candidate cell.

- For this issue, we should discuss whether to consider early TCI state activation for multiple cells at the same time. RAN2 has reached consensus that early TCI state activation for multiple candidate cells, but each TCI state activation via MAC-CE is executed independently in previous meeting. That’s to say, one MAC-CE activates TCI states only for a single candidate cell. With this information, early TCI state activation for multiple cells at the same time only comes from the scenario with multiple time-overlapping MAC CE commands. As mentioned by other company, it is not a typical configuration that network simultaneously indicates multiple LTM TCI activation MAC CE for multiple cells. Based on the above analysis, we prefer not to specify requirements for the scenario with multiple time-overlapping MAC CE commands.

- Proposal 1: Early TCI state activation delay requirements to be defined for one or more TCI states for a single candidate cell, because one MAC-CE activates TCI states only for a single candidate cell.

Issue 1-3-2: The definition of Tfirst-SSB in early candidate cell’s TCI state activation delay for inter-frequency with gap for known TCI state case < Agreement>: - In early candidate cell’s TCI state activation delay for known TCI state case: Tfirst-SSB is the time to the first SSB occasion overlapped with MGL after slot n + THARQ +3Nslotsubframe,µ + [2ms] if the SSB needs to be measured with MG. - FFS: this is only applicable to UE supporting inter-frequency L1 measurement with MG. - Same agreement applies to the definition of Tfirst-SSB in PDCCH order RACH delay requirement.

- In the current specification, the description of Tfirst-SSB in MAC-CE based TCI state switch delay can be used as baseline, that is, Tfirst-SSB is time to first SSB transmission after MAC CE command is decoded by the UE. Different from legacy, we should consider the cases of SSB being within and outside active BWP for L1-RSRP measurement. Therefore, the value of Tfirst-SSB can be divided into two parts. It’s common understamding that Tfirst-SSB is the time to first SSB occasion after slot n + THARQ +3Nslotsubframe,µ when SSB is within the active BWP, and TSSB is the time to first SSB occasion after slot n + THARQ +3Nslotsubframe,µ when SSB is outside the active BWP.

- For this issue, there remains a question that the first bullet is only applicable to UE supporting inter-frequency L1 measurement with MG. In our understanding, it’s reasonable. In last meeting, some companies proposed to add additional time to execute some adjustments related to MG. If there is no misunderstanding of the intention, we think that [2ms] only applies to UE supporting inter-frequency L1 measurement with MG is reasonable. Meanwhile, for UE supporting intra-frequency and inter-frequency L1 measurement without MG, Tfirst-SSB is the time to first SSB occasion after slot n + THARQ +3Nslotsubframe,µ.

Proposal 2: In early candidate cell’s TCI state activation delay for known TCI state case: Tfirst-SSB is the time to the first SSB occasion overlapped with MGL after slot n + THARQ +3Nslotsubframe,µ + [2ms] if the SSB needs to be measured with MG, which is only applicable to UE supporting inter-frequency L1 measurement with MG.

Issue 1-3-3: Whether and how to support unknown TCI state in FR2 <Way Forward >: Further discuss the following options: - Option 1 (vivo): - RAN4 further discuss whether the scenario below is an important scenario in FR2, and whether RRM requirements need to be specified. - Based on the agreed big CR, and the requirement applicability rules for L1-RSRP measurements, LTM TCI activation delay requirements are not applicable to the case when activated TCI state(s) of one candidate cell already exist, but gNB activates one more unknown TCI state from another candidate cell on top of activated TCI state(s) from the existing candidate cell. - Option 2 (Huawei, MTK): - Not to define requirements of SSB based TCI state activation delay for FR2 unknown TCI state case. - Option 3 (Nokia): - When UE is performing and reporting L1 measurements for LTM candidate cells, unknown TCI state activation delay may follow the legacy requirement. - When UE is not performing L1 measurements and LTM decision is based on L3 measurements (if supported), L1-RSRP measurement period in legacy unknown TCI state activation delay can be replaced with L3-RSRP measurement period. - Option 4 (Ericsson, QC): In the TCI state activation requirements for LTM candidate cells, for FR2, T L1-RSRP is the time for Rx beam refinement in FR2, defined as - TL1-RSPR_Measurement_Period_SSB for SSB as specified in clause 9.14 and 9.15, - with the assumption of M=1 - with TReport = 0 - NNeighbor_Cell is the number of neighbour cells that are to be activated with TCI states

In FR1, it was agreed to support both known TCI state and unknown TCI state in FR1 in RAN4#110bis meeting. To avoid the case that network blindly activates one candidate TCI state from neighbor cell as it has no any information of the TCI, side condition was added in RAN4#111 meeting that UE has reported beam-level L3 measurement result of the associated SSB of the TCI state within [1280ms or 5 seconds] before the LTM TCI state activation command and the SNR of the associated SSB is above -3dB.

In FR2, there is no concensus on whether to consider unknown TCI state. In last meeting, some companies provided their views on this topic and there are different views we can observe from above. Companies opposing the consideration of unknown TCI states in FR2 think that NW will not blindly activate a candidate TCI state from neighbor cell without any information. We agree with this point, but in our understanding, similar to FR1, it is feasible to constrain reporting measurements from a prior period. In other words, we are fine to restrict the unknown TCI state activation only when there is a measurement report within certain time.

Another concern is UE needs to sweep RX beam to perform L1-RSRP measurement in FR2 if the candidate TCI state is unknown, the activation delay maybe long. For this issue, we need further discuss how to short delay, i.e. reduce the number of RX beam sweeping .

Proposal 3: Unknown TCI state in FR2 for early TCI state activation is supported with the following conditions:

  - UE has reported measurement result of the associated SSB of the TCI state within [TBD] before the LTM TCI state activation command.

  - SNR of the associated SSB is above -3dB.

2.2 LTM - L1-RSRP measurement requirements

Issue 2-2-1: Measurement period of serving cell L1-RSRP measurement <Way Forward >: Further discuss the following options: - Option 1 (Apple): - introduce NLayer in serving cell L1 RSRP measurement requirement and clarify that it is for UE capable of RTD>CP configured with L1 RSRP measurement on neighbour cell. - Option 2 (OPPO): - Consider to revisit the agreements for L1 RSRP measurement on neighbour cell, e.g., either follow the logic of serving cell L1-RSRP measurement or L3 intra-frequency measurement. - Option 3 (Ericsson, QC): - RAN4 not to modify the serving cell measurement period by scaling it with NLayer for UE supporting RTD > CP.

For this issue, we know the intention of proponet. In current spec., L1-RSRP measurement period is different between serving cell and neighbor cell. Specifically, current measurement period requirement for UE supporting RTD>CP, there is a scaling factor NLayer for neighbour cell, but in serving cell measurement there is no such scaling factor. As mentioned by proponet, measuring SSB from serving cell or neighbour cell are quite similar from physical layer point of view and similar design would be beneficial to UE implementation.

However, we think serving cell is more important, NW may not need the L1 measurement from neighbour cell as it is needed from the serving cell. Hence, we think we do not need to scale the serving cell measurements with NLayer. Therefore, we prefer to support option3.

Proposal 4: RAN4 not to modify the serving cell measurement period by scaling it with NLayer for UE supporting RTD > CP.

2.3 LTM - Cell switch delay requirements

Issue 3-1-2-1: Extra time for PL-RS measurement Ad hoc agreement < Agreement>: - No additional PL-RS measurement time is needed, provided - Option 1: L3-RSRP or L1-RSRP on the SSB associated with PL-RS has been measured/reported - Option 2: Target PL-RS is maintained.

The issue to discuss is whether extra time for PL-RS measurement is needed during cell switch. In last meeting, we have reached concensus no additional PL-RS measurement time is need with some conditions. Two options were proposed.

In active uplink TCI state switching delay for unified TCI, additional time is needed for PL-RS measurement if the target PL-RS is not maintained.When uplink TCI state is activated, the UE is allowed a delay of NM* (Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms) for evaluating the path loss through PL-RS signals if the target PL-RS is not maintained by the UE.

In legacy L3 HO, UE is supposed to use SSB as PL-RS to determine UL transmission power during RACH procedure and no additional PL-RS measurement time is needed. Therefore, similar principle can be used to RACH based cell switch and no additional PL-RS measurement time is needed.

In the legacy unified TCI state framework, PL-RS can be either SSB or CSI-RS. Meanwhile, there were a lot of discussions in RAN4 and finally agreed to only consider maintained case in FR2 when the target PL-RS is SSB in R17 unified TCI state switching.

For a cell with a different PCI, PL-RS is always SSB in the current requirements. Since it was agreed that UE is configured to measure and report the SSB based LTM in Rel-18, it makes sense that the PL-RS would also be configured to be an SSB. The UE will perform L1 measurements and can also be assumed to perform L3 measurements for the cell before TCI state activation or cell switch command, so it should have some understanding on the path loss already based on those measurements. Based on this, we think UE can derive PL from the L1-RSRP samples and no additional delay due to PL-RS is needed.

- Proposal 5: No additional delay for PL-RS measurement time is needed in cell switch delay for PL-RS measurement provide that the following condition is fulfilled:

- L3-RSRP or L1-RSRP on the SSB associated with PL-RS has been measured/reported.

Issue 3-1-4-1: Which cell(s) TLTM-RRC-processing = 0 apply to when candidate cells configured are more than UE capability? <Way Forward >: Further discuss the following options: - Option 1 (Apple): - NW is not expected to trigger TCI activation or RACH toward candidate cell unless cell switch is likely to happen soon. - TLTM_RRC-processing in TS38.133 is zero only applicable to the cells with early TCI activation or early TA acquisition, provided the number of these cells doesn’t exceed UE capability regarding number of cells for early ASN.1 decoding. - Option 2 (Huawei): - TLTM-RRC-processing = 0, if the UE supports [earlyDecodingAndValidityCheck] capability, and the maximum number of serving cell(s) and candidate cell(s) and the maximum number of LTMCandidateConfigs does not exceed [number of candidate cells for early ASN.1 decoding and validity check]. Early TCI state activation or PDCCH order RACH does not necessarily trigger early decoding and validity check. - Option 3 (Nokia): - When TCI state activation MAC-CE or PDCCH order is sent for more cells than UE capability for fast processing, the cells for which the UE received TCI state activation MAC-CE or PDCCH order the most recently before cell switch command are the ones that are pre-processed - Option 4 (MTK): - TLTM_RRC-processing in TS38.133 is zero only applicable to the cells whose TCI state(s) are activated, if early TCI state activation on any candidate cell is triggered, and the number of cells to perform early RRC decoding and validity check does not exceed UE capability. - TLTM_RRC-processing in TS38.133 is zero only applicable to the latest cell on which PDCCH-order RACH is triggered if NW doesn’t active TCI state of any candidate cell - Option 5 (Ericsson, QC) - The ltm-CandidateConfig IEs associated with at least one active TCI state - The ltm-CandidateConfig IEs associated with previously performed PDCCH-order PRACH. - If the number of the ltm-CandidateConfig IEs associated with active TCI state and PDCCH-order PRACH transmission is larger than maxLTMCandidateConfig, the ltm-CandidateConfig IEs for fast RRC processing are chosen in reverse chronological order of Candidate Cell TCI States Activation MAC CE and PDCCH-order PRACH, i.e. maxLTMCandidateConfig ltm-CandidateConfig IEs with the most recently activated TCI states and PDCCH-order PRACH transmission. - The current serving cells and the cells inside the ltm-CandidateConfig, chosen by the above condition, across cell groups (i.e. MCG and SCG) is not larger than maxServingAndCandidteCells

This issue is about the condition on time gap which can guarantee TLTM_RRC-processing =0. In previous meeting, we have consensus that both TCI state activation comand and PDCCH-order RACH can trigger early ASN.1 decoding and validity check on the candidate cell. And there is an agreement that TLTM_RRC-processing in TS38.133 is zero if the time gap is larger than a certain value.

In this issue, we should handle the case how to choose cells to do early decoding and validity check when candidate cells configured are more than UE capability. As mentioned in option2, if number of cells to perform early RRC decoding and validity check does not exceed UE capability, there is no need to consider whether early TCI state activation or PDCCH order RACH. Conversely, if candidate cells configured are more than UE capability, we need to do early RRC decoding and validity check for the cells which have higher propability to be target cell. Consequently, we agreed TCI state activation comand and PDCCH-order RACH can trigger early ASN.1 decoding and validity check on the candidate cell.

In our understanding, option3 and option5 is reasonable. If the UE capability is still exceeded under the screening of these two conditions, then further screening will be conducted based on the principle of the most recent occurrence.

- Proposal 6: When TCI state activation MAC-CE or PDCCH order is sent for more cells than UE capability for fast processing, the cells for which the UE received TCI state activation MAC-CE or PDCCH order the most recently before cell switch command are the ones that are pre-processed.

Issue 3-3-1: known TCI state conditions Online Agreement < Agreement > - For FR1, the cell switch delay requirement is applicable for the case without L1 measurement and report, under certain conditions. - Further discuss the how the requirement applies together with the condition. < Agreement>: - In FR2: the RAN4 Rel-18 related LTM requirements are not applicable if L1 measurement of LTM candidate cell is not supported or configured. <Way Forward >: Further discuss the following options in FR1: - Option 1: change known TCI state conditions to make cell switch delay requirements applicable to the case without L1 measurement and report - Option 2: Allow cell switch delay requirements applicable to unknown TCI state case with conditions. - Regarding the conditions, further discuss the following options when SNR of the TCI state≥ -3dB - Condition 1: TCI state has been activated and - Condition 1A: the TCI state activation command was received not more than 1280 ms before the cell switch command - Condition 1B: Beam-level L3 measurement results have been reported in 1280 ms - Condition 1C: The RS associated to the target TCI state is available at least every 1280 ms after TCI state activation command - Condition 2: UE has reported L3-RSRP measurements for the SSB associated to the target TCI state in [1280]ms before the cell switch command

- In last meeting, we have discussed whether it is flexible to decouple LTM and L1 measurement and reached the agreement that the cell switch delay requirement is applicable for the case without L1 measurement and report under certain conditions in FR1. As to FR2, due to the benefit is less without L1 measurement, the RAN4 Rel-18 related LTM requirements are not applicable if L1 measurement of LTM candidate cell is not supported or configured. In this meeting, we should discuss how the requirement applies together with the condition in FR1.

- There are two options. The first one is to change known TCI state conditions to make cell switch delay requirement applicable to the case without L1 measurement and report. The second one is to allow cell switch delay requirements applicable to unknown TCI state case with condition. In our understanding, both options are fine and can handle the case that the cell switch delay requirement is applicable without L1 measurement and report. However, for the first one, it will cause the different definition from legacy which will make specification more complex, so we perfer the second one.

- Observation 1: Both option1 and option2 are fine and can handle the case that the cell switch delay requirement is applicable without L1 measurement and report. And option1 will cause the different definition from legacy which maybe make specification more complex.

Regarding conditions of unknown TCI state in FR1, one important thing is that target cell shall be known. From UE side, UE is not expected to active TCI of target unknown cell and there should be some conditions to ensure the activated TCI state is really known at UE side. Therefore, SNR of the SSB associated to TCI state should be larger than -3dB. From NW side, NW won’t indicate the TCI state blindly and the activation command is expected to be sent after receiving L1/L3 report. Consequently, L3 report is required from the NW side since the scenario under discussion does not involve L1 measurement.

Proposal 7: For unknown TCI state in FR1, cell switch delay requirements applicable when SNR of the TCI state≥ -3dB and UE has reported L3-RSRP measurements for the SSB associated to the target TCI state in [1280]ms before the cell switch command

2.4 Other

We have agreemrnt in RAN4#11bis meeting that UE is not required to report carriers which are not configured by NW [2] and capture it in the spec..

Agreement in RAN4#110bis - It is RAN4 understanding that UE only needs to follow NW configuration for measurement and validity check. - FG 39-x2 does not apply to any measurements during IDLE/INACTIVE but no included in configuration of R18 CA/DC setup improvement. - UE is not required to report carriers which are not configured by NW.

TS 38.133 clause 4.7.1 UE is not required to report for fast CA/DC setup from any measurements during IDLE mode for carriers which are not configured by higher layers.

However, RAN2 maybe have different understand based on their design. We can observe from the agreement blew. In RAN2’s view, measReselectionCarrierListNR is optional and UE should report any measurement it has if  measReselectionCarrierListNR is not configured [3].

Agreement in RAN2#125bis (R2-2404006) For reselection measurements: Confirm that the freq is optionally configured by the network, applicable to UE availability indication and UE reporting (doesn’t impact the actual measurements). If not configured, the UE reports what is has.

TS 38.331 clause 5.7.10.3 1> if the reselectionMeasurementReq is included in the UEInformationRequest: 2> if validatedMeasurementsReq is included in the UEInformationRequest and measReselectionValidityDuration is included in VarMeasReselectionConfig; 3> if measReselectionCarrierListNR is present in VarMeasReselectionConfig: 4> set the measResultReselectionNR in the UEInformationResponse message the valid NR measurement results, if available for any frequency listed in measReselectionCarrierListNR in VarMeasReselectionConfig and set validityStatus to the value of measIdleValidityDuration in VarMeasReselectionConfig for each reported measurement; 3> else: 4> set the measResultReselectionNR in the UEInformationResponse message to any valid NR measurement results, if available, and set validityStatus to the value of measIdleValidityDuration in VarMeasReselectionConfig; 2> else: 3> if measReselectionCarrierListNR is present in VarMeasReselectionConfig: 4> set the measResultReselectionNR in the UEInformationResponse message the NR measurement results, if available for any frequency listed in measReselectionCarrierListNR in VarMeasReselectionConfig; 3> else: 4> set the measResultReselectionNR in the UEInformationResponse message to any NR measurement results, if available;

Based on the above analysis, we have some doubt that how to understand RAN4’s agreement on UE is not required to report carriers which are not configured by NW.

- Understanding 1: it only uses for the case that measReselectionCarrierListNR is configured and UE is not required to report carriers which are not in measReselectionCarrierListNR. If measReselectionCarrierListNR is not configured, UE will repoet any measurement results, if available.

- Understanding 2: if measReselectionCarrierListNR is not configured, UE will not repoet any measurement results.

Understanding 1 is algin with RAN2 and understanding2 is conflict with RAN2. therefore, we should confirm what  the UE behavior is if measReselectionCarrierListNR is not configured.

Observation 2: when measReselectionCarrierListNR is not configured, there maybe disalignment between RAN2 and RAN4.

Proposal 8: RAN4 should confirm what the UE behavior is if measReselectionCarrierListNR is not configured. UE will repoet any measurement results if available or UE will not repoet any measurement results.

Conclusions

In this contribution, we put forward the following proposals and observations on L1/L2 triggered mobility and report of available reselection measurement .

- Proposal 1: Early TCI state activation delay requirements to be defined for one or more TCI states for a single candidate cell, because one MAC-CE activates TCI states only for a single candidate cell.

Proposal 2: In early candidate cell’s TCI state activation delay for known TCI state case: Tfirst-SSB is the time to the first SSB occasion overlapped with MGL after slot n + THARQ +3Nslotsubframe,µ + [2ms] if the SSB needs to be measured with MG, which is only applicable to UE supporting inter-frequency L1 measurement with MG.

Proposal 3: Unknown TCI state in FR2 for early TCI state activation is supported with the following conditions:

  - UE has reported measurement result of the associated SSB of the TCI state within [TBD] before the LTM TCI state activation command.

  - SNR of the associated SSB is above -3dB.

Proposal 4: RAN4 not to modify the serving cell measurement period by scaling it with NLayer for UE supporting RTD > CP.

- Proposal 5: No additional delay for PL-RS measurement time is needed in cell switch delay for PL-RS measurement provide that the following condition is fulfilled:

- L3-RSRP or L1-RSRP on the SSB associated with PL-RS has been measured/reported.Proposal 6: When TCI state activation MAC-CE or PDCCH order is sent for more cells than UE capability for fast processing, the cells for which the UE received TCI state activation MAC-CE or PDCCH order the most recently before cell switch command are the ones that are pre-processed.

- Observation 1: both option1 and option2 are fine and can handle the case that the cell switch delay requirement is applicable without L1 measurement and report. And option1 will cause the different definition from legacy which maybe make specification more complex.

Proposal 7: For unknown TCI state in FR1, cell switch delay requirements applicable when SNR of the TCI state≥ -3dB and UE has reported L3-RSRP measurements for the SSB associated to the target TCI state in [1280]ms before the cell switch command

Observation 2: when measReselectionCarrierListNR is not configured, there maybe disalignment between RAN2 and RAN4.

Proposal 8: RAN4 should confirm what the UE behavior is if measReselectionCarrierListNR is not configured. UE will repoet any measurement results if available or UE will not repoet any measurement results.

References

- R4-2410302, WF on NR mobility enhancements (part 1), MediaTek Inc.

- R4-2406346, WF on R18 Further NR mobility enhancement - part 2, Apple

- R2-2404006, [Post125bis][513][R18Mob] Idle/Inactive and Reselection Meas Reporting CR, Nokia