3GPP TSG-RAN WG4 Meeting # 112                                R4-2412385

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	5.20.2

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on RRM requirements for multi-carrier enhancements

Document for:	Approval

Introduction

In the last meeting, we discussed the requirements for DL interruption for Tx switching across 2 bands and 2 TAGs case and there were no consensus. In this contribution, we want to provide our view on this issue.

Discussion

According to the agreed WF[1], there is a leftover issue in core part should be discussed. This issue is about DL interruption for Tx switching accorss two bands and two TAGs case and there are two options can be chosen.

(Way Forward) Issue 1-3: DL interruption for Tx switching across 2 bands and 2 TAGs case

- Proposals

  - Option 1: Existing requirements for DL interruption for Tx switching across 2 bands can be applied to DL interruption for Tx switching across 2 bands and 2 TAGs case. (Nokia, E///)

  - Option 2: Existing requirements for DL interruption for Tx switching across 3/4 bands with 2 TAGs can be applied to DL interruption for Tx switching across 2 bands and 2 TAGs case. (HW, MTK, vivo)

For option1, some companies proposed to reuse DL interruption for Tx switching across 2 bands. They explained that DL interruption requirements for UL Tx switching in the case of 2 bands and 2 TAGs case are agreed in legacy requirements and there no distinction was made between single TAG and dual TAGs cases, as mentioned in Table 8.2.2.210-1 and Table 8.2.2.210A-1 of TS 38.133 and shown below as reference.

##### 8.2.2.2.10 DL Interruptions at UE switching between two uplink carriers The DL interruption requirements at dynamic switching between two uplink carreirs specified in this clause are applicable for an uplink band pair of an inter-band UL CA configuration in FR1 when the capability uplinkTxSwitchingPeriod is present, and is only applicable for uplink switching mechanism specified in clause 6.1.6 of TS 38.214 [26], where NR uplink carrier 1 is capable of one transmit antenna connector and NR uplink carrier 2 is capable of two transmit antenna connectors, and the two uplink carriers are in different bands with different carrier frequencies. When dynamic switching between two uplink carriers is conducted, UE is allowed to cause DL interruption of X OFDM symbols in NR downlink carrier(s) as indicated by uplinkTxSwitching-DL-Interruption [2]. The DL interruption starts from the first OFDM symbol which fully or partially overlaps with the UL switching period located in either NR carrier 1 or carrier 2 as indicated in RRC signalling [2]. The DL interruption lengths of X are defined in Table 8.2.2.2.10-1. No DL interruption is allowed in the NR downlink carrier(s) which is not indicated by uplinkTxSwitching-DL-Interruption. No DL interruption is allowed for some inter-band UL CA configurations as specified in clause 5.2A.2 of TS 38.101-1 [18]. Table 8.2.2.2.10-1: DL interruption length on NR carrier(s) in the unit of OFDM symbols (X) for switching between two uplink carriers NR Slot length (ms); Uplink Tx switching period Note1. 35us; 140us; 210us. 0; 1; 2; 3; 4. 1; 0.5; 3; 6; 7. 2; 0.25; 4; 10; 14. Note 1: Uplink Tx switching period depends on UE capability uplinkTxSwitchingPeriod. ##### 8.2.2.2.10A DL Interruptions at UE switching between two uplink carriers with two transmit antenna connectors The DL interruption requirements at dynamic switching between two uplink carriers specified in this clause are applicable for an uplink band pair of an inter-band UL CA configuration when the capability [uplinkTxSwitchingPeriod2T2T] is present, and is only applicable for uplink switching mechanism specified in clause 6.1.6 of TS 38.214 [26], where NR UL carrier 1 is capable of two transmit antenna connectors and NR UL carrier 2 is capable of two transmit antenna connectors, and the two uplink carriers are in different bands with different carrier frequencies. When dynamic switching between two uplink carriers is conducted, UE is allowed to cause DL interruption of X OFDM symbols in NR downlink carrier(s) as indicated by uplinkTxSwitching-DL-Interruption [2]. The DL interruption starts from the first OFDM symbol which fully or partially overlaps with the UL switching period located in either NR carrier 1 or carrier 2 as indicated in RRC signalling [2]. The DL interruption lengths of X are defined in Table 8.2.2.2.10A-1. No DL interruption is allowed in the NR downlink carrier(s) which is not indicated by uplinkTxSwitching-DL-Interruption. No DL interruption is allowed for some inter-band UL CA configurations as specified in clause 5.2A.2 of TS 38.101-1 [18]. Table 8.2.2.2.10A -1: DL interruption length on NR carrier(s) in the unit of OFDM symbols (X) for switching between two uplink carriers NR Slot length (ms); Uplink Tx switching period Note1. 35us; 140us; 210us. 0; 1; 2; 3; 4. 1; 0.5; 3; 6; 7. 2; 0.25; 4; 10; 14. Note 1: Uplink Tx switching period depends on UE capability [uplinkTxSwitchingPeriod2T2T]..

In our understanding, Tx switching across 2 bands and 2 TAGs case is introduced in Rel-18 NR_MC_enh and whether the requirement in 8.2.2.2.10A is apply for 2TAGs is confused.

Observation 1: Tx switching across 2 bands and 2 TAGs case is introduced in Rel-18 NR_MC_enh and whether the requirement in 8.2.2.2.10A is apply for 2TAGs is confused.

For option2, it’s clear to understand the motivation. We have reached agreement on DL interruption for Tx switching across 3/4 bands with 2 TAGs and the difference from single TAG is the value of RTD, which may impact the value of DL interruption. We can observe from tables blew, RTD=3us for single TAG and RTD=9us for two TAGs. Meanwhile, based on different values of RTD, DL interruption length is different when μ=1 and uplink Tx switching period=210us.

Table 8.2.2.2.10D -1: DL interruption length on NR carrier(s) in the unit of OFDM symbols (X) for switching across three or four uplink bands for single TAG NR Slot length (ms); Uplink Tx switching period Note1. 35us; 140us; 210us. 0; 1; 2; 3; 4. 1; 0.5; 3; 6; 7. 2; 0.25; 4; 10; 14. Note 1: Uplink Tx switching period depends on UE capability. Note 2: RTD=3us is assumed to derive the DL interruption length.. Table 8.2.2.2.10D -2: DL interruption length on NR carrier(s) in the unit of OFDM symbols (X) for switching across three or four uplink bands for two TAGs NR Slot length (ms); Uplink Tx switching period Note1. 35us; 140us; 210us. 0; 1; 2; 3; 4. 1; 0.5; 3; 6; 8. 2; 0.25; 4; 10; 14. Note 1: Uplink Tx switching period depends on UE capability. Note 2: RTD=9 us is assumed to derive the DL interruption length..

Observation 2: For Tx switching across 3/4 bands, values for RTD are different from 2 TAGs and single TAG. Concerently, RTD=3us is assumed for single TAG and RTD=9us is assumed for two TAGs to derive the DL interruption length.

Observation 3: For Tx switching across 3/4 bands, the difference between two TAGs and single TAG is small. And DL interruption length is different when μ=1 and uplink Tx switching period=210us.

Based on above analysis, we perfer to reuse existing requirements for DL interruption for Tx switching across 3/4 bands with 2 TAGs can be applied to DL interruption for Tx switching across 2 bands and 2 TAGs case. We should algin the requirements for 2TAGs between 2 bands and 3/4 bands.

Proposal 1: RAN4 should algin the requirements for 2TAGs between 2 bands and 3/4 bands.

Conclusions

In this contribution, we discuss the RRM requirements for Rel-18 Tx switching and propose that:

Observation 1: Tx switching across 2 bands and 2 TAGs case is introduced in Rel-18 NR_MC_enh and whether the requirement in 8.2.2.2.10A is apply for 2TAGs is confused.

Observation 2: For Tx switching across 3/4 bands, values for RTD are different from 2 TAGs and single TAG. Concerently, RTD=3us is assumed for single TAG and RTD=9us is assumed for two TAGs to derive the DL interruption length.

Observation 3: For Tx switching across 3/4 bands, the difference between two TAGs and single TAG is small.

Proposal 1: RAN4 should algin the requirements for 2TAGs between 2 bands and 3/4 bands.

References

- R4-2410300, WF on RRM requirements on Multi-carrier enhancements