3GPP TSG-RAN WG4 Meeting # 112                                R4-2412386

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.4.3

Source:         ZTE Corporation, Sanechips

Title:           Initial discussion on RRM impacts for less than 5MHz BW

Document for:	Approval

Introduction

The work item on NR support for dedicated spectrum less than 5MHz for FR1 was approved at RAN#103[1]. according to the WID, RAN4 RRM works starts in this meeting and main RAN4 scopes are summarized as following:

- Define common co-located and non-co-located inter-band NR CA/DC UE RF requirements with 3MHz CBW in the one band and 5MHz or 10MHz CBW in the other band - Example band combination: CA/DC of 3MHz in band n100 and 5MHz or 10MHz in band n101 - Define RRM requirements for inter-band CA and DC for combinations introduced in RF part Note: other band combinations than example band combinations can be specified in basket WIs after the above generic requirements are specified.

As this is the first meeting for this WI, we will provide our initial views on potential impacts on RRM requirements.

Discussion

In Rel-18, the impact to support operation in dedicated spectrum allocations from approximately 3 MHz up to below 5 MHz has been discussed for single carrier and relative RRM requirements were updated. In Rel-19, RAN4 should define RRM requirements for inter-band CA and DC cases.

In Rel-18, RAN4 only defined SSB-based L3/L1 measurement requirements and not defined CSI-based L3/L1 measurement requirements. Similar as Rel-18, we perfer only to define SSB-based measurement. As to CSI-RS based measurement, it should be depriority.

Proposal 1: RAN4 will define SSB-based L1/L3 measurement requirements. And CSI-RS based L1/L3 measurement is recommended to be accorded a lower priority.

As per the WID in Rel-19, CA and DC are supported and the RRM requirements maybe impact due to introduction of NR in less than 5 MHz bandwidth. RANP agreed to use a PBCH transmission bandwidth is 20 PRBs for 5MHz channel bandwidth and a PBCH transmission bandwidth is 12 PRBs for 3MHz channel bandwidth. So, there is no impact when CBW equals to 5MHz. For the case that CBW equals to 3MHz, the transmission bandwidth for PBCH is smaller and RAN4 should further discuss whether to update relative requirements or not.

The reduced PBCH bandwidth for 3MHz channel bandwidth will have an impact on RRM requirements that depend on PBCH DMRS detection, PBCH decoding etc. RAN4 to study impact of reduced PBCH BW of 12PRBs for 3MHz channel bandwidth on at least the following RRM requirements:

| RRM requirements | CBW = 3MHz | CBW = 5MHz |
| --- | --- | --- |
| IDLE/inactive mode mobility | FFS: EMR measurement with SSB index reading (section: 4.4.2.2 Measurements of inter-frequency CA/DC candidate cells) | No impact |
| SCell activation/deactivation | Activation delay requirement: FFS if PBCH BW reduction will impact SSB index reading during SCell activation | No impact |
| PSCell addition/change/release | PSCell addition: FFS if PBCH BW reduction will impact SSB index reading during Tsearch | No impact |
| SCG activation/deactivation | SCG activation: FFS if PBCH BW reduction will impact SSB index reading during Tsearch | No impact |

Proposal 2: RAN4 to study impact of reduced PBCH BW of 12PRBs for 3MHz channel bandwidth on at least the following RRM requirements:

  - EMR measurement with SSB index reading

  - SCell activation/deactivation

  - PSCell addition/change/release

  - SCG activation/deactivation

For EMR measurement, the requirement for legacy UE is N2 = 3 if the NR inter-frequency carrier for idle mode CA/DC measurement reporting is in FR1[2].

Table 4.4.2.2-1: TSSB_index,NR_Inter

| DRX cycle length [s] | Scaling Factor (N1) |  | TSSB_index,NR_Inter [s] (number of DRX cycles) |
| --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |
| 0.32 | 1 | 8 | N2 x 1.28 x N1 x 1.5 (N2 x 4 x N1 x 1.5) |
| 0.64 |  | 5 | N2 x 1.28 x N1 (N2 x 2 x N1) |
| 1.28 |  | 4 | N2 x 1.28 x N1 (N2 x 1 x N1) |
| 2.56 |  | 3 | N2 x 2.56 x N1 (N2 x 1 x N1) |
| Note 1: Applies for UE supporting power class 2&3&4. For UE supporting power class 1, N1 = 8 for all DRX cycle length. NOTE 2: N2 = 3 if the NR inter-frequency carrier for idle mode CA/DC measurement reporting is in FR1, and N2 = 3, 5 if the NR inter-frequency carrier for idle mode CA/DC measurement reporting is in FR2. |  |  |  |

- In Rel-18, RAN4 has discussed SSB index identification delay for unknown intra-frequency cell and unknown inter-frequency cell. Finially, RAN4 has reached concensus that 3+4 samples for intra-frequency and 3+3 samples for inter-frequency. In our understanding, same requirements can be reused for EMR measurement. That’s to say, N2=[3+3] if the NR inter-frequency carrier for idle mode CA/DC measurement reporting is in FR1and PBCH bandwidth for 3MHz channel bandwidth.

Observation 1: In Rel-18, SSB index identification delay

  - Unknown intra-frequency cell (Es/Iot≥-6 dB): 3+4 samples

  - Unknown inter-frequency cell (Es/Iot≥-4 dB): 3+3 samples

- Proposal 3: For 3MHz channel bandwidth, N2=[3+3] if the NR inter-frequency carrier for idle mode CA/DC measurement reporting is in FR1.

As to PSCell addition/change/release, we can refer to the discussion on side condition for handover in Rel-18. In Rel-18, the value of T_delta has be updated and additional Trs has be added in handover requirements due to PBCH BW of 12PRBs for 3MHz channel bandwidth. In our understanding, the same principle can be reused for PSCell addition/change/release delay.

Observation 2: In the latest version of TS 38.133, T_delta = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW.

Proposal 4: Samilar as HO requirements, T_delta should be updated and additional two Trs should be added for PSCell addition/change/release delay

Conclusions

In this contribution, we provide our views on the RRRM impacts for less than 5MHz BW and the proposals are:

Proposal 1: RAN4 will define SSB-based L1/L3 measurement requirements. And CSI-RS based L1/L3 measurement is recommended to be accorded a lower priority.

Proposal 2: RAN4 to study impact of reduced PBCH BW of 12PRBs for 3MHz channel bandwidth on at least the following RRM requirements:

  - EMR measurement with SSB index reading

  - SCell activation/deactivation

  - PSCell addition/change/release

  - SCG activation/deactivation

Observation 1: In Rel-18, SSB index identification delay

  - Unknown intra-frequency cell (Es/Iot≥-6 dB): 3+4 samples

  - Unknown inter-frequency cell (Es/Iot≥-4 dB): 3+3 samples

Proposal 3: For 3MHz channel bandwidth, N2=[3+3] if the NR inter-frequency carrier for idle mode CA/DC measurement reporting is in FR1.

Observation 2: In the latest version of TS 38.133, T_delta = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW.

Proposal 4: Samilar as HO requirements, T_delta should be updated and additional two Trs should be added for PSCell addition/change/release delay

References

- RP-240832, WI resulting from discussion on NR channel BW less than 5MHz for TN, vivo

- TS 38.133