3GPP TSG-RAN WG4 Meeting # 112                                R4-2412388

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.5.3

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on intra-band non-collocated EN-DC and NR-CA

Document for:	Approval

Introduction

For intra-band non-collocated EN-DC/NR-CA deployment, RAN4 has diasussed and achieved agreements on MRTD/MTTD requirements, interruption requirements and scheduling restriction requirements for Type 2 UE in Rel-18. In Rel-19, we should focus on Type 4 UE accroding to WID [1] to better support the intra-band non-collocated EN-DC/NR-CA deployment scenarios.

The core part of the work item includes: - Specify the core requirements for type 4a/4b capable FWA UE that supports non-co-located scenarios for FR1 inter-band non-contiguous EN-DC with overlapping or partially overlapping bands and/or FR1 intra-band non-contiguous NR-CA for up to 4-layer DL MIMO per CC, where MRTD>CP is assumed. - RF power imbalance requirements of up to 25dB - RRM requirements including MTTD and MRTD requirements and other identified requirement such as scheduling restriction/availability, SCell operation related delay and interruption. - Work is limited to EN-DC/NR-CA for bands 42, n77/n78 - Discuss and if needed, specify the UE behavior when the UE supports type 4a/b but operates in co-located scenarios depending on whether network signaling is provided. R18 handling of UE behavior can be used as a reference.

In this contribution, we will provide the discussion on RRM impacts for intra-band non-collocated EN-DC/NR-CA, including MTTD and MRTD requirements and other identified requirement such as scheduling restriction/availability, SCell operation related delay and interruption.

Discussion

Based on the agreements in [2], Type 3a/3b and Type 4a/4b are assumed for 4-layer MIMO case. The assumed UE architectures for different Types are listed as Table.

| UE Type | CC# | antenna / LNA |  | Mixer | Analog BB | #Rx | Frequency Separation between 2cc | NRCA/ENDC | power imbalance | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 shared |  | 4 shared | 4 shared | 4Rx | ≤ X MHz | NRCA,ENDC | 6dB full range | Baseline architecture (i.e. legacy architecture) |
|  | 2 |  |  |  |  | 4Rx |  |  |  |  |
| 2 | 1 | 2 | 4 total | 2 | 2 | 2Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Reuse of baseline architecture restricted to 2Rx/band but need 2LO frequencies |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 3a | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 2 | 2 | 2Rx |  |  |  |  |
| 3b | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 4 | 4 | 4Rx |  |  |  |  |
| 4a | 1 | 4 | 6 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 25dB full range | Requires 6 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 4b | 1 | 4 | 8 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Requires 8 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 4 |  | 4 | 4 | 4Rx |  |  |  |  |

It can be observed from the table that Type4 UE is assumed to have separate Rx chains for different CCs, similar with Type 2 UE. Different from Type2 UE, Type4 UE is up to 4-layer DL MIMO per CC.

2.1 MRTD/MTTD requirements

MRTD is specified with the combination of BS TAE and the propagation delay difference among the cells. BS TAE has been already specified to 3usec in TS 38.104. For non-collocated deployment, the distance between the cells in non-collocated deployment is same for FR1 inter-band or FR1 intra-band EN-DC or NR-CA, and we assume 9km distance for FR1 cell coverage, the maximum propagation delay difference would be 30us to support 9km FR1 cell coverage.

- It’s the common understanding that the maximum propagation delay difference will be impacted by the allowed power balance level, where the propagation delay difference is increased with the power balance level. In RF session, there is already confirm to use 25dB power imbalance for 4Rx for Type 4 [3]. Therefore, MRTD/MTTD requirements of type2 UE is enough to type4 UE.

RAN4#111 < Issue 2-1-1: Power imbalance requirements and REFSENS > Agreement: - Use 25dB power imbalance for 4Rx REFSENS + 1dB for Type 4

In our understanding, the current MRTD/MTTD requirements of Type 2 UE can be reused for Type 4 UE.

RAN4#105 Issue 1-2-1: MRTD/MTTD requirements for non-collocated FR1 intra-band non-contiguous NR-CA for Type 2 UE - Agreements - MRTD/MTTD requirements for non-collocated FR1 intra-band non-contiguous NR-CA for Type 2 UE - MRTD = 33us Issue 1-2-2: MRTD/MTTD requirements for non-collocated FR1 intra-band synchronous EN-DC for Type 2 UE - Discussion - QC: Change “non-collocated FR1 intra-band synchronous EN-DC” => “non-collocated FR1 inter-band synchronous EN-DC with overlapping DL bands” - Agreements - MRTD/MTTD requirements for non-collocated FR1 inter-band synchronous EN-DC with overlapping DL bands for Type 2 UE - MRTD = 33us - MTTD = 35.21us

Proposal 1: For 4 layer MIMO case, the existing MRTD/MTTD requirements defined in 38.133 for inter-band case can be reused for intra-band EN-DC/NR-CA in non-collocated deployment.

2.2 SCell operation related delay and interruption

- 2.2.1 SCell activation delay requirements

In R18, MRTD for intra-band non-contiguous CA is extended from 3us to 33us, which is same as that for inter-band CA. Due to the same SCell activation delay requirements are applied for FR1 intra-band collocated non-contiguous CA and FR1 inter-band CA cases, the SCell activation delay requirements for inter-band CA case have been reused for intra-band non-contiguous case. With the same reason, if MRTD requirement can be reused for Type 4 UE, the current SCell activation delay requirements for FR1 intra-band non-collocated non-contiguous CA for Type 2 UE and FR1 inter-band CA can also be applied for FR1 intra-band non-collocated non-contiguous CA for Type 4 UE.

| Scenario# |  | Tactivation_time |
| --- | --- | --- |
| Known SCell | The activated SCell measurement period is no larger than 2400ms | - TFirstSSB+ 5ms |
|  | The activated SCell measurement period is larger than 2400ms | - TFirstSSB_MAX + Trs + 5ms |
| Unknown SCell | When one of the following conditions is met: - ‘ssb-PositionInBurst’ indicates only one SSB is being actually transmitted, or - ‘ssb-PositionInBurst’ indicates multiple SSBs and TCI indication is provided in same MAC PDU with SCell activation | - TFirstSSB_MAX + TSMTC_MAX + 2*Trs + 5ms |
|  | When both of the following conditions are not met: - ‘ssb-PositionInBurst’ indicates only one SSB is being actually transmitted, or - ‘ssb-PositionInBurst’ indicates multiple SSBs and TCI indication is provided in same MAC PDU with SCell activation | - 6ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP,measure + TL1-RSRP,report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting, - 3ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP,measure + TL1-RSRP,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting. |

Observation1: If MRTD requirement can be reused for Type 4 UE, the current SCell activation delay requirements for FR1 intra-band non-collocated non-contiguous CA for Type 2 UE and FR1 inter-band CA can also be applied for FR1 intra-band non-collocated non-contiguous CA for Type 4 UE.

2.2.2 Interruptions at SCell addition/release/activation/deactivation

As to interruptions requirements, separate Rx chains are assumed For inter-band case and intra-band non-collocated (Type 2 UE) case, and there is no interruption due to AGC settling. For intra-band case collocated case, shared Rx chain is assumed and the interruption due to AGC settling is included. In R19, Type 4 UE with separate Rx chains are assumed for intra-band non-contiguous case, which is same as the assumption for inter-band case and intra-band non-collocated for Type 2 UE. Therefore, the current interruption requirements for inter-band CA also can be applied for Type 4 UE.

Proposal 2: The existing interruptions requirements as SCell addition/release/ activation/deactivation in FR1 for FR1 inter-band CA can be applied for FR1 intra-band non-contiguous CA supporting non-collocated deployment for 4-layer MIMO case.

- 2.2.3 SCell BFD requirements

Similar as Type2 UE, we should revised BFD/CBD measurements requirements for FR1 intra-band non-contiguous CA supporting non-collocated deployment for 4-layer MIMO case. As mentioned in previous meeting, it is assumed that the serving cells in the same band have the similar link quality in co-located deploymen for link recovery procedure, and the link qualities of serving cells in the same band could be different for Type 2 UE in non-collocated deployment. Consequently, there is no need to limit BFD/CBD requirements only on one serving cell. The same principle can be applied to type4 UE.

- For non-collocated FR1 intra-band non-contiguous CA, Type 2 UE can be configured with BFD/CBD measurements on up to two serving cells in the same band

Proposal 3: It is suggested that Type 4 UE can be configured with BFD/CBD measurements on more than one serving cells in the same band.

2.3 Scheduling availability requirements

For current scheduling availability requirements, the scheduling restrictions on one FR1 serving cell shall also apply to other serving cells in the same band based on the assumption that shared Rx chain is used for intra-band CA, and there is no scheduling restrictions on serving cells in other bands based on the assumption that separate Rx chains are used for inter-band CA or intra-band CA. Due to separate Rx chains assumed for intra-band non-contiguous CA for supporting non-collocated deployment, there is no scheduling restrictions on non-contiguous serving cells in the same band.

- For Type 2 UE with FR1 intra-band CA, there are no scheduling restrictions on carriers in the same band.

Proposal 4: For Type 4 UE with FR1 intra-band CA, there are no scheduling restrictions on carriers in the same band.

Conclusions

In this contribution, we put forward the following proposals:

Proposal 1: For 4 layer MIMO case, the existing MRTD/MTTD requirements defined in 38.133 for inter-band case can be reused for intra-band EN-DC/NR-CA in non-collocated deployment.

Observation1: If MRTD requirement can be reused for Type 4 UE, the current SCell activation delay requirements for FR1 intra-band non-collocated non-contiguous CA for Type 2 UE and FR1 inter-band CA can also be applied for FR1 intra-band non-collocated non-contiguous CA for Type 4 UE.

Proposal 2: The existing interruptions requirements as SCell addition/release/ activation/deactivation in FR1 for FR1 inter-band CA can be applied for FR1 intra-band non-contiguous CA supporting non-collocated deployment for 4-layer MIMO case.

Proposal 3: It is suggested that Type 4 UE can be configured with BFD/CBD measurements on more than one serving cells in the same band.

Proposal 4: For Type 4 UE with FR1 intra-band CA, there are no scheduling restrictions on carriers in the same band.

References

- RP-240838, New WID on Support of intra-band non-collocated EN-DC/NR-CA, Moderator (Apple)

- R4-2220537, WF on the requirement for intra-band non-collocated CA/EN-DC, KDDI

- R4-2410686, WF on Rel-19 non-collocated UE RF requirements