3GPP TSG-RAN WG4 Meeting # 112                                R4-2412389

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.15.3

Source: 	ZTE Corporation, Saneships

Title: 	Discussion on the fast SCell activation for UE supporting Rel-18 EMR

Document for:	Approval

Introduction

In the RAN #113 meeting, the WID on several enhancements for NR and MR-DC RRM requirements was approval. There are two objectives need to discuss in Rel-19, one is FR2-1 SSB based L3 measurement delay reduction for connected mode, and the other is FR2-1 SSB based L3 measurement delay reduction for connected mode. In this meeting, we need to study the latter according to schedule [1].

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - For UE not in multiple-Rx simultaneous reception mode: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

In this contribution, we would like to provide our views on fast SCell activation for UE supporting Rel-18 EMR.

Discussion

According the work item, we want to consider valid early measurement report in the existing known/unknown SCell condition during SCell configuration/activation, which may reduce delay requiremeny for the target SCell activation.

In Rel-18 mob_enh, RAN4 has discussed how to reuse the IDLE/INACTIVE mode measurement results which are to be reported during RRC connection setup/resume in order to improve SCell/SCG setup delay. We think it’s feasible and beneficial to consider fast SCell activation for UE supporting Rel-18 EMR.

In existing known SCell condition, there is a sub-condition that the UE has sent a valid measurement report for the SCell being activated in FR1 or the UE has sent a valid L3-RSRP measurement report with SSB index in FR2.

SCell in FR1 is known if it has been meeting the following conditions: - During the period equal to max(5*measCycleSCell, 5*DRX cycles) for FR1 before the reception of the SCell activation command: - the UE has sent a valid measurement report for the SCell being activated and - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3. - the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3. Otherwise SCell in FR1 is unknown. For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions: - During the period equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable): - the UE has sent a valid L3-RSRP measurement report with SSB index, and - SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation - During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes. Otherwise, the first SCell in FR2 band is unknown.

In our understanding, if there is valid EMR report for the SCell being activated, it can be consider as UE has sent a valid measurement report. Meanwhile, according to RAN2’s design, SSB index has been included in EMR.

Observation1：If there is valid EMR report for the SCell being activated, it can be consider as UE has sent a valid measurement report with SSB index.

For SCell activation requirements, Tactivation_time is different for known and unknown SCell in FR1 and FR2. Taking FR2-1 as an example, if the SCell being activated belongs to FR2 and if there is no active serving cell on that FR2 band, we can observe from the table that unknown SCell needs more time to excute cell search, measurement and report.

Table 1-1. Tactivation_time for known and unknown SCell in FR2

(no active serving cell on that FR2 band)

| Scenario# |  | Tactivation_time |
| --- | --- | --- |
| Known SCell | semi-persistent CSI-RS is used for CSI reporting | 3ms + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP) |
|  | periodic CSI-RS is used for CSI reporting | max(Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay-THARQ) |
| Unknown SCell | semi-persistent CSI-RS is used for CSI reporting | 6ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs + TL1-RSRP, measure + TL1-RSRP, report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP) |
|  | periodic CSI-RS is used for CSI reporting | 3ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs + TL1-RSRP, measure + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)} |

If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, Tactivation_time is defined as below.

Table 1-2. Tactivation_time for known and unknown SCell in FR2

(at least one active serving cell on that FR2 band)

| Scenario# | Tactivation_time |
| --- | --- |
| - The UE is provided with SMTC for the target SCell, and - The SSBs in the serving cell(s) and the SSBs in the SCell fulfil the condition defined in clause 3.6.3, and - The parameter ssb-PositionsInBurst is same for the serving cell(s) and the Scell, and - SSB is in the same half-frame on the SCell and the contiguous FR2 active serving cell. | TFirstSSB+ 5ms |
| if the UE supporting scellWithoutSSB is not provided with any SMTC for the target SCell | Tactivation_time is 3 ms |

For FR1, if valid EMR be considered, activation delay for unknown SCell with valid EMR reporting would be reduced as well.

Table 2. Tactivation_time for known and unknown SCell in FR1

| Scenario# |  |  | Tactivation_time |
| --- | --- | --- | --- |
| Known SCell | measurement period<2400ms |  | - TFirstSSB+ 5ms |
|  | measurement period not smaller than 2400ms |  | - TFirstSSB_MAX + Trs + 5ms |
| Unknown SCell | - with indicated ‘ssb-PositionInBurst’ | intra-band contiguous case | - TFirstSSB_MAX + TSMTC_MAX + Trs + 5ms, |
|  |  | intra-band non-contiguous case and inter-band case | - TFirstSSB_MAX + TSMTC_MAX + 2*Trs + 5ms |
|  | without indicated ‘ssb-PositionInBurst’ | intra-band contiguous case | - Tactivation_time is 3 ms |
|  |  | intra-band non-contiguous case and inter-band case | - 6ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP,measure + TL1-RSRP,report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting, - 3ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP,measure + TL1-RSRP,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting. |

Fig1. Illustration of unknown Scell activation delay in FR2

Observation 2: Tactivation_time for unknown SCell is longer than known SCell due to measurement and report. SCell activation delay could be reduced if valid EMR can be considered into known SCell condition.

Proposal 1: To reduce the SCell activation delay, RAN4 should consider valid EMR into fast SCell activation.

In R18 Mob_enb, not only EMR, but also cell reselection measurement has been considered. In Rel-19 RRM_enh, RAN4 should study fast SCell activation for UE supporting Rel-18 EMR. For Rel-18 EMR, it is unclear whether it includes non-EMR or not. Therefore, we prefer to consider cell reselection measurement as well in fast SCell activation procedure, which is same as Rel-18.

Observation 3: In R18 Mobility, IDLE/INACTIVE mode measurement results including EMR and cell reselection measurement are used to improve SCell/SCG setup delay.

Proposal 2: Similar as R18, both EMR and cell reselection measurement should be considered for fast SCell activation.

Conclusions

In this paper, we have some discussion on fast SCell activation for UE supporting Rel-18 EMR. And we have the following proposals:

Observation1：If there is valid EMR report for the SCell being activated, it can be consider as UE has sent a valid measurement report with SSB index.

Observation 2: Tactivation_time for unknown SCell is longer than known SCell due to measurement and report. SCell activation delay could be reduced if valid EMR can be considered into known SCell condition.

Proposal 1: To reduce the SCell activation delay, RAN4 should consider valid EMR into fast SCell activation.

Observation 3: In R18 Mobility, IDLE/INACTIVE mode measurement results including EMR and cell reselection measurement are used to improve SCell/SCG setup delay.

Proposal 2: Similar as R18, both EMR and cell reselection measurement should be considered for fast SCell activation.

References

- RP-240830, New WID: WI resulting from discussion on RRM enhancements, vivo (Moderator)