3GPP TSG-RAN WG4 Meeting #112									                          R4-2412404

Maastricht, The Netherlands, Aug. 19 – 23, 2024

Agenda item:			8.1.2

Source:	Ericsson

Title:	Discussions on RRM impact of Rel-19 WI on UE RF enhancements WI

Document for:	Discussions

# Introduction

In 3GPP Release 18, RAN4 finalized the standardization efforts for enabling 8Rx for CPE/FWA/vehicle/industrial type of UEs. Although 6Rx was proposed and deliberated upon, it was given lower priority during the Rel-18 discussion. To enhance DL coverage and throughput for handheld UEs with larger sizes, RAN4 has approved a new WI to specify requirements for both handheld and FWA UEs [1].

High power UE (HPUE) for CA in terrestrial network (TN) - Specify the generic requirements of high power UE (HPUE) for NR uplink (UL) CA in FR1 and EN-DC with NR FR1 bands - Power class 1.5 (PC1.5) UE for NR TDD intra-band UL contiguous and non-contiguous CA with 2Tx - Specify the requirements for intra-band UL contiguous CA with or without UL-MIMO - Example band combinations: - CA_n41C, CA_n78C, CA_n77C, CA_n79C for intra-band uplink contiguous CA configurations - Focus on the maximum output power (MOP), MPR/A-MPR requirements, SAR solution - Specify the requirements for intra-band UL non-contiguous CA without UL-MIMO - Example band combinations: - CA_n78(2A), CA_n77(2A) for intra-band uplink non-contiguous CA configurations - Focus on the maximum output power (MOP), MPR/A-MPR requirements, SAR solution - NOTE: leave the other band combination specific requirements to the corresponding Rel-19 basket WIs - PC1.5 UE for two band NR inter-band uplink CA with 2Tx and/or 3Tx for handheld and FWA, and PC1.5 and PC2 for two band EN-DC with 2Tx and/or 3Tx for handheld and FWA - Focus on the SAR solution - Enable power class 2 (PC2) and PC1.5 of two band inter-band uplink CA and EN-DC with 3Tx for handheld UE - Identify and update the requirements if necessary - Only PC3 is considered for LTE FDD in EN-DC - NOTE: leave the band combination specific requirements, e.g., MSD to the corresponding Rel-19 basket WIs - Investigate and if feasible, support increasing UE transmission power limit up to the sum of maximum output power per band for NR inter-band uplink CA and EN-DC HPUE with the different existing power classes which have already been specified - Introduce the signaling to support the above objectives, if needed. - Consider release independency, if needed Power boosting and/or MPR reduction - Specify power domain enhancement, e.g., MPR reduction for NR single carrier and NR intra-band UL CA - Study the scenarios, and if feasible, specify the power domain enhancement, e.g., MPR reduction, for PC2 and PC3 with applicable ACLR/SEM/spurious emission modification with BS indication for NR FR1 on a single UL carrier - Include the following scenarios: - when there is no adjacent in-band/out-of-band co-existence issue - when a UE uses a narrower channel bandwidth within a wider BS bandwidth - Include both (e)RedCap UE (only PC3) and non-RedCap UE - Limited to QSPK and 16QAM - Specify MPR applicability based on the UL CCs with activated cells for NR intra-band UL CA configuration - Include both intra-band UL contiguous CA and intra-band non-contiguous UL CA for FR1 - Include intra-band UL contiguous CA and intra-band DL contiguous CA with single UL for FR2 - MPR requirement is not applicable until the SCell is activated - Necessary signaling to support the above objectives 6Rx for handheld and FWA UE - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

In this contribution we discuss the RRM impact of this objective specified in the WI.

# Discussions

The first objective is related to supporting higher power UE for CA in the terrestrial network. No RRM work is expected due to this objective.

- No RRM work is expected due to the WI objective on introducing support for the higher power UE for CA.

The second objective is related to specifying power domain enhancements (power boosting and/or MPR reduction) which do have any RRM impact.

- No RRM work is expected due to the WI objective on power domain enhancements.

The third objective is related to enabling support for operating handheld UEs with 6Rx in FR1.This objective is expected to have RRM impact. It shall be noted that most of the core RRM requirements (such as cell identification delay, measurement delay, mobility requirements etc.) can be reused since they are defined under the assuming that the UE capable of at least 2 Rx. Therefore, the legacy RRM core requirements can be reused for the 6 Rx UEs.

- The legacy core RRM requirements are reused for 6 Rx capable UEs.

Introducing support for 6Rx capable UE is expected to trigger some RRM performance work. More specifically, RAN4 needs to revisit the existing test configurations in Annex 3 of Ts 38.133 and check whether they can be applied for 6 Rx capable UE. RAN4 may need to add a separate set of antenna configurations for 6 Rx capable UEs in Annex A.3.6 of TS 38.133 and may also need to define principle of testing similar to work that was done when introducing 8 Rx capable UE in Rel-18.

In addition, discussions are also needed about the impact on RLM and BFD tests when operating with 6 Rx UEs. For example, whether the existing (2 Rx, or 4 Rx requirements) SNR levels can be reused or not. Due to that the UE is capable of 6 Rx, it is expected that the different RLM/BFD events (in-sync, out-of-sync, BFD, CBD) can be triggered at different SNR levels compared to those defined for the legacy UEs.

- Impact on RRM performance requirements is expected due to introducing 6 Rx capable UEs in FR1.

- RAN4 to define RRM performance requirements for operating 6 Rx capable UEs in FR1 which includes:

      - New antenna configurations and principle of testing

- RAN4 to discuss impact on RLM/BFD tests when operating with 6 Rx UEs.

# Conclusion

In the previous sections, we made the following observations:

- No RRM work is expected due to the WI objective on introducing support for the higher power UE for CA.

- No RRM work is expected due to the WI objective on power domain enhancements.

- Impact on RRM performance requirements is expected due to introducing 6 Rx capable UEs in FR1.

- The legacy core RRM requirements are reused for 6 Rx capable UEs.

- RAN4 to define RRM performance requirements for operating 6 Rx capable UEs in FR1 which includes:

      - New antenna configurations and principle of testing

- RAN4 to discuss impact on RLM/BFD tests when operating with 6 Rx UEs.

# References

- RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4.