3GPP TSG-RAN4 Meeting #112	R4-2412405

# Maastricht, NL, 19th Aug 2024 – 23rd Aug 2024

Source:	LG Electronics

Title:	Discussion for 6 Rx REFSENS

Agenda item:	8.1.1.3.1

Document for:	Discussion

Introduction

In the RAN4#111 meeting,  the WF on requirements for 6Rx was approved. In this discussion, we share our point of view for the 6Rx REFSENS based on the approved WF[1]

Discussion

Table 1 Way forward for 6 Rx REFSENS [1]

Topic 1: REFSENS (delta RIB,6R) Sub-topic 1-1: General considerations for specifying ΔRIB,6R value Issue 1-1-1: Whether band n104 should be included in the high band (n77, n78 and n79) category for 6Rx case Agreement: For ΔRIB,6R, include n77, n78, n79 and [n104] into the same group. Issue 1-1-2: Whether to use same ΔRIB,6R value for handheld UE and FWA Way Forward: Further discuss the following options. - Option 1: RAN4 needs to determine whether to define different ΔRIB,6R value for handheld UE and FWA separately - Option 2: Same value for handheld UE and FWA - Option 3: Different value for handheld UE and FWA Issue 1-1-3: Release independence of ΔRIB,6R value Way Forward: Follow RAN4 Chair guidance and defer the release independence discussions until the requirements are stable. Sub-topic 1-2: ΔRIB,6R values for handheld UE and FWA Issue 1-2-1: Proposed ΔRIB,6R values for handheld UE and FWA Way Forward: Further discuss the following options for the ΔRIB,6R values. Ultimately, the decision to specify separate or same requirements for handheld UE and FWA will be dependent on outcome of Issue 1-1-2. - Option 1: Consider range of ΔRIB,6R in brackets for each case below for further discussion. Operating bands; ΔRIB,6R for FWA; ΔRIB,6R for Handheld. n77, n78, n79, [n104]; [-3.0 ~ -3.3] dB; [-3.0 ~ -3.2] dB. n41; [-3.0 ~ -4.0] dB; [-3.0 ~ -4.0] dB. - Option 2: Consider average value of ΔRIB,6R from company proposals for further discussion. - Option 3: Other options are not precluded.

Table 2 Considerations for deriving ΔRIB,6R [2]

Observation 1: The frequency range of 4Rx high band(n48, n77, n78, n79 and n104) is quite broad, around 3800 MHz. In the case of 4 Rx, the n104 band could be considered a high band because the high frequency performance degradation of 4Rx UEs is less than 6Rx or 8Rx UEs.The complexity of UEs for 6Rx or 8Rx can be higher than 4Rx UEs, which may result in a higher potential for highfrequency performance degradation. Observation 2: Depending on the operating frequency bands, the optimum antenna placement may vary and may affect the REFSENS. For example, handheld UEs typically have less space for antenna placement than FWA UEs, which means that applying the optimal antenna placement for the handheld UEs can be more difficult than FWA UEs. Observation 3: The 6Rx UEs are more complex than 4Rx but less complex than 8Rx, therefore the Relaxation6Rx can be considered to be larger than Relaxation4Rx(0.3~0.8 dB) and smaller than Relaxation8Rx(1.5~2 dB) .

Table 3 Analysed ΔRIB,6R

| Operating band | Handheld ΔRIB,6R | Non-handheld ΔRIB,6R |
| --- | --- | --- |
| n41 | -3.5 | -3.7 |
| n77/n78, n79 | -3.2 | -3.4 |
| n104 | -3.0 | -3.4 |

Our views we presented at the last meeting are shown in Table2 and the analysed ΔRIB,6R value is shown in Table 3.

Our point of views on each issue on the approved WF are as follows.

For Issue 1-1-1: Whether band n104 should be included in the high band (n77, n78 and n79) category for 6Rx case

,

In table 3,non-handheld ΔRIB,6R values has no difference between   n104 and n77/n78/n79, but there is a 0.2 dB difference for handheld. If n104 is included in category n77/n78/n79, it is necessary to consider the worst ΔRIB,6R value among n77,n78,n79 and n104.

Proposal 1: It is necessary to consider the worst ΔRIB,6R value among n77,n78,n79 and n104, if n104 is included in category n77, n78, n79

For Issue 1-1-2: Whether to use same ΔRIB,6R value for handheld UE and FWA,

handheld UEs typically have less space for antenna placement than FWA UEs, which means that applying the optimal antenna placement for the handheld UEs can be more difficult than FWA UEs. Therefore, it is necessary to consider different ΔRIB,6R value for handheld UE and non-handheld UE and consider the ΔRIB,6R relaxation for non-handheld UE about 0.2 ~0.4 dB.

Proposal 2: it is necessary to consider different ΔRIB,6R value for handheld UE and non-handheld UE and consider the ΔRIB,6R relaxation for non-handheld UE about 0.2 ~0.4 dB.

For Issue 1-2-1: Proposed ΔRIB,6R values for handheld UE and FWA,

Proposal 3: Proposed ΔRIB,6R values

| Operating band | Handheld ΔRIB,6R | Non-handheld ΔRIB,6R |
| --- | --- | --- |
| n41 | -3.5 | -3.7 |
| n77, n78, n79, n104 | -3 | -3.4 |

Conclusion

Proposal 1: It is necessary to consider the worst ΔRIB,6R value among n77,n78,n79 and n104, if n104 is included in category n77, n78, n79

Proposal 2: it is necessary to consider different ΔRIB,6R value for handheld UE and non-handheld UE and consider the ΔRIB,6R relaxation for non-handheld UE about 0.2 ~0.4 dB.

Proposal 3: Proposed ΔRIB,6R values

| Operating band | Handheld ΔRIB,6R | Non-handheld ΔRIB,6R |
| --- | --- | --- |
| n41 | -3.5 | -3.7 |
| n77, n78, n79, n104 | -3 | -3.4 |

- References

# [1] R4-2410751, WF on requirements for 6Rx  (AT&T)

# [2] R4-2407988, Discussion for 6 Rx REFSENS (LGE)