3GPP TSG-RAN4 Meeting #112	R4-2412406

# Maastricht, NL, 19th Aug 2024 – 23rd Aug 2024

Source:	LG Electronics

Title:	Discussion for 6 Rx ΔTRxSRS

Agenda item:	8.1.1.3.3

Document for:	Discussion

Introduction

In the RAN4#111 meeting,  the WF on requirements for 6Rx was approved. In this discussion, we share our point of view for the ΔTRxSRS requirement based on the approved WF[1]

Discussion

Table 1 Way forward for 6 Rx SRS antenna switching and ΔTRxSRS [1]

Topic 2: SRS antenna switching and ΔTRxSRS Sub-topic 2-1: General considerations for SRS antenna switching and ΔTRxSRS Issue 2-1-1: SRS antenna switching configurations Way forward: RAN4 to focus on discussing ∆TRxSRS for 1tr6 and t2r6 until RAN1 concludes work on 3t6r and 4t6r. Pending RAN1 feedback, RAN4 can consider any necessary updates to the work scope in a Revised WID for the SRS antenna configurations subject to RAN Plenary approval. Issue 2-1-2: Whether to use different ∆TRxSRS based on operating frequency Way forward: RAN4 defines different ∆TRxSRS values based on operating frequency. Further discuss whether to introduce an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104. Sub-topic 2-2: ΔTRxSRS values Issue 2-2-1: Proposed ∆TRxSRS values Way forward: Further discuss the following options for the ∆TRxSRS requirements. - Option 1: Consider average value of ∆TRxSRS for further discussion. Operating bands; ΔTRxSRS requirement for t1r6; ΔTRxSRS requirement for t2r6; ΔTRxSRS requirement for t1r6-t2r6. Band n41, n77, n78; 4.0 dB; 3.4 dB; 4.4 dB. Band n79, [n104]; 5.3 dB; 4.5 dB; 5.7 dB. - Option 2: Consider range of ∆TRxSRS in brackets for each case below for further discussion. Operating bands; ΔTRxSRS requirement for t1r6; ΔTRxSRS requirement for t2r6; ΔTRxSRS requirement for t1r6-t2r6. Band n41, n77, n78; [2.8 ~ 5.0] dB; [2.3 ~ 4.0] dB; [3.3 ~ 5.5] dB. Band n79, [n104]; [3.6 ~ 6.0] dB; [3.0 ~ 5.5] dB; [4.2 ~ 6.5] dB. Note: Maximum level for t1r6 corrected for OPPO proposals per OPPO. - Option 3: Other options are not precluded.

ΔTRxSRS analysis

ΔTRxSRS analyses for t1r6, t2r6 and t1r6-t2r6 have been performed in this section. Insertion loss parameters in table 2 are used to analyse the ΔTRxSRS.

as described in detail in [2]

Table 2 insertion loss parameters (dB)

|  | n41 | n77/n78 | n79 | n104 |
| --- | --- | --- | --- | --- |
| SPDT | 0.5 | 0.5 | 0.6 | 0.7 |
| SP3T | 0.9 | 1.1 | 1.4 | 1.7 |
| SP4T | 1 | 1.2 | 1.5 | 2.1 |
| DP3T | 1.1 | 1.4 | 1.8 | 2.3 |
| SRS trace | 1.6 | 2 | 2.4 | 3 |

t1r6 architecture

Table 3 ΔTRxSRS for ‘t1r6’

|  | n41 | n77/n78 | n79 | n104 |
| --- | --- | --- | --- | --- |
| Main | 1 | 1.2 | 1.5 | 2.1 |
| Worst | 1+0.9+0.5+1.6 =4 | 1.2+1.1+0.6+2 =4.9 | 1.5+1.4+0.6+2.4 =5.9 | 2.1+1.7+0.7+3 =7.5 |
| delta | =3 | =3.7 | =4.4 | =5.4 |
| With 1.5 dB margin | 4.5 | 5.2 | 5.9 | 6.9 |

t2r6 architecture

Table 4 ΔTRxSRS for ‘t2r6’

|  | n41 | n77/n78 | n79 | n104 |
| --- | --- | --- | --- | --- |
| Main | 0.9 | 1.1 | 1.4 | 1.7 |
| Worst | 0.9+0.5+1.6 =3 | 1.1+0.5+2 =3.6 | 1.4+0.6+2.4 =4.4 | 1.7+0.7+3 =5.4 |
| delta | =2.1 | =2.5 | =3 | =3.7 |
| With 1.5 dB margin | 3.6 | 4 | 4.5 | 5.2 |

t1r6-t2r6 architecture

Table 5 ΔTRxSRS for ‘t1r6-t2r6’

|  | n41 | n77/n78 | n79 | n104 |
| --- | --- | --- | --- | --- |
| Main | 1 | 1.2 | 1.5 | 2.1 |
| Worst | 1+1.1+0.5+1.6 =4.2 | 1.2+1.4+0.5+2 =5.1 | 1.5+1.8+0.6+2.4 =6.3 | 2.1+2.3+0.7+3 =8.1 |
| delta | =3.2 | =3.9 | =4.8 | =6 |
| With 1.5 dB margin | 4.7 | 5.3 | 6.3 | 7.5 |

Analysis summary

Table 6 Summary on ΔTRxSRS analysis

|  | n41 | n77/n78 | n79 | n104 |
| --- | --- | --- | --- | --- |
| t1r6 | 3.5 | 4.2 | 5.9 | 6.9 |
| t2r6 | 2.6 | 3 | 4.5 | 5.2 |
| t1r6-t2r6 | 3.7 | 4.3 | 6.3 | 7.5 |

Proposal 1: Proposed ΔTRxSRS values

|  | n41 | n77/n78 | n79 | n104 |
| --- | --- | --- | --- | --- |
| t1r6 | 3.5 | 4.2 | 5.9 | 6.9 |
| t2r6 | 2.6 | 3 | 4.5 | 5.2 |
| t1r6-t2r6 | 3.7 | 4.3 | 6.3 | 7.5 |

Operation frequency effect on ΔTRxSRS

In clause 2.1.1, the ΔTRxSRS values for t1r6, t2r6 and t1r6t2r6 were analysed for all 6Rx example bands. The passive elements used in the analysis are highly influenced by the parasitic component, especially the parasitic capacitance, which causes the high insertion loss at the high frequency band. Therefore, the larger value of  ΔTRxSRS can be specified at the higher frequency band. In the case of 4Rx and 8Rx, higher  ΔTRxSRS was defined for the high frequency band considering their frequency characteristics. However, UHB band was not considered in ΔTRxSRS values of 4Rx and 8Rx. In the analysis in Clause 2.1.1, the difference between the ΔTRxSRS values of n79 and n104 is about 0.7~1.2 dB, and the following method can be applied in consideration of this difference.

Issue 2-1-2: Whether to use different ∆TRxSRS based on operating frequency

- Option 1: Consider the worst case ΔTRxSRS among n79 and n104 for high band ΔTRxSRS value

- Option 2: Add the one more break point for UHB ΔTRxSRS

Proposal 2: For Issue 2-1-2: , following options can be considered

Issue 2-1-2: Whether to use different ∆TRxSRS based on operating frequency

- Option 1: Consider the worst case ΔTRxSRS among n79 and n104 for high band ΔTRxSRS value

- Option 2: Add the one more break point for UHB ΔTRxSRS

Conclusion

Proposal 1: Proposed ΔTRxSRS values

|  | n41 | n77/n78 | n79 | n104 |
| --- | --- | --- | --- | --- |
| t1r6 | 3.5 | 4.2 | 5.9 | 6.9 |
| t2r6 | 2.6 | 3 | 4.5 | 5.2 |
| t1r6-t2r6 | 3.7 | 4.3 | 6.3 | 7.5 |

Proposal 2: For Issue 2-1-2: , following options can be considered

Issue 2-1-2: Whether to use different ∆TRxSRS based on operating frequency

- Option 1: Consider the worst case ΔTRxSRS among n79 and n104 for high band ΔTRxSRS value

- Option 2: Add the one more break point for UHB ΔTRxSRS

- References

# [1] R4-2410751, WF on requirements for 6Rx  (AT&T)

# [2] R4-2407989, Discussion for SRS antenna switching and ΔTRxSRS (LGE)