3GPP TSG-RAN WG4 Meeting #112	R4-2412407

Maastricht, Netherlands, 19th August – 23rd August 2024

Agenda item:	5.35

Source: 		NTT DOCOMO, INC.

Title: 	(TEI18)Discussion on PRACH demodulation impact of adding TDD bands for HAPS.

Document for:	Discussion

- Introduction

In the last meeting RAN4 #111, it was discussed that RAN4 impact of adding TDD band. We should specify PRACH format 1 for BS demodulation part to add TDD band for HAPS. In this contribution, we state the view of PRACH specification content.

- Discussion

HAPS (High Altitude Platform Station) is the radio stations operating in the stratosphere and effective in extending coverage. In the current BS specification TS38.104, the applied NR operating band is only FDD n1. From operator’s perspective, it is important to increase choices for HAPS band not only FDD but also TDD (e.g., Band and n34) based on the need taking the outcome of WRC-23. Increasing the number of supported bands, interference with other communication systems can be avoided.

During RAN4#111, it’s the common understanding the RAN4 should add PRACH format 1 for TDD band in TEI [1].

Observation 1: Adding TDD band (e.g., Band 34/n34) for HAPS is effective in extending coverage and the RAN4 impact on adding the TDD band is mainly PRACH format 1, it can be handled in TEI.

20 km

101.98 km

The necessary distance of HAPS-UE is more than 100km in terms of Physical roundness of the earth in figure 1.

Figure 1 The deployment scenario of HAPS-UE communication

The current NR TN specification can adapt the distance with SCS: 30kHz and the RAN4 possible impact includes HARQ process, TA (Timing Advance), GP (Guard period) and PRACH preamble as the follows.

With PRACH format 1(in TS38.211 and TS36.211) whose CP length is 684 µs, the applicable cell radius is more than 100km ((0.6849 - 0.3) ×Co/2=102.6867). RAN1 has already specify PRACH format 1 but RAN4 PRACH does not correspond with it as below. Thus, we need specify PRACH preamble 1.

TS38.104 Table 8.4.2.1-1: Time error tolerance for AWGN, TDLC300-100 and TDLA30-10

PRACH; PRACH SCS; Time error tolerance. preamble; (kHz); AWGN; TDLC300-100; TDLA30-10. 0; 1.25; 1.04 us; 2.55 us; N/A. A1, A2, A3, B4,; 15; 0.52 us; 2.03 us; 0.67 us. C0, C2; 30; 0.26 us; 1.77 us; 0.41 us.

We propose 2 options to specify PRACH format 1.

Proposal 1: Specify PRACH preamble 1 in either of the following options.

Option1: Considering multi path scenario, reuse TDLC300-100 for channel model.

Option2: Considering only single path scenario, reuse LTE burst format3 or specify new specification, AWGN + doppler shift.

TS36. 104 Table 8.4.2.1-1 PRACH missed detection requirements for Normal Mode

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR [dB] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 | Burst format 1 | Burst format 2 | Burst format 3 | Burst format 4 |
| 1 | 2 | AWGN | 0 | -14.2 | -14.2 | -16.4 | -16.5 | -7.2 |
|  |  | ETU 70 Low* | 270 Hz | -8.0 | -7.8 | -10.0 | -10.1 | -0.1 |
|  | 4 | AWGN | 0 | -16.9 | -16.7 | -19.0 | -18.8 | -9.8 |
|  |  | ETU 70 Low* | 270 Hz | -12.1 | -11.7 | -14.1 | -13.9 | -5.1 |
|  | 8 | AWGN | 0 | -19.8 | -19.4 | -21.5 | -21.3 | -11.8 |
|  |  | ETU 70 Low* | 270 Hz | -16.3 | -15.9 | -17.8 | -17.5 | -8.6 |
| Note*: Not applicable for Local Area BS and Home BS. |  |  |  |  |  |  |  |  |

HAPS radio wave reaches the UE as it hits building and other objects around the user, we should consider NLOS scenario from a strictly technical point of view, but most communication scenarios are LOS. We are fine to either option as long as we have an enough discussion.

- Conclusion

This contribution states general aspects of RAN4 impact on adding TDD band for HAPS. The summary is follows.

Observation 1: Adding TDD band (e.g., Band 34/n34) for HAPS is effective in extending coverage and the RAN4 impact on adding the TDD band is mainly PRACH format 1, it can be handled in TEI.

Proposal 1: Specify PRACH preamble 1 in either of the following options.

Option1: Considering multi path scenario, reuse TDLC300-100 for channel model.

Option2: Considering only single path scenario, reuse LTE burst format3 or specify new specification, AWGN + doppler shift.

References

- R4-2409121, (TEI18)Discussion on RAN4 impact of adding TDD bands for HAPS (High Altitude Platform Station), NTT DOCOMO, RAN4#111, May 2024