3GPP TSG-RAN WG4 Meeting #112	R4-2412408

Maastricht, Netherlands, Aug 19 – Aug 23, 2024

Source:	Nokia

Title:	(NR_newRAT-Perf) Discussion on PT-RS configuration in FR2 QPSK PDSCH demodulation requirements

Agenda item:	4.6

Document for:	Discussion

Introduction

During RAN4#111 an issue on PT-RS configuration in FR2 QPSK PDSCH demodulation requirements was raised and discussed [1]. The decision on this issue was ultimately postponed to RAN4#112, to give all contributors (and especially TE vendors) time to check internally.

In this contribution we discuss our views on the identified issue.

Discussion

Summary of issue and RAN4/RAN5 mismatch

There is a mismatch between RAN4 TS 38.101-4 and RAN5 TS 38.508-1 specifications concerning test setups for a number of FR2 QPSK PDSCH demodulation requirements. RAN4 seems to configure PT-RS for FR2 QPSK (MCS4) test cases, while RAN5 does not:

Note: We observe K and L are denoted “Frequency density (KPT-RS)” and “Time density (LPT-RS)” respectively.

Figure 1: PT-RS configuration in TS 38.101-4 FR2.

Derivation Path: Table 4.6.3-50. Information Element; Value/remark; Comment; Condition. DMRS-DownlinkConfig ::= SEQUENCE {. dmrs-AdditionalPosition; pos1. phaseTrackingRS; Not present; DEMOD_FR1. phaseTrackingRS SEQUENCE {; DEMOD_FR2. epre-Ratio; 0. resourceElementOffset; Offset10. }. }.

Figure 2: PT-RS configuration in TS 38.508-1 FR2.

Potential confusion causing RAN4/RAN5 mismatch

The mismatch between RAN4/RAN5 was potentially caused by the default configuration rules of PT-RS. From TS 38.214 (section 5.1.6.3) we see the default configuration rule of

[…] otherwise, if neither of the additional higher layer parameters timeDensity and frequencyDensity are configured and the RNTI equals MCS-C-RNTI, C-RNTI or CS-RNTI, the UE shall assume the PT-RS is present with LPT-RS = 1, KPT-RS = 2, and the UE shall assume PT-RS is not present when - the scheduled MCS from Table 5.1.3.1-1 is smaller than 10, or […]

Figure 3: TS 38.214 PT-RS default configuration.

Hence, if no PT-RS configuration is given in higher layer parameters (timeDensity/LPT-RS and frequencyDensity/KPT-RS), then no PT-RS is to be expected for MCS4/QPSK.

if no PT-RS configuration is given in higher layer parameters (timeDensity/LPT-RS and frequencyDensity/KPT-RS), then no PT-RS is to be expected for MCS4/QPSK.

However, since the configuration in RAN4 test cases matches the default configuration (LPT-RS = 1, KPT-RS = 2) it may have happened that RAN5 mistook the RAN4 intent to be “default configuration” and not “LPT-RS = 1, KPT-RS = 2”.

The PT-RS configuration in RAN4 test cases matches the default configuration (LPT-RS = 1, KPT-RS = 2) which may have led RAN5 understood the RAN4 intent to be “default configuration” and not “LPT-RS = 1, KPT-RS = 2”.

RAN4 intent and proposal

The remaining question now is what was RAN4’s intent in Rel-15. Either to use “default configuration” for PT-RS in FR2 QPSK, or to use the specific “LPT-RS = 1, KPT-RS = 2” configuration.	
If the RAN4 intent was “default” then RAN5 is correct and RAN4 needs to fix specification, if the RAN4 intent was “L=1 and K=2” then RAN5 needs to fix specifications.

In our understanding, there are at least three specific clarifications of the RAN4 intent being to configure “L=1 and K=2”.

Prior RAN4 agreements

In Rel-15 it was explicitly agreed to have PT-RS configured for FR2 requirements without out any mentioned exceptions. The “L=1 and K=2” configuration was explicitly chosen [2]:

[…] - Previous agreements - PTRS - PTRS is configured when introducing performance requirements in FR2 […] - Proposals for requirements definition - PTRS - FR1: PTRS is not configured when introducing performance requirements - FR2: PTRS configuration (port 1, per 2PRB in frequency domain, per symbol in time domain) [..]

Common test parameters specification

The TS 38.101-4 common test parameters specify a PT-RS configuration, it would have been left out, if default/no-higher-layer-signaling was intended:

[…] - PTRS configuration; - Frequency density (KPT-RS); - 2. - Time density (LPT-RS); - 1. - Resource Element Offset; - 2. [..]

RMC xOverhead settings

After the Rel-15 agreements, the second most direct statement of intent from RAN4 is the xOverhead setting in the RMCs for FR2 QPSK in TS 38.101-4:

[…] Table A.3.2.2.5-1: PDSCH Reference Channel for TDD UL-DL pattern FR2.120-1 and FR2.120-1A (QPSK) Parameter; Unit. Reference channel; R.PDSCH.5-1.1 TDD; R.PDSCH.5-1.2 TDD. Channel bandwidth; MHz; 100; 100. Subcarrier spacing; kHz; 120; 120. Allocated resource blocks; PRBs; 66; 66. Number of consecutive PDSCH symbols. For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159}; N/A; N/A. For Slot i, if mod(i, 5) = 3 for i from {0,…, 159}; 9; 2. For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159}; 13; 2. Allocated slots per 2 frames; 127; 127. MCS table; 64QAM; 64QAM. MCS index; 4; 4. Modulation; QPSK; QPSK. Target Coding Rate; 0.30; 0.30. Number of MIMO layers; 1; 1. Number of DMRS REs. For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159}; N/A; N/A. For Slot i, if mod(i, 5) = 3 for i from {0,…, 159}; 12; 6. For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159}; 12; 6. Overhead for TBS determination; 6; 0. Information Bit Payload per Slot. […]. [..]

The configuration of xOverhead=6 clearly shows that PT-RS is intended to be used with this RMC.	
For such RMCs RAN5 needs to correct their specification.

RAN4 shall decide what to do in xOverhead=0 cases. I.e., whether PT-RS configuration is “default” or “LPT-RS = 1, KPT-RS = 2”, and clarify RAN4 specification accordingly.

RAN4 WF/agreements, common test parameters specification, and non-zero RMC xOverhead settings, indicate that RAN4 intended for PT-RS to be configured and transmitted in certain FR2 QPSK requirements.

RAN4 intended for PT-RS to be configured with LPT-RS = 1, KPT-RS = 2 also for FR2 QPSK requirements, which have xOverhead not equal to 0. RAN4 shall send LS to RAN5 requesting configuration of PT-RS of corresponding test cases.

RAN4 shall decide what to do in xOverhead=0 cases. I.e., whether PT-RS configuration is “default” or “LPT-RS = 1, KPT-RS = 2”, and clarify RAN4 specification accordingly.

Conclusion

The paper discusses the topic of PTRS configuration in FR2 QPSK PDSCH demodulation requirements in RAN4.	
The following Observations and Proposals were made:

Observation 1: if no PT-RS configuration is given in higher layer parameters (timeDensity/LPT-RS and frequencyDensity/KPT-RS), then no PT-RS is to be expected for MCS4/QPSK.

Observation 2: The PT-RS configuration in RAN4 test cases matches the default configuration (LPT-RS = 1, KPT-RS = 2) which may have led RAN5 understood the RAN4 intent to be “default configuration” and not “LPT-RS = 1, KPT-RS = 2”.

Observation 3: RAN4 WF/agreements, common test parameters specification, and non-zero RMC xOverhead settings, indicate that RAN4 intended for PT-RS to be configured and transmitted in certain FR2 QPSK requirements.

Proposal 1: RAN4 intended for PT-RS to be configured with LPT-RS = 1, KPT-RS = 2 also for FR2 QPSK requirements, which have xOverhead not equal to 0. RAN4 shall send LS to RAN5 requesting configuration of PT-RS of corresponding test cases.

Proposal 2: RAN4 shall decide what to do in xOverhead=0 cases. I.e., whether PT-RS configuration is “default” or “LPT-RS = 1, KPT-RS = 2”, and clarify RAN4 specification accordingly.

- References

- R4-2409994, Ad-hoc minutes for [111][316] Demod_Maintenance, Moderator (Nokia), RAN4#111.

- R4-1809392, Way forward on parameters for PDSCH demodulation requirements, Intel, NTT DOCOMO, RAN4-AH-1807.