# 3GPP TSG-RAN WG4 Meeting #112                                                                R4-2412411

# Maastricht , Netherlands, 19th – 23th August, 2024

Title: 	Remaining open issues for NR CA/DC with 3MHz CBW

Source: 	Huawei, HiSilicon

Agenda item:	8.4.2

Document for:	Discussion

# Introduction

The WF [1] of the last meeting contained the following open issues, which are supposed to be discussed in this document.

### Issue 1-6: UE capability and network signalling - FFS whether any updates to Rel-18 UE capability signalling are needed to enable less than 5 MHz CBW operation for CA/DC. - Further potential enhancements can be further discussed for SCell with less than 5MHz. - FFS whether UEs needs to be informed that SCell has 12 PRB or 20 PRB transmission bandwidth for 3MHz or 5MHz CBW, respectively ### Issue 1-7: Sync raster applicability for Scell with < 5MHz CBW for CA_n100-n101 - FFS whether any restrictions on ARFCN for SSB locations for SCell SSB need to be defined - Option 1: Require less than 5 MHz SCell to be associated with the new sync raster points for less than 5MHz, similar as PCell. - Other options are not precluded

In the next session we first discuss Issue 1-7 as its discussion can be used for the other issue.

# Discussion

## Sync raster applicability for Scell with < 5MHz CBW for CA_n100-n101

In the last meeting the issue was raised in [2] stating the following

However, if the SCell with less than 5MHz is NOT associated with new sync raster points, the SSB bandwidth and the transmission bandwidth for SCell need to be known by the UE. If the carrierBandwidth for SCell (indicated by SCellConfig > ServingCellConfigCommon > DownlinkConfigCommon > FrequencyInfoDL > SCS-SpecificCarrier > carrierBandwidth) is 15PRBs for carrier of 3MHz (according to Table 5.3.2-1 [TS38.101-1]), the SSB bandwidth can be determined as 12PRBs, but the transmission bandwidth is still unclear between 15PRBs or 12PRBs. Similarly, if the carrierBandwidth for SCell is not for 3MHz but for 5MHz, the SSB bandwidth can be determined as 20PRBs, but the transmission bandwidth is still unclear between 25PRBs or 20PRBs.

In our opinion some clarifications to the above mentioned issue can be helpful. For the legacy sync raster, the SSB bandwidth is 20 PRB and transmission bandwidth is at least 25 PRB (for the min CBW 5MHz). Based on TS 38.101-1 Table 5.4.3.1-3 an additional sync raster for on n100 is considered at 921.45 MHz with a transmission bandwidth and SSB bandwidth of 20 PRB. For the new sync raster, in TS 38.101-1 Table 5.4.3.1-2, the transmission bandwidth is 15 PRB, in other words the SSB bandwidth is 15 PRB. Moreover on TS 38.101-1 Table 5.4.3.1-3 an additional sync raster for on n100 is considered at 920.73 MHz with a transmission bandwidth of 12 PRB, for the new sync raster. Therefore the transmission bandwidth and the SSB bandwidth on this sync raster point are of 12 PRB. Therefore, one can map a transmission bandwidth to an SSB bandwidth as shown in Table 1.

Table 1 Mapping between transmission BW and the SSB BW

|  | sync raster points (TS 38.101-1) | Transmission BW (carrierBandwidth) (PRB) | SSB BW (PRB) |
| --- | --- | --- | --- |
| Legacy sync raster | Table 5.4.3.1-1 | =>25 | 20 |
|  | Table 5.4.3.1-3 (921.45 MHz) | 20 | 20 |
| less than 5MHz (new) sync raster | Table 5.4.3.1-2 | 15 | 15 |
|  | Table 5.4.3.1-3 (920.73 MHz) | 12 | 12 |

Observation 1: For every SSB BW can be mapped to one or multiple transmission BW that do not overlap.

On another point the carrierBandwidth for every Scell is transmitted to the UE via the following signalling on Pcell.

RRCReconfiguration > masterCellGroup > CellGroupConfig > sCellToAddModList > SCellConfig > ServingCellConfigCommon > DownlinkConfigCommon > FrequencyInfoDL > SCS-SpecificCarrier > carrierBandwidth

Therefore the UE can be aware of the carrierBandwidth of the other Scells and, using Table 1, the SSB bandwidth (and even their exact ARFCN in case of 15 and 12 PRB) via Pcell and there is no need to more requirement to associate Scell with the new sync raster points.

Proposal 1: a UE can be aware of the carrierBandwidth of the other Scells and, using Table 1, the SSB bandwidth (and even their exact ARFCN values in case of 15 and 12 PRB) via Pcell and there is no need to add more requirements to associate Scell with the new sync raster points.

Table 1 Mapping between transmission BW and the SSB BW

|  | sync raster points (TS 38.101-1) | Transmission BW (carrierBandwidth) (PRB) | SSB BW (PRB) |
| --- | --- | --- | --- |
| Legacy sync raster | Table 5.4.3.1-1 | =>25 | 20 |
|  | Table 5.4.3.1-3 (921.45 MHz) | 20 | 20 |
| less than 5MHz (new) sync raster | Table 5.4.3.1-2 | 15 | 15 |
|  | Table 5.4.3.1-3 (920.73 MHz) | 12 | 12 |

## UE capability and network signalling

In Rel-18, 3MHz CBW was only specified for single carrier operations, where UE reports support3MHz-ChannelBW-Symmetric-r18 or/and support3MHz-ChannelBW-Asymmetric-r18, which correspond to symmetric or asymmetrical CBW configurations, respectively. For Rel-19 this condition needs to be removed to be able to have NR CA/DC with 3MHz. Either these UE capabilities can be updated or new dedicated UE capabilities can be defined for Rel19, which are better to be left to RAN2 to decide.

Proposal 2: To enable NR CA/DC with 3MHz CBW, in Rel-19, either support3MHz-ChannelBW-Symmetric-r18 or/and support3MHz-ChannelBW-Asymmetric-r18 need to be updated or new Rel-19 UE capabilities need to be defined. RAN2 should decide on which path to take.

Proposal 3: Send an LS to RAN2 to inform them of the Rel-19 UE capabilities that RAN4 need.

# Conclusions

Proposal 1: a UE can be aware of the carrierBandwidth of the other Scells and, using Table 1, the SSB bandwidth (and even their exact ARFCN values in case of 15 and 12 PRB) via Pcell and there is no need to add more requirements to associate Scell with the new sync raster points.

Table 1 Mapping between transmission BW and the SSB BW

|  | sync raster points (TS 38.101-1) | Transmission BW (carrierBandwidth) (PRB) | SSB BW (PRB) |
| --- | --- | --- | --- |
| Legacy sync raster | Table 5.4.3.1-1 | =>25 | 20 |
|  | Table 5.4.3.1-3 (921.45 MHz) | 20 | 20 |
| less than 5MHz (new) sync raster | Table 5.4.3.1-2 | 15 | 15 |
|  | Table 5.4.3.1-3 (920.73 MHz) | 12 | 12 |

Proposal2: To enable NR CA/DC with 3MHz CBW, in Rel-19, either support3MHz-ChannelBW-Symmetric-r18 or/and support3MHz-ChannelBW-Asymmetric-r18 need to be updated or new Rel-19 UE capabilities need to be defined. RAN2 should decide on which path to take.

Proposal 3: Send an LS to RAN2 to inform them of the Rel-19 UE capabilities that RAN4 needs.

# Reference

[1]	“R4-2410602, WF on UE RF requirements for Rel-19 NR channel BW less than 5MHz for FR1 Phase 2, Intel.”

[2]	“R4-2408815, Scell bandwidth and sync raster, Qualcomm.”