3GPP TSG-RAN WG4 Meeting #112	    R4- 2412415

Maastricht, Netherlands, 19th – 23rd August 2024

Agenda item:	8.4.3

Source:	Intel Corporation

Title:	RRM scope for Rel-19 less than 5MHz work item phase 2

Document for:	Discussion

# Introduction

The Rel-19 NR channel BW less than 5MHz for FR1 Phase 2 work item was updated in June 2024 in [1]. The work item Core part objectives are provided below.

- Define common co-located and non-co-located inter-band NR CA/DC UE RF requirements with 3MHz CBW in the one band and 5MHz or 10MHz CBW in the other band - Example band combination: CA/DC of 3MHz in band n100 and 5MHz or 10MHz in band n101 - Define RRM requirements for inter-band CA and DC for combinations introduced in RF part Note: other band combinations than example band combinations can be specified in basket WIs after the above generic requirements are specified.

The idea of the objectives is to first define generic RF requirements for inter-band NR-CA and NR-DC n100-n101 band combinations considering less than 5MHz bandwidth in FDD band n100 and 5 MHz or 10 MHz in TDD band n101, and specify general RF requirements/applicability for inter-band CA/DC. RRM work is scheduled to start from August WG meeting 2024 according to the RAN4 TU allocation.

In this paper we provide our observations on the RRM scope of this item.

# RRM scope

## Carrier aggregation with 12 or 15 PRB BW carrier

Regarding supporting carrier aggregation when one of the bands in the combination is configured with less than 5MHz channel bandwidth, we need to specify new UE requirements for SCell operations targeting an SCell with smaller SSB bandwidth than those in the legacy specifications.

In Rel-18, consensus was that the measurement performance is not impacted when the target SSB has smaller BW than 5MHz but only the PBCH bandwidth was reduced. However, the detection delay of a cell with 12 or 15 PRB bandwidth is relaxed when SSB index detection is necessary in the process.

In the Rel-18 specification, the SSB index detection delay is relaxed in the below manner – comparison is shown in the below box between T_SSB_time_index_intra and T_SSB_time_index_intra_less_than_5Mhz.

Table 9.2.6.2-3: Time period for time index detection (Frequency range FR1) DRX cycle; TSSB_time_index_intra. No DRX; max(120ms, ceil(3 x Kgap ) x max(MGRP, SMTC period)) x CSSFintra. DRX cycle≤ 320ms; max(120ms, ceil(M2Note 1x 3 x Kgap) x max(MGRP, SMTC period,DRX cycle) x CSSFintra). DRX cycle>320ms; Ceil(3 x Kgap )x max(MGRP, DRX cycle) x CSSFintra. NOTE 1: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. NOTE 2: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 3: For a UE supporting concurrent GAPs, if multiple concurrent GAPs are configured, the MGRP is the periodicity of the activated Pre-MG or the MG pattern associated to the intra-frequency layer. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured, the requirements apply to UE on measurements of secondary component carrier with active SCell.. Table 9.2.6.1-12: Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (Frequency range FR1) (Frequency range FR1) DRX cycle; TSSB_time_index_intra_less_than_5Mhz. No DRX; max(120ms, 7 x max(MGRP, SMTC period)) x CSSFintra_less_than_5Mhz. DRX cycle≤ 320ms; max(120ms, ceil(1.5 x 7) x max(MGRP, SMTC period,DRX cycle) x CSSFintra_less_than_5Mhz). DRX cycle>320ms; 7 x max(MGRP, DRX cycle) x CSSFintra_less_than_5Mhz. Note 1: CSSFintra_less_than_5Mhz is 1 NOTE 2: FFS When highSpeedMeasInterFreq-r17.

Similarly, we need to specify the time period requirements for time index detection for UE operating on a target cell with less than 5MHz SSB for deactivated SCell measurements in FR1.

Proposal 1: Specify time period requirements for time index detection for UE operating on a target cell with less than 5MHz SSB for deactivated SCell measurements in FR1.

In the SCell activation delay requirements, fine time tracking after search of the target cell is considered and when the target SSB BW is smaller than 5MHz the legacy requirement does not apply. This is to say that we need to specify new requirements for fine time tracking time which is included in the SCell activation delay requirements.

To help understand, fine time tracking time for less than 5MHz target HO requirements is copied in the below box.

When intra-frequency or inter-frequency handover is commanded, the interruption time shall be less than Tinterrupt Tinterrupt = Tsearch + TIU + Tprocessing + T∆ + Tmargin ms Where: - If the target cell is an unknown inter-frequency cell and Tsearch = 3* Trs ms if the target cell Es/Iot≥-2 dB. T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = Trs for both known and unknown target cells operating with 20 PRB SSB BW. T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW.

Proposal 2: Specify new requirements for fine time tracking time which is included in the SCell activation delay requirements in FR1.

## NR-DC

In the scope of NR-DC when PSCell is configured with less than 5MHz channel bandwidth, RAN4 needs to specify new PSCell addition delay requirements.

Similar to handover and SCell activation, the fine time tracking delay needs to be relaxed.

Proposal 3: Specify new PSCell addition delay requirements for PSCell configured with less than 5MHz channel bandwidth.

Automatically, the PSCell change delay requirements follow the PSCell addition delay requirements.

# Conclusion

In this paper we provide our observations on the RRM scope of this item.

Proposal 1: Specify time period requirements for time index detection for UE operating on a target cell with less than 5MHz SSB for deactivated SCell measurements in FR1.

Proposal 2: Specify new requirements for fine time tracking time which is included in the SCell activation delay requirements in FR1.

Proposal 3: Specify new PSCell addition delay requirements for PSCell configured with less than 5MHz channel bandwidth.

# References

- RP-241622, Revised WID: NR channel BW less than 5MHz for FR1 Phase 2, Intel Corporation