3GPP TSG-RAN WG4 Meeting #112		R4-2412418

Maastricht, Netherlands, 19th – 23rd August 2024

Agenda item:	8.15.2.2

Source:	Intel Corporation

Title:	Discussion on delay reduction by optimizing CSSF outside gap

Document for:	Discussion

# Introduction

The Rel-19 NR RRM enhancements work item was approved in March 2024 in [1]. The work item descriptions of objectives of the item are shown in the below box.

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - For UE not in multiple-Rx simultaneous reception mode: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

In the last WG meeting, a WF was approved in [2] to capture all the discussions and agreements.

In this paper we provide our analysis on delay reduction by optimizing CSSF outside gap.

# Discussion

## CSSF outside gap in CA/DC operations

Applicability

One of the motivations to have this general enhancement for all FR2-1 UE CSSF outside gap is that the intra-band carriers in FR2 share the most similar characteristics as targets to SSB based L3 measurements. There is a consensus in RAN4 since Rel-15 that for FR2 target cells of a certain band, the UE chooses target cells within a single frequency layer for neighbour cell measurements. This is specified in TS 38.133 clause 9.2.3.2.

For one single intra-frequency layer in a band, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least: - 6 identified cells, and - 24 SSBs with different SSB index and/or PCI, where this single intra-frequency layer shall be: - PCC when UE is configured with SA NR operation mode with PCC in the band; or - PSCC when UE is configured with EN-DC with PSCC in the band; or - PSCC when UE is configured with NR-DC with PSCC in the band; or - One of the SCCs on which UE is configured to report SSB based measurements when neither PCC nor PSCC is in the same band, so that the selected SCC shall be an SCC where the UE is configured with SS-RSRP measurement reporting if such SCC exists, otherwise the selected SCC is determined by UE implementation. The UE shall also be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least 2 SSBs on serving cell for each of the other intra-frequency layer(s) in the same band.

Based on this consensus, for FR2 measurements, there seems no need to scale the measurement delay by number of carriers within the same band for each serving SCC. The UE measures on one carrier of a band in FR2 and all other carriers in the band is considered as known and measured. The measurement results can be shared among all the intra-band carriers. Note that there are indeed preconditions for this enhancement to apply to the existing requirements for FR2-1 SSB based L3 measurement delay.

CSSF outside gap for FR2 CA

CSSF outside gap for FR2 CA is specified in TS 38.133 Table 9.1.5.1.2-1. Since only SSB is considered in this work item scope we eliminate the factors for other reference signals and measurements and translate the table into the below one. Of all the highlighted parts, the measurement delay is scaled by N_(SCC_SSB) which is the number of configured SCells with SSB based L3 measurements without gap.

| Scenario | CSSFoutside_gap,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSF outside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gap | CSSFoutside_gap,i for E-UTRA inter-RAT MO with no measurement gap [TBD] |
| --- | --- | --- | --- | --- | --- |
| FR2 only intra band CA | 1 | N/A | NSCC_SSB +Y+Z | NSCC_SSB +Y+Z | NSCC_SSB +Y+Z |
| FR2 only inter band CA | 1 | 2Note 3,5 | 2×( NSCC_SSB +Y+Z-1) | 2×( NSCC_SSB +Y+Z-1) | 2×( NSCC_SSB +Y+Z-1) |
| FR1 +FR2 CA (FR1 PCell) Note 1 | N/A | 2Note 3,5 | 2×( NSCC_SSB +Y+Z-1) | 2×( NSCC_SSB +Y+Z-1) | 2×( NSCC_SSB +Y+Z-1) |
| FR1 +FR2 CA (FR2 PCell) Note 1 | 1 | N/A | NSCC_SSB +Y+Z | NSCC_SSB +Y+Z | NSCC_SSB +Y+Z |
| Note 1: Only one FR1 operating band and one FR2 operating band are included for FR1+FR2 inter-band CA. Note 2: Selection of FR2 SCC where neighbour cell measurement is required follows clause 9.2.3.2. Note 3: CSSFoutside_gap,i =1 if only one SCell is configured and no inter-frequency MO without gap and only SSB based L3 measurement is configured on SCC; CSSFoutside_gap,i =2 if only one SCell is configured and no inter-frequency MO without gap and either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement is configured on SCC. Note 4: Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG; otherwise, it is 0. Note 5: Only two NR FR2 operating bands are included for FR2 inter-band CA. Note 9: NSCC_SSB=Number of configured SCell(s) with only SSB based L3 measurement configured, which is measured without MG. Note 11: Z is the number of configured E-UTRA inter-RAT MOs without MG that are being measured outside of MG; otherwise, it is 0. |  |  |  |  |  |

We propose to enhance the scaling factor for the highlighted contents so that the UE is required to only measure one of the FR2 carriers and share the results among all for each band when carrying out SSB based FR2 L3 measurements.

Proposal 1: RAN4 enhances the scaling factor of CSSF outside gap so that the UE is required to only measure one of the FR2 carriers and share the results among all for each band when carrying out SSB based FR2 L3 measurements.

In general, we think N_(SCC_SSB) = 0 for FR2 only intra-band CA and N_(SCC_SSB) = 1 for FR2 only inter-band CA.

CSSF outside gap for DC

Regarding NR-DC, there is no intra-band cases and FR2 inter-band NR-DC is considered exactly the same with FR2 inter-band CA.

Proposal 2: Regarding NR-DC, requirements follow CA cases.

Regarding EN-DC, we don’t see any urgent need to introduce enhancements for EN-DC at this stage and we propose to deprioritize EN-DC scope in this work item.

Proposal 3: Deprioritize EN-DC scope in this work item.

Number of searchers is two

It is agreed in the WID that the baseline assumption for number of searchers is two searchers for a UE. We can consider enhancements on the number of searchers in a future release beyond Rel-19.

# Conclusion

In this paper we provide our views on the scope of FR2-1 SSB based L3 measurement delay reduction for connected mode.

Proposal 1: RAN4 enhances the scaling factor of CSSF outside gap so that the UE is required to only measure one of the FR2 carriers and share the results among all for each band when carrying out SSB based FR2 L3 measurements.

Proposal 2: Regarding NR-DC, requirements follow CA cases.

Proposal 3: Deprioritize EN-DC scope in this work item.

# References

- RP-240830, New WID: WI resulting from discussion on RRM enhancements, Moderator (vivo)

- R4-2410260, Way Forward for [111][228] NR_RRM_Ph5, Apple