3GPP TSG-RAN WG4 Meeting #112		R4-2412419

Maastricht, Netherlands, 19th – 23rd August 2024

Agenda item:	8.21.2

Source:	Intel Corporation

Title:	RRM scope for Rel-19 network energy saving enhancements

Document for:	Discussion

# Introduction

The Rel-19 NR network energy saving enhancements work item was approved in [1]. The work item Core part objectives are provided below.

The objectives of the work item are the following: - Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] - Triggering method by uplink wake-up-signal using an existing signal/channel. - Wake-up-signal configuration provisioning to UE - Note: No modification of SSB will be discussed under this objective - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. - Checkpoint for normative work in RAN#105 - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

There are three major parts in the agreed objectives in the item: On-demand SSB in CA, On-demand SIB1 and adaptation of common signals. According to the work plan and TU allocation of the item, RAN4 work starts in this meeting.

In this paper we provide the initial observations and proposals regarding the possible RAN4 RRM scope of the work item which in general are indicated in the work item description.

# RRM scope discussion

## On-demand SSB in SCell

In general this part of the discussion is about the On-demand SSB that is used by UE for SCell time/frequency synchronization and L1 or L3 measurements during or outside SCell activation when the UE is configured with carrier aggregation operations.

In RAN4, it is natural that we look at the activation procedure and its corresponding RRM requirements first and then move to discuss the serving cell measurement requirements enhancements after CA is configured to the UE (the measurements enhancements based on On-demand SSB aim to serve the activation procedure eventually) or after SCell activation is complete.

RAN1 discussed two scenarios and we summarize the agreements in the below box. These scenarios are the references for further RAN4 specifications.

For the identified scenarios and cases on-demand SSB can be triggered/indicated by gNB at least for the following scenarios: Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321) Scenario #2A refers to “When UE receives SCell activation command (e.g., as defined in TS 38.321)” Scenario #3: After UE receives SCell activation command (e.g., as defined in TS 38.321): This does not preclude SCell for which activation is completed Scenario #3A refers to “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” Scenario #3B refers to “When SCell activation is completed and SCell is activated” or “After SCell activation is completed and SCell is activated”

Correspondingly we can identify the below aspects of RAN4 RRM specification impacts:

- Enhanced SCell activation delay requirements based on On-demand SSB

  - Enhanced L3 measurement delay

  - Enhanced L1 measurement delay

- Enhanced deactivated SCell measurement delay requirements based on On-demand SSB

- Enhanced intra-frequency (activated SCell) measurement delay requirements based on On-demand SSB

The below figure helps to understand the RRM scope regarding On-demand SSB enhancements in RAN4.

Figure 1 Correspondence between RAN1 scenarios and RRM scope

L1 and L3 measurements based on On-demand SSB during SCell activation

RAN1 confirms that:

- For a cell supporting on-demand SSB SCell operation, - L1 and/or L3 measurement based on on-demand SSB is supported for the cell. - FFS further details on L1 and/or L3 measurement

During a typical activation procedure, the UE needs to get the L3 results on the target SCell first and L1 measurements are carried out on the SCell for better beam information. One of the typical applications of On-demand SSB is to trigger transmissions together with SCell activation command and to seize transmissions after receiving UE activation CSI report. It is possible that during the whole process we could allow the UE to measure on On-demand SSB-s for either L3 or L1 measurements or both.

It is likely that RAN1 confirms reusing physically layer designs for the On-demand SSB-s for activation procedure. RAN4 specifies the enhanced L3 and L1 measurements during the activation based on the On-demand SSB configurations.

Proposal 1: RAN4 specifies SCell activation delay requirements considering L3 and L1 measurements based on On-demand SSB configurations during the SCell activation process.

Serving cell measurements based on On-demand SSB

In the above sector we discussed about scenarios #2A and #3A which are about enhancements during the SCell activation process.

Regarding scenario #2, the idea is to allow On-demand SSB transmissions before the activation command where the SCell is a deactivated SCell. Meanwhile the UE carries out deactivated SCell measurements based on On-demand SSB. RAN4 specifies the enhanced deactivated SCell measurement requirements based on On-demand SSB for this case.

Proposal 2: RAN4 specifies deactivated SCell measurement requirements based on On-demand SSB.

Regarding scenario #3B, the idea is to allow On-demand SSB transmissions after the activation is completed. Under such case, the UE is allowed to carry out intra-frequency measurements on the activated serving cell based on On-demand SSB. RAN4 specifies the enhanced intra-frequency measurement requirements based on On-demand SSB.

Proposal 3: RAN4 specifies intra-frequency measurement requirements based on On-demand SSB.

No always-on SSB transmission

RAN1 identified the following two cases in their discussions.

- Case #1: No always-on SSB on the cell - Case #2: Always-on SSB is periodically transmitted on the cell

RAN4 will be looking at the scenarios where On-demand SSB is configured in the SCell due to energy saving purposes and seek to minimize the impact of less SSB transmissions and to enhance the performance by allowing the UE to measure on the On-demand SSB specifically transmitted for the SCell activation procedure.

From RAN4 perspective, it is possible that at least for some of the scenarios there is no always-on SSB on the target SCell. SSB-less operations are applicable to intra-band and inter-band carrier aggregations already in the existing specifications (intra-band NCCA activation requirement is in progress). With the help of On-demand SSB, we see great benefits as it completes the SSB-less SCell systematically.

Observation: From RAN4 perspective, it is possible that at least for some of the scenarios there is no always-on SSB on the target SCell.

## On-demand SIB1 enhancements

PRACH like request for On-demand SIB1

RAN1 is currently debating on the On-demand SIB1 design. Gain evaluations are carried out to confirm the benefits of this feature in general. The idea of the feature is to allow the UE to read On-demand SIB1 messages instead of mandate the network to transmit SIB1 messages in all the carriers.

PRACH has been selected as the baseline for considering the UE request carrier for UL wake up signals.

Requirements for UE triggered On-demand SIB1 procedures

In RAN4 the specification of UL-WUS is necessary if RAN1 confirms to specify the On-demand SIB1 features. In the CONNECTED mode mobility requirements, a new section is expected to be added for the UL-WUS transmission requirements for the UE.

Proposal 4: Wait until RAN1 confirms On-demand SIB1 feature for RAN4 to start working on requirements for UL-WUS.

## SSB and PRACH adaptation

A summary of RAN1 agreements on the SSB and PRACH adaptation can be found in this sector and we propose to wait until the concrete conclusion of RAN1 is available that RAN4 works on details of the requirements.

Regarding SSB adaptation => Agreement For adaptation of SSB in time-domain, consider the following adaptation mechanisms for further study - Adaptation of SSB burst periodicity - Adaptation based on two SSB configurations where up to two configurations can be active - Adaptation based on skipping/transmitting some SSB bursts non-uniformly with single SSB configuration - Adapting the transmitted number of SSBs within a SSB burst - Cell DTX for SSB adaptation - Whether to support new SSB burst periodicity value(s) - Whether to support new SSB burst(s) (i.e. how SSB transmission is made within a burst) - New compact SSB burst(s) - Adapting the position of SSBs within a SSB burst - Other mechanisms/combinations are not precluded => Agreement For the adaptation mechanisms of SSB in time-domain, study further applicable scenarios and associated legacy UE impact/handling (if any) based on the following: - Applicability to UE in idle/inactive and/or connected mode - Applicability to PCell and/or SCell(s) => Agreement For the adaptation mechanisms of SSB in time-domain, study further following mechanisms: - Adaptation mechanism indicated or configured by gNB without UE trigger - Adaptation triggered by UE (if any) FFS: Details of associated signaling/indication/configuration Regarding PRACH adaptation => Agreement For adaptation of PRACH in time-domain, consider the following adaptation mechanisms for further study - Adaptation based on configuration of additional[/different] PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any) - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs - For the additional PRACH resources, - Adaptation of PRACH resource periodicity/PRACH occasion - Adaptation at PRACH configuration/association period/association pattern period level and SSB to RO mapping cycle - Adaptation based on extending cell DRX operation for PRACH - Concentrating ROs in time domain - Other options are not precluded => Agreement For the adaptation mechanisms of PRACH in time-domain - Support at least PRACH adaptation provided by gNB without UE trigger - FFS: PRACH adaptation with UE trigger - Note: UE trigger means UE requests adaptation of PRACH - Study at least the following, - Dynamic signaling and/or semi-static signaling of PRACH adaptation - Adaptation of PRACH transmission according to certain condition - Applicability to idle/inactive and/or connected mode UEs - Which scenarios the adaptation mechanism is applicable to (e.g. cell with both legacy and Rel-19 UE, cell with only Rel-19 UEs)

Proposal 5: Wait until the concrete conclusion of RAN1 is available that RAN4 works on details of the requirements.

# Conclusion

In this paper we provide the initial observations and proposals regarding the possible RAN4 RRM scope of the work item which in general are indicated in the work item description.

Proposal 1: RAN4 specifies enhanced SCell activation delay requirements considering L3 and L1 measurements based on On-demand SSB configurations during the SCell activation process.

Proposal 2: RAN4 specifies enhanced deactivated SCell measurement requirements based on On-demand SSB.

Proposal 3: RAN4 specifies enhanced intra-frequency measurement requirements based on On-demand SSB.

Observation: From RAN4 perspective, it is possible that at least for some of the scenarios there is no always-on SSB on the target SCell.

Proposal 4: Wait until RAN1 confirms On-demand SIB1 feature for RAN4 to start working on requirements for UL-WUS.

Proposal 5: Wait until the concrete conclusion of RAN1 is available that RAN4 works on details of the requirements.

# References

- RP-234065, New WID: Enhancements of network energy savings for NR, Moderator (Ericsson)