3GPP TSG-RAN4 Meeting #112	R4-2412427

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4824 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for Less Than 5MHz Performance part |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR1_lessthan_5MHz_BW-Perf |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Updating test parameters based on the latest Specification version. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removing [ ] and TBDs from the following test cases: A.6.1.1.9 Cell reselection to FR1 intra-frequency NR case for UE operating on a cell with less than 5MHz BW A.6.3.1.18 Intra-frequency handover from FR1 to FR1; unknown target cell operating with 12 PRB SSB bandwidth A.6.5.1.13 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for UE operating on a cell with less than 5MHz BW A.6.5.1.14 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for UE operating on a cell with less than 5MHz BW A.6.5.5.8 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for a UE operating on a cell with less than 5 MHz BW A.6.6.1.12 SA event triggered reporting tests without gap under non-DRX with SSB index reading and 12 PRB SSB A.6.6.2.15 SA event triggered reporting tests for FR1 with 3MHz Channel Bandwidth configured without SSB time index detection when DRX is used |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification remains incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.1.1.9 A.6.3.1.18 A.6.5.1.13 A.6.5.1.14 A.6.5.5.8 A.6.6.1.12 A.6.6.2.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change 1>

#### A.6.1.1.9	Cell reselection to FR1 intra-frequency NR case for UE operating on a cell with less than 5MHz BW

##### A.6.1.1.9.1	Test Purpose and Environment

This test is to verify the requirement for the intra frequency NR cell reselection requirements specified in clause 4.2.2.3. for UE capable of operating on a cell with less than 5MHz BW.

Supported test configurations are specified in Table A.6.1.1.9.2-1. General test parameters as specified in Table A.6.1.1.1.2-2 with config 1 apply except those specified in Table A.6.1.1.9.2-2. Cell specific test parameters as specified in Table A.6.1.1.1.2-3 apply. The test parameters specified in A.6.1.1.1.2 applies to this test. The test procedure specified in A.6.1.1 applies to this test.

Table A.6.1.1.9.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | FDD duplex mode, 15 kHz SSB SCS, 3 MHz bandwidth |

Table A.6.1.1.9.2-2: General test parameters for intra frequency NR cell re-selection test case

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 |  |
| BWchannel | MHz | Config 1 | 3: NRB,c = 12 |  |
| SSB Configuration |  | Config 1 | SSB.13 |  |
| PRACH Configuration index |  | Config 1 | 1 |  |

<End  of change 1>

<Start of change 2>

#### A.6.3.1.18	Intra-frequency handover from FR1 to FR1; unknown target cell operating with 12 PRB SSB bandwidth

##### A.6.3.1.18.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR1-NR FR1 intra frequency handover requirements for unknown target cell operating with 12 PRB SSB bandwidth specified in clause 6.1.1.2.

##### A.6.3.1.18.2	Test Parameters

Supported test configurations are shown in table A.6.3.1.18.2-1. General test parameters as specified in A.6.3.1.2.2-2 with config 1 apply except those specified in A.6.3.1.18.2-2. Cell specific test parameters as specified in A.6.3.1.2.2-3 with config 1 apply except those specified in A.6.3.1.18.2-3.

The test procedure specified in A.6.3.1.2.2 applies to this test. The cell 2 is the unknown target cell operating with 12 PRB SSB bandwidth.

Table A.6.3.1.18.2-1: Intra-frequency handover from FR1 to FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 3 MHz bandwidth with 15PRB CBW, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 3 MHz bandwidth with 15PRB CBW, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 3 MHz bandwidth with 12PRB CBW, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 3 MHz bandwidth with 12PRB CBW, FDD duplex mode |
| Note1: The UE is only required to be tested in one of the supported test configurations Note2: Configuration 2 is only applied for the testing on band n100 |  |

Table A.6.3.1.18.2-2: General test parameters Intra-frequency handover from FR1 to FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 | unknown target cell operating with 12 PRB SSB bandwidth |
| Final condition | Active cell |  | Cell 2 | unknown target cell operating with 12 PRB SSB bandwidth |

Table A.6.3.1.18.2-3: Cell specific test parameters for NR FR1-FR1 Intra frequency handover test case

| Parameter |  |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| BWchannel |  | Config 1 | MHz | 3: NRB,c = 15 |  |  |  |
|  |  | Config 2 |  | 3: NRB,c = 12 |  |  |  |
| BWP BW |  | Config 1 | MHz | 3: NRB,c = 15 |  |  |  |
|  |  | Config 2 |  | 3: NRB,c = 12 |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.2 FDD |  |  |  |
|  |  | Config 2 |  | SR.1.2 FDD |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CCR.1.6 FDD |  |  |  |
|  |  | Config 2 |  | CCR.1.7 FDD |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.13 |  |  |  |
| IoNote1 | Config 1 |  | dBm/ 2.7MHz | -66.81 | -64.11 | -66.81 | -64.11 |
|  | Config 2 |  | dBm/ 2.16MHz | -67.78 | -65.07 | -67.78 | -65.07 |
| Propagation condition |  |  | - | AWGN |  | AWGN |  |
| Note 1: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

<End of change 2>

<Start of change 3>

#### A.6.5.1.13	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for UE operating on a cell with less than 5MHz BW

##### A.6.5.1.13.1	Test Purpose and Environment

The purpose of this test is to verify that the UE supporting [FG-x capability] properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell operating on a 3MHz channel bandwidth. This test will partly verify the FR1 radio link monitoring requirements in clause 8.1.

Supported test configurations are specified in Table A.6.5.1.13.1-1. General test parameters as specified in Table A.6.5.1.3.1-2 with config 1 apply except those specified in Table A.6.5.1.13.1-2. Cell specific test parameters as specified in Table A.6.5.1.3.1-3 apply except those specified in Table A.6.5.1.13.1-3.

The test procedure specified in A.6.5.1.3.1 applies to this test.

Table A.6.5.1.13.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD duplex mode, 15 kHz SSB SCS, 3 MHz bandwidth |

Table A.6.5.1.13.1-2: General test parameters for FR1 OOS 12 PRB in DRX mode

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| BWchannel | Config 1 | MHz | 3: NRB,c = 12 |
| RMSI CORESET Reference Channel | Config 1 |  | CR.1.2 FDD |
| Dedicated CORESET Reference Channel | Config 1 |  | CCR.1.6 FDD |
| SSB Configuration | Config 1 |  | SSB.13 FR1 |
|  | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 2 |
| In sync transmission parameters | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 4 |
| Out of sync transmission parameters | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 4 |
|  | REG bundle size |  | 6 |
|  | CP length |  | Normal |
|  | Mapping from REG to CCE |  | Distributed |

Table A.6.5.1.13.1-3: Cell specific test parameters for FR1 OOS 12 PRB in DRX mode

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| SNR_SSB of set q0 | Config 1 | dB | 1 | -7 | -15 |
| SNR_SSB of set q1 | Config 1 | dB | 1 | -7 | -15 |
| SSB_RP of set q1 | Config 1 | dBm/SCS kHz | -98 | -98 | -98 |

##### A.6.5.1.13.2	Test Requirements

Test requirements specified in Clause A.6.5.1.3.2 apply to this test.

<End  of change 3>

<Start of change 4>

#### A.6.5.1.14	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for UE operating on a cell with less than 5MHz BW

##### A.6.5.1.14.1	Test Purpose and Environment

The purpose of this test is to verify that the UE supporting [FG-x capability] properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell operating on a 3MHz channel bandwidth. This test will partly verify the FR1 radio link monitoring requirements in clause 8.1.

Supported test configurations are specified in Table A.6.5.1.14.1-1. General test parameters as specified in Table A.6.5.1.1.1-2 with config 1 apply except those specified in Table A.6.5.1.14.1-2. Cell specific test parameters as specified in Table A.6.5.1.1.1-3 apply except those specified in Table A.6.5.1.14.1-3. The test procedure specified in A.6.5.1.1 applies to this test.

Table A.6.5.1.14.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD duplex mode, 15 kHz SSB SCS, 3 MHz bandwidth |

Table A.6.5.1.14.1-2: General test parameters for FR1 OOS 15 PRB in DRX mode

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| BWchannel | Config 1 | MHz | 3: NRB,c = 15 |
| RMSI CORESET Reference Channel | Config 1 |  | CR.1.3 FDD |
| Dedicated CORESET Reference Channel | Config 1 |  | CCR.1.7 FDD |
| SSB Configuration | Config 1 |  | SSB.13 FR1 |
|  | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 3 |
|  | Aggregation level | CCE | 4 |
| In sync transmission parameters | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 0 |
|  | Number of Control OFDM symbols |  | 3 |
|  | Aggregation level | CCE | 8 |
| Out of sync transmission parameters | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 4 |
|  | REG bundle size |  | 6 |
|  | CP length |  | Normal |
|  | Mapping from REG to CCE |  | Non-Distributed |

Table A.6.5.1.14.1-3: Cell specific test parameters for FR1 PCell

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| SNR_SSB of set q0 | Config 1 | dB | TBD | TBD | TBD | TBD | TBD |
| SNR_SSB of set q1 | Config 1 | dB | TBD | TBD | TBD | TBD | TBD |
| SSB_RP of set q1 | Config 1 | dBm/SCS kHz | TBD | TBD | TBD | TBD | TBD |

<End of change 4>

<Start of change 5>

#### A.6.5.5.8	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for a UE operating on a cell with less than 5 MHz BW

##### A.6.5.5.8.1	Test Purpose and Environment

The purpose of this test is to verify that the UE supporting [FG-x capability] properly detects SSB-based beam failure in the set q0 configured for a serving cell operating on a less than 5MHz bandwidth, and that the UE performs correct SSB-based link recovery based on beam candidate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when no DRX is used. This test will partly verify the SSB based beam failure detection and link recovery for an FR1 serving cell requirements in clause 8.5.

Supported test configurations are specified in Table A.6.5.5.8.1-1. General test parameters as specified in Table A.6.5.5.1.1-2 with config 1 apply except those specified in Table A.6.5.5.8.1-2. Cell specific test parameters as specified in Table A.6.5.5.1.1-3 apply except those specified in Table A.6.5.5.8.1-3.

The test procedure specified in A.6.5.5.1.1 applies to this test.

Table A.6.5.5.8.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD duplex mode, 15 kHz SSB SCS, 3 MHz bandwidth |

Table A.6.5.5.8.1-2: General test parameters for FR1 PCell

| Parameter |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |  |
| BWchannel |  | Config 1 | MHz | 3: NRB,c = 15 |  |
| SSB Configuration |  | Config 1 |  | SSB.13 FR1 |  |
| PRACH Configuration |  | Config 1 |  | 1 |  |
| Beam failure detection transmission parameters | Number of Control OFDM symbols |  |  | 3 |  |
|  | Aggregation level |  | CCE | 8 |  |
|  | Mapping from REG to CCE |  |  | [Non-Distributed] |  |
| T1 |  |  | s | TBD | During this time the the UE shall be fully synchronized to cell 1 |
| T2 |  |  | s | 0.37 |  |
| T3 |  |  | s | 0.24 |  |
| T4 |  |  | s | 0 |  |
| T5 |  |  | s | 0.17 |  |
| D1 |  |  | s | 0.13 |  |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |

Table A.6.5.5.8.1-3: Cell specific test parameters for FR1 PCell

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| SNR_SSB of set q0 | Config 1 | dB | 5 | -3 | -12 | -12 | -12 |
| SNR_SSB of set q1 | Config 1 | dB | -10 | -10 | 10 | 10 | 10 |
| SSB_RP of set q1 | Config 1 | dBm/SCS kHz | -108 | -108 | -88 | -88 | -88 |

<End of change 5>

<Start of change 6>

#### A.6.6.1.12	SA event triggered reporting tests without gap under non-DRX with SSB index reading and 12 PRB SSB

##### A.6.6.1.12.1	Test purpose and Environment

The purpose of this test is to verify that the UE supporting [FG-x capability] makes correct reporting of an event when the intra-frequency target cell is transmitting 12 PRB SSB. This test will partly verify the FDD intra-frequency cell search requirements in clause 9.2.5.1 and 9.2.5.2.

##### A.6.6.1.12.2	Test parameters

The test procedure in clause A.6.6.1.5.2 applies for this test. Supported test configurations are specified in Table A.6.6.1.12.2-1. General test parameters as specified in Table A.6.6.1.5.2-2 with config 1 apply to this test, except those specified in Table A.6.6.1.12.2-2. Cell specific test parameters as specified in Table A.6.6.1.5.2-3 with config 1 apply to this test, except those specified in Table A.6.6.1.122-3.

Table A.6.6.1.12.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 3 MHz bandwidth, FDD duplex mode |

Table A.6.6.1.12.2-2: General test parameters for SA intra-frequency event triggered reporting without gap for FDD PCell in FR1 with SSB index reading

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| SSB configuration |  | 1 | SSB.13 FR1 |  |

Table A.6.6.1.12.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting without gap for FDD PCell in FR1 with SSB index reading

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| BWchannel | MHz | 1 | 3: NRB,c = 15 |  | 3: NRB,c = 15 |  |
| PDSCH RMC configuration |  | 1 | SR.1.2 FDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.3 FDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.7 FDD |  | N/A |  |
| Io | dBm/2.7 MHz | 1 | -70.00 | -67.65 | -70.00 | -67.65 |

<End of change 6>

<Start of change 7>

#### A.6.6.2.15	SA event triggered reporting tests for FR1 with 3MHz Channel Bandwidth configured without SSB time index detection when DRX is used

##### A.6.6.2.15.1	Test Purpose and Environment

The purpose of this test is to verify that the UE supporting [FG-x capability] makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 2.  The test parameters are given in Tables A.6.6.2.15.1-1 and A.6.6.2.15.1-2.

Measurement gap pattern configuration defined in Table A.6.6.2.15.1-2 is provided for a UE that does not support per-FR gap, and no gap pattern (Gap Pattern Id and Measurement gap offset) is configured for a UE capable of per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

UE needs to be provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment. Furthermore, UE is allocated with PUSCH resource at every DRX cycle. Supported test configurations are specified in Table A.6.6.2.15.1-1. General test parameters as specified in Table A.6.6.2.2.1-2 with config 1 apply to this test. Cell specific test parameters as specified in Table A.6.6.2.2.1-3 with config 1 apply except those specified in Table A.6.6.1.13.1-2. DRX-Configuration and TimeAlignmentTimer -Configuration specified in Table A.6.6.2.2.1-4 and Table A.6.6.2.2.1-5, respectively, apply to this test case.

The test environment specified in A.6.6.2.2.1 applies to this test.

Table A.6.6.2.15.1-1: SA event triggered reporting tests without SSB index reading for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 3 MHz bandwidth, FDD duplex mode |
| Note 1: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.2.15.1-2: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Duplex mode |  | Config 1 | FDD |  |  |  |
| BWchannel | MHz | Config 1 | 3: NRB,c = 15 |  |  |  |
| BWP BW | MHz | Config 1 | 3: NRB,c = 15 |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 | SR.1.2 FDD |  | NA |  |
| RMSI CORESET Reference Channel |  | Config 1 | CR.1.3 FDD |  | NA |  |
| Dedicated CORESET Reference Channel |  | Config 1 | CCR.1.7 FDD |  | NA |  |
| SSB parameters |  | Config 1 | SSB.x FR1 |  | SSB.y FR1 |  |
| IoNote3 | dBm/3MHz | Config 1 | -69.9 | -69.9 | -75.4 | -67.6 |

<End of change 7>