# 3GPP TSG-RAN WG4 Meeting #112                                                                R4-2412431

# Maastricht , Netherlands, 19th – 23th August, 2024

Title: 	Remaining open RF discussions for type 4 UEs

Source: 	Huawei, HiSilicon

Agenda item:	8.5.2.1

Document for:	Discussion

# Introduction

The WF [1]of the last meeting contained the following open issues, which are supposed to be discussed in this document. In the section we will present solutions to thoese issues based on our previous contributions and new methods.

# Discussion

## Minimum DL frequency separation

During Rel18 it was decided that for type 2 UE, no upper limit to be imposed for the DL frequency separation. On the other hand for lower limit of the DL frequency separation, it was discussed to set the minimum separation of the center of BWanother relative to edge of BWwanted to be 80MHz+BWanother/2. Moreover, it was agreed not to set any general requirements in RAN4 but to send an LS to RAN5 to inform them of such restrictions to limit the number of test cases [2]. A similar approach can be considered for type 4 as it is an extension of type 2 architecture. On the other hand no new LS to RAN5 is needed and the test cases for type 2 can be used for type 4, too.

Proposal 1: Center of BWanother relative to edge of BWwanted is assumed to be at least 80MHz+BWanother/2 away from the edge of the wanted CC.

Proposal 2: For a type 4 UE, no minimum or maximum DL separation is needed to be specified in RAN4, as it was the case for type 2 UEs.

# Conclusions

Proposal 1: Center of BWanother relative to edge of BWwanted is assumed to be at least 80MHz+BWanother/2 away from the edge of the wanted CC.

Proposal 2: For a type 4 UE, no minimum or maximum DL separation is needed to be specified in RAN4, as it was the case for type 2 UEs.

# Reference

[1]	“R4-2410686, WF on Rel-19 non-collocated UE RF requirements, KDDI.”

[2]	“R4-2316951, LS on frequency separation for Type 2 UE NR-CA PDSCH demodulation requirements, Ericsson.”