3GPP TSG-RAN WG4 Meeting # 112	R4- 2412433

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	China Telecom

Title: 	Views on Power domain enhancements for single carrier

Agenda Item:	8.1.1.2.1	Power domain enhancements for single carrier

Document for:	Discussion

Introduction

In last RAN4 meeting, there were many papers discussing about relaxation of requirements for identified scenarios and MPR reduction mechanisms. Finally we reached an agreement in WF[1].

1.Power domain enhancements for single carrier 1.1 Consideration on relaxation of requirements for identified scenarios Agreement in main session: - No relaxation of level of spurious emission requirements - FFS on whether to change the spurious emission boundary 1.2. Mechanisms for enabling MPR reduction and/or power boosting Way forward: - Study the approach to convert full RB allocation in UE CBW to “inner RB allocation” with an extended UE BW, e.g. each side of the UE BW is equal to ½ of the UE BW, inside a larger BS CBW as starting point with consideration of the following aspects - The edge of the extended UE BW, i.e. the UE CBW plus the shifted frequency symmetrically at each side of the UE CBW, should be inside BS CBW or at least aligned with the BS CBW edge - ① FFS feasibility of case where extended UE CBW edge exceeds the BS CBW edge, i.e. gap between edges of UE CBW and BS CBW < 1/2 UE CBW - ② FFS whether IBE is used between edges of UE CBW and BS CBW or between edges of UE CBW and extended UE CBW - ③ FFS ACLR and SEM are applicable from the edge of extended UE CBW or edge of BS CBW, i.e. the start point of ΔfOOB - ④ FFS integral region and boundary of OOBE is based on extended UE CBW or UE CBW - Other mechanisms are not precluded

So in this contribution, we provide our views on it.

Discussion

The boundary or region of SE and OOBE are defined in TS 38.101-1 with fOOB and ΔfOOB. The four aspects of full RB allocation of “inner RB allocation” also concentrate on the distinguishing about SE/OOBE/IBE and it should be start point for MPR reduction and/or power boosting study. So following the agreement about scenarios and Mechanisms in WF[1], SE/OOBE/IBE should study the boundary or region aspect at first rather than relaxation level.

Observation1: SE/OOBE/IBE should all study the boundary or region at first rather than relaxation level.

Since the outer RB allocation has more room for MPR reduction we convert it to a kind of “inner RB allocation”, so we should abide by the decision and treat the extended UE CBW as a new “assigned NR channel bandwidth” .

- For aspect, UE assigned NR CBW seldom exceeds the BS CBW edge in any scenarios so we cannot use extend way to analyse the MPR reduction mechanism under this aspect. We should preclude this aspect for outer RB allocation convert evaluation and using legacy MPR requirements.

- For aspect, IBE requirement is the emission impact to the assigned NR band(for UE) from UE “actual” CBW which is in the band, hence the IBE requirement should follows the definition of inner or outer RB allocation. NRB is the “assigned NR band”, also the “extended UE CBW”, and LCRB is the UE CBW. So obviously, IBE should be used between edges of UE CBW and extended UE CBW rather than between edges of UE CBW and BS CBW. Nevertheless, the UE and NW should have same understanding on this handling under “extension” analytical method, so signalling for indication may be needed.

- For aspect, the criterion for distinguishing IBE and OOBE is edge of UE actual CBW, therefore under the current circumstances the start point of ΔfOOB should be extended UE CBW edge. In other word, extension method is a kind of compromise between ACLR/SEM and IBE.

- For aspect, the boundary has been elucidated in aspect, and region(BWchannel in ΔfOOB definition) study should start with a simple and tighter way but gradually complex and detailed. So for tighter OOBE requirement and keeping to actual UE CBW, basing on UE CBW should be a start point.

The views can be summarized as:

|  | Preclude this aspect for outer RB allocation convert evaluation and use legacy MPR requirements. |
| --- | --- |
|  | IBE should be used between edges of UE CBW and extended UE CBW rather than between edges of UE CBW and BS CBW. |
|  | The start point of ΔfOOB should be extended UE CBW edge. |
|  | Base on UE CBW should be a start point. |

Proposal1:

| RB allocation convert scenarios | Preclude this aspect for outer RB allocation convert evaluation and use legacy MPR requirements. |
| --- | --- |
| RB allocation convert scenarios | IBE should be used between edges of UE CBW and extended UE CBW rather than between edges of UE CBW and BS CBW. |
| RB allocation convert scenarios | The start point of ΔfOOB should be extended UE CBW edge. |
| RB allocation convert scenarios | Basing on UE CBW as a start point. |

As elucidated in WID[2], all the MPR reduction methods must be indicated by BS for single UL carrier UE, so some configuration or report-configuration mechanism should be studied. But it’s not urgent since the MPR requirement still under discussion and we should establish a start point and continue to details with the deepen in MPR reduction mechanism.

Proposal2: Establish a start point and continue to details for BS indication with the deepen in MPR reduction mechanism.

Conclusion

Observation1: SE/OOBE/IBE should all study the boundary or region at first rather than relaxation level.

Proposal1:

| RB allocation convert scenarios | Preclude this aspect for outer RB allocation convert evaluation and use legacy MPR requirements. |
| --- | --- |
| RB allocation convert scenarios | IBE should be used between edges of UE CBW and extended UE CBW rather than between edges of UE CBW and BS CBW. |
| RB allocation convert scenarios | The start point of ΔfOOB should be extended UE CBW edge. |
| RB allocation convert scenarios | Basing on UE CBW as a start point. |

Proposal2: Establish a start point and continue to details for BS indication with the deepen in MPR reduction mechanism.

References

- R4-2410749, WF on power domain, RAN4#111

- RP-241656, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN#104