3GPP TSG-RAN WG4 Meeting # 112	R4- 2412434

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	China Telecom

Title: 	On SRS IL imbalance

Agenda Item:	8.1.1.3.4	SRS IL imbalance

Document for:	Discussion

Introduction

In last RAN4 meeting, companies submitted a bulk of SRS IL imbalance analysis papers and the discussion was extremely heated. The WF[1] was captured as below but the general solution is still misty.

So in this contribution, we reaffirm our perspective and concisely provide a candidate scheme.

Discussion

- 2.1 General

With the introduce of 6Rx UE the power imbalance between SRS ports are more non-negligible and the value of ∆TRxSRS will be higher. For 8Rx handheld UE, the IL relaxation in TS 38.101-1 could reach 10.3dB at some scenarios and it is too large in performance evaluation.

Due to the diversity of UEs implement the compensation mechanism at UE side is unpredictable much less the limited power hea droom. On the one hand, the 3GPP specification only address the upper boundary of SRS IL issue. On the other hand, the relaxation in PCMAX_L,f formula are ∆TC,c, ΔTIB,c, and ∆TRxSRS and each of them contributes to the MOP so the IL imbalance is difficult to acquire for UE, which most likely reflect directly to the NW.

In last meeting, many contributions also proved the necessity of introducing reporting scheme and corresponding analysis or simulation result. The WF[2] after RAN4#110-bis can be reused and nothing more than just this several cases for IL analysis.

The actually power needed for SRS transmission can be assume as:

PSRS,total = PSRS,b,f,c(I,qs,l) + PIL

Where PSRS,b,f,c(I,qs,l) can be calculated according to TS 38.213.

If PSRS,total ≤ PCMAX_H,f,c, which means the power headroom is enough for UE to compensate the SRS IL (i.e. if PSRS,total ≤ PCMAX,f,c, UE can simply increase PSRS,total to compensate IL and do not have to change PCMAX,f,c; if PSRS,total ≥ PCMAX,f,c, UE also have chance to compensate IL through increasing PCMAX,f,c since the upper bound will not be exceeded.);

If PSRS,total ≥ PCMAX_H,f,c, UE is impossible to compensate the whole SRS IL, but can still compensate partial of it by extending output power to PCMAX_H,f,c.

No matter what strategy UE takes to compensate the imbalance, a situation will arise that UE cannot compensate all the SRS IL due to limit power headroom, so NW will misunderstand the actual SRS power and cannot perform compensation at it is side. Therefore, an SRS IL imbalance reporting mechanism should exist to handle the mismatch between UE and NW when UE cannot compensate the total IL.

Obsveration1: SRS IL imbalance is non-negligible and reporting mechanism is necessarily to be introduced.

- 2.2 solutions

The UE behaviour, no matter compensation or not, does make a difference in SRS MOP and influence the NW decision in IL imbalance reporting. In last meeting Lenovo’s paper[3] proved the information for NW to determine the SRS power per port is different for UE w/o compensation. It will be helpful if UE indicate whether it support insertion loss compensation.

Proposal1: UE should indicate to NW whether it support insertion loss compensation ability.

Static reporting could report ∆TRxSRS (or another fix value) at different SRS antenna switching mode to NW. But the actual IL is influenced by power headroom and it changes when PCMAX,f,c changes configured by NW (i.e. TPC command). UE can compensate partial of it. Static reporting is a compromise between complexity and accuracy.

Dynamic reporting can reflect the actual information in real time, therefore it is more helpful for gNB to take steps. Just like DPC reporting, dynamic reporting based on trigger- execute mechanism will let NW have options to acquire more information timely。

Continuing follow the RAN4 tradition that define the requirement based on the worst case, the static reporting is more sensible. But if we want more explicit report and complexity is tolerable, dynamic method should be considered.

Proposal2: From complexity point, the static reporting is more feasible. But if we want more accurate report, dynamic method should be considered.

Motivated by UE strategy and network demand, both static and dynamic way need to consider reporting granularity, i.e. per band or per BC or per UE or per SRS antenna switching mode or per SRS resource, and analyse the corresponding impact. Besides, assistance information like MOP or power headroom of SRS port also should be considered.

Proposal3: Both static and dynamic SRS IL report need to consider reporting granularity and other information.

Conclusion

Obsveration1: SRS IL imbalance is non-negligible and reporting mechanism is necessarily to be introduced.

Proposal1: UE should indicate to NW whether it support insertion loss compensation ability.

Proposal2: From complexity point, the static reporting is more feasible. But if we want more accurate report, dynamic method should be considered.

Proposal3: Both static and dynamic SRS IL report need to consider reporting granularity and assistance information.

References

- R4-2410751, WF on requirements for 6Rx, RAN4#111

- R4-2406585, WF on requirements for 6Rx, RAN4#110-bis

- R4-2409736, SRS power imbalance Determination, MTK, RAN 4#111