# 3GPP TSG-RAN WG4 Meeting #112                                                                R4-2412435

# Maastricht , Netherlands, 19th – 23th August, 2024

Title: 	NTN UE RF requirements for less than 5MHz CBW

Source: 	Huawei, HiSilicon

Agenda item:	8.8.3.2

Document for:	Discussion

# Introduction

3 MHz CBW was specified during Rel-18 for the bands n26, n28, n31, n72, n85, n100 and n106. It is to be included as one of the CBWs of the NTN FR1 bands, n254, n255, and n256, based on the WID[1]. The WID states to drive the requirements as much as possible from Rel-18 less than 5MHz requirements. Therefore in this tdoc we are going to analyse and propose RF requirements based on those requirements.

# Discussion

In Table 1 a summary of TS 38.101-5 sections, which are impacted or unimpacted by introduction of 3MHz CBW, is provided.

| NR NTN Tx/Rx requirement | Proposal for NR 3 MHz |
| --- | --- |
| 6.2.1 UE maximum output power | No specification impact |
| 6.2.2 MPR | No specification impact (same as TS 38.101-1) |
| 6.2.3 A-MPR | 3MHz needs to be added for all NSs - NS_01: 3 MHz shall be added to the list of considered channel BW for the NS. - NS_24: A-MPR for 3MHz is not needed. NS_24 for LTE is not applied for 3MHz CBW, neither - NS_02N, NS_03N, NS_04N, NS_05N: A-MPR for 3MHz should be studied - NS_100 is not dependent on the considered channel BW. |
| 6.2.4 Configured transmitted power | No specification impact (same as TS 38.101-1) |
| 6.3.1 Minimum output power | Is channel bandwidth dependent, specification change needed but re-using -40 dBm can be considered with a measurement BW of 2.715 MHz. MBW= REF_SCS*(12*NRB+1)/1000 = 15*(12*15+1)=2.715 MHz |
| 6.3.2 Transmit OFF power | Value - 50 dBm is channel bandwidth agnostic --> 3 MHz / 15 kHz SCS shall still be added in table 6.3.2-1. |
| 6.3.3 Transmit ON/OFF time mask | No specification impact (same as TS 38.101-1) |
| 6.3.4 Power control | No specification impact |
| 6.4.1 General | No specification impact (same as TS 38.101-1) |
| 6.4.2.2 Phase continuity requirements for DMRS bundling | No specification impact |
| 6.5.1 Occupied bandwidth | Specification impact, addition of 3 MHz into Table 6.5.1-1 |
| 6.5.2.2 Spectrum emission mask | Specification impact, reuse the general SEM from Table 6.5.2.2-1 of TS38.101-1 |
| 6.5.2.3 Additional Spectrum emission mask | No specification impact. Channel bandwidth agnostic |
| 6.5.2.4.1 NR ACLR | Specification impact, addition of 3 MHz into Table 6.5.2.4.1-1 |
| 6.5.2.4.2 UTRA ACLR | No specification impact. Channel bandwidth agnostic |
| 6.5.3.1 General spurious emissions | Specification impact, addition of 3 MHz into Table 6.5.3.1-1 with a OOB boundary of 6 MHz |
| 6.5.3.2 Spurious UEtoUE co-ex | No specification impact |
| 6.5.3.3 Additional spurious emissions | Specification impact: for NS_02N, NS_03N and NS_04N, 3 MHz needs to be listed. |
| 6.5.4 Transmit intermodulation | Channel bandwidth agnostic --> no specification impact |
| 7.3.2 Reference sensitivity | Channel bandwidth dependent --> specification impact 3MHz REFSENS with 15 RB UL configuration Two antenna port reference sensitivity QPSK PREFSENS for FDD bands Operating band / SCS / Channel bandwidth. Operating Band; SCS kHz; 3 MHz (dBm). 15; -101.7. n256; 30. 60. 15; -102.1. n255; 30. 60. 15; -101.7. n254; 30. 60. NOTE：The transmitter shall be set to PUMAX as defined in clause 6.2.4 of 3GPP TS 38.101-1 [5].. |
| 7.4 Max input level | Specification impact, addition of 3 MHz into Table 7.4-1: -40dBm |
| 7.5 Adjacent channel selectivity | Specification impact --> 3 MHz is added into Table 7.5-1: 33 dBc (same as TS 38.101-1) 3 MHz is added into Table 7.5-3 (same as TS 38.101-1) |
| 7.6.2 In-band blocking | Specification impact --> 3 MHz is added into Table 7.6.2-1 similar to 38.101-1 Table 7.6.2-1 |
| 7.6.3 Out of band blocking | Specification impact --> 3 MHz is added into Table 7.6.3-1 similar to 38.101-1 Table 7.6.3-1 |
| 7.6.4 Narrow band blocking | Specification impact --> 3 MHz is added into Table 7.6.4-1 similar to 38.101-1 Table 7.6.4-1 |
| 7.7 Spurious response | Specification impact --> 3 MHz is added into Table 7.7-1 similar to 38.101-1 Table 7.7-1 |
| 7.8 Intermodulation characteristics | No specification impact (same as TS 38.101-1) |
| 7.9 Spurious emissions | No specification impact. Channel bandwidth agnostic |

# Conclusions

Proposal 1: In Table 1 a summary of TS 38.101-5 sections which are impacted or unimpacted by introduction of 3MHz CBW, is provided.

# Reference

[1]	“RP-241281, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN, Samsung.”