# 3GPP TSG-RAN WG4 Meeting #112                                                                R4-2412437

# Maastricht , Netherlands, 19th – 23th August, 2024

Title: 	NTN System parameters for less than 5MHz CBW

Source: 	Huawei, HiSilicon

Agenda item:	8.8.3.1

Document for:	Discussion

# Introduction

3 MHz CBW was specified during Rel-18 for the bands n26, n28, n31, n72, n85, n100 and n106. It is to be included as one of the CBWs of the NTN FR1 bands, n254, n255, and n256, based on the WID[1]. The WID states to drive the requirements as much as possible from Rel-18 less than 5MHz requirements. Therefore in this tdoc we are going to analyse and propose systems parameters based on those requirements.

# Discussion

In Rel-18 the 3 MHz CBW is limited to single carrier applications excluding RedCap UEs per RAN-P guidelines[2]. Therefore the 3MHz CBW should be limited to these applications.

Proposal 1: for n254, n255 and n256, the 3MHz CBW is only used in single-carrier operations, excluding RedCap.

## NRB and the guard band

In [3] we showed that, for a 15 KHz SCS, between 15 or 16 NRB for 3MHz CBW can be considered. An NRB of 15 better reduces the out of band emissions because of having larger guard bands. Consequently, a guard band of 142.5 KHz was agreed for this CBW.

Proposal 2: For SCS 15 KHz, set NRB =15 which provides larger guard band that results in lower out of band emissions.

Proposal 3: Consider a guard band of 142.5 KHz for 3MHz CBW.

## Channel space and Channel raster

In Rel-18 the channel space and channel raster was enough to be used for 3MHz CBW. Therefore there is no need to change them for the NTN bands.

Proposal 4: No change to be made on the channel raster and channel space.

## Sync raster

During Rel-18, there were extensive discussions on the design of the sync raster for less than 5MHz. One of the main criteria of choice was to make sure to have no overlap with the legacy sync raster[2]. Therefore the following sync raster was chosen for less than 5MHz applications.

Rel-18 Sync raster below 1GHz: N * 600 kHz + M * 50 kHz + 300 kHz. N = 1:1665, M ϵ {1,3,5} (eq.1)

There is no need to change the granularity of the sync raster in eq.1 as it was designed to ensure operating with a 3MHz CBW, properly. The 300 KHz offset was set to ensure that there is no overlap with the Rel-15 sync raster (legacy). However the sync raster needs to be extended beyond 1GHz and cover n254, n255 and n256. Therefore we propose to include up to 3 GHz with N= 4999 (highest sync raster at 2.999.850.000 Hz). On the other hand, the extension to 3GHz makes the 3MHz sync raster frequency range be aligned with Rel-15 sync raster frequency range.

Proposal 5: Extend the Rel-18 3MHz sync raster to 3GHz with N=4999.

Table 1: GSCN parameters for the global frequency for 3 MHz channel bandwidth

| Range of frequencies (MHz) | SS block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 0 – 3000 | N * 600 kHz + M * 50 kHz + 300 kHz, N = 1:4999, M ϵ {1,3,5} (Note 1) | 26638+3N + (M-3)/2 | 26640 – 41636 |
| NOTE 1: Only applicable for 15 PRB transmission bandwidth configuration within 3 MHz channel bandwidth with punctured PBCH defined in TS 38.211 [6] clause 7.4.3.1. |  |  |  |

In Rel-18 two additional sync raster points were defined for n100 (TS 38.101-1 Table 5.4.3.1-3), because of the special deployment scenario of the UIC on n100 requires replacing GSM-R channels with 5G NR gradually[4]. Therefore those additional sync raster would enable better spectrum utilization for UIC. For NTN bands, no such migration scenario is considered so additional sync raster points are not needed.

Proposal 6: No additional sync raster point is needed for n254, n255 and n256. The sync raster in proposal 5 is enough.

## NTN signaling

In Rel-18 UE capabilities support3MHz-ChannelBW-Symmetric-r18 and support3MHz-ChannelBW-Asymmetric-r18 were introduced to show that the UE supports 3MHz CBW in symmetrical and asymmetrical (3MHz in UL) configurations. These UE capabilities are for single carrier operations , excluding RedCap. Therefore they can be used in the NTN communication, too.

Proposal 7: Use the Rel-18 UE capabilities support3MHz-ChannelBW-Symmetric-r18 and support3MHz-ChannelBW-Asymmetric-r18 for NTN communications.

# Conclusions

Proposal 1: for n254, n255 and n256, the 3MHz CBW is only used in single-carrier operations, excluding RedCap.

Proposal 2: For SCS 15 KHz, set NRB =15 which provides larger guard band that results in lower out of band emissions.

Proposal 3: Consider a guard band of 142.5 KHz for 3MHz CBW.

Proposal 4: No change to be made on the channel raster and channel space.

Proposal 5: Extend the Rel-18 3MHz sync raster to 3GHz with N=4999

Table 1: GSCN parameters for the global frequency for 3 MHz channel bandwidth

| Range of frequencies (MHz) | SS block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 0 – 3000 | N * 600 kHz + M * 50 kHz + 300 kHz, N = 1:4999, M ϵ {1,3,5} (Note 1) | 26638+3N + (M-3)/2 | 26640 – 41636 |
| NOTE 1: Only applicable for 15 PRB transmission bandwidth configuration within 3 MHz channel bandwidth with punctured PBCH defined in TS 38.211 [6] clause 7.4.3.1. |  |  |  |

Proposal 6: No additional sync raster point is needed for n254, n255 and n256. The sync raster in proposal 5 is enough.

Proposal 7: Use the Rel-18 UE capabilities support3MHz-ChannelBW-Symmetric-r18 and support3MHz-ChannelBW-Asymmetric-r18 for NTN communications.

# Reference

[1]	“RP-241281, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN, Samsung.”

[2]	“RP-230781, Response to LS from RAN4 on spectrum less than 5 MHz, Nokia.”

[3]	“R4-2302514, Introduction of Channel BW Smaller Than 5 MHz in NR, Huawei, HiSilicon.”

[4]	“RP-230189, UIC response to LS from RAN1 on transmission bandwidths for NR on dedicated spectrum less than 5 MHz, Union International Chemins de Fer.”