# 3GPP TSG-RAN WG4 Meeting #112                                                                R4-2412439

# Maastricht , Netherlands, 19th – 23th August, 2024

Title: 	Remaining open UE capability discussions for type 4 UEs

Source: 	Huawei, HiSilicon

Agenda item:	8.5.2.2

Document for:	Discussion

# Introduction

The WF [1]of the last meeting contained the following open issues, which are supposed to be discussed in this document. In the section we will present solutions to those issues based on our previous contributions and new methods.

# Discussion

## New UE Capabilities for Type 4a(EN-DC) and 4b(EN-DC/NR-CA) and signalling

Type 4a and type 4b architectures are depicted in [2]. There are two different approaches to be taken for type4. The first one is to use separate UE capabilities so if a UE supports type 4a (6Rx available in total) the network will know its capabilities directly and it can distinguish it from type 4b (8Rx available in total). The second method would be to only create a single UE capability for type 4 and the UE will use the legacy signalling to report the maximum number of layers that it can support per CC. UE will report the following parameters in EN-DC configurations. If the UE is capable having 2 or 4 Rx in LTE bands, it will report [interBandMRDC-WithOverlapDL-Bands-r19] to show it is a type 4 and it will send the following to set the max number of DL MIMO layers per CC (TS 36.331):

FeatureSetsEUTRA-r15> FeatureSetDL-PerCC-r15 > supportedMIMO-CapabilityDL-MRDC-r15 > MIMO-CapabilityDL-r10 ::= ENUMERATED {twoLayers, fourLayers, eightLayers}

Using the second method the network will be aware that it has a UE with type 4a or type 4b capabilities. Therefore we propose the second method because it requires creation of lesser UE capabilities.

Proposal 1: Introduce a single type 4 UE capability [interBandMRDC-WithOverlapDL-Bands-r19] and use the legacy UE capabilities to distinguish between type 4a and 4b in EN-DC configuration. No need to introduce two separate UE capabilities for type 4a and type 4b.

FeatureSetsEUTRA-r15> FeatureSetDL-PerCC-r15 > supportedMIMO-CapabilityDL-MRDC-r15 > MIMO-CapabilityDL-r10 ::= ENUMERATED {twoLayers, fourLayers, eightLayers}

Proposal 2: Introduce a single type 4 UE capability for NR-CA

Proposal 3: No need to have a special signalling to switch between type 4a and type 4b, as the NW can change the number of layers/CC using its legacy signallings easily.

## Support Type 2 capability by UE having Type 4 capabilities and Switching

In order to ensure a proper service for the legacy network it is better for a type 4 UE to report type 2 related capabilities. intraBandNR-CA-non-collocated-r18, interBandMRDC-WithOverlapDL-Bands-r16. Therefore when a type 4 UE is communicating with a Rel-18 network (for example), it will at least be able to operate as a type 2 UE. It should be mentioned here that the description of intraBandNR-CA-non-collocated-r18, interBandMRDC-WithOverlapDL-Bands-r16, in TS 38.306 , are not directly related to type 2 UEs. They refer to sections of  TS 38.101-1 and TS 38.133 which contained type 2 RF and RRM requirements in Rel-18 but in Rel-19, RAN4 wants to use those sections to include type 4 requirements. Therefore, the descriptions of these UE capabilities might needed to be updated by RAN2.

It should be mentioned that type 4 is more performant than type 2 because they support higher number of DL MIMO layers per cell. Type 4 is for FWA UEs, which are designed for performance, are connected to a stable power source. Therefore a dedicated NW IE to perform such a switching is not needed. If operators would need, for some corner cases, to reduce the power consumption (for instance, due to power source outage), they can simply set maxMIMO-Layers =2 to change from type 4 to type 2. Consequently to switch from type 2 to type 4, set maxMIMO-Layers =4

Proposal 4: A type 4 UE should report type 2 capabilities to be able to operate on legacy networks. An update on the agreement 2.2.2 of the WF R4-2410686 is necessary.

Proposal 5: Either the descriptions of intraBandNR-CA-non-collocated-r18 and interBandMRDC-WithOverlapDL-Bands-r16 in TS 38.306 need to be updated to link them to type 2 UEs, directly, or type 4 UE requirements should be on different sections than type 2 in TS 38.101-1 and TS 38.133

Proposal 6: No new NW IE is needed to perform type 4 to type 2 switching. Set maxMIMO-Layers =2 to change from type 4 to type 2. Consequently to switch from type 2 to type 4, set maxMIMO-Layers =4

## New BS Signalling to switch between Type 4a/4b and Type 1(collocated)

It should be noted that type 4 UE RF and RRM requirements allow it to perform in collocated network deployment as well. Therefore a switch from type 4 (default type) to type 1 with 4Rx is not useful. On the other hand, a switch from type 4b to type 1 with 8Rx (which is specified since Rel-18) or a switch from type 4a to type 1 with 6Rx (which is being discussed in Rel-19) can be beneficial as the operate can gain in number of  DL layers when it makes the switch. Therefore either a NW signalling should be defined to allow a switching from type 4a/b to type 1 with 6Rx/8Rx, or nonCollocatedTypeNR-CA/ nonCollocatedTypeMRDC needs to be extended to be applicable to UEs configured with maxMIMO-Layers ≤ 4, too.

Observation 1: As type 4 is capable of performing in collocated scenarios, there is no need to switch from type 4 to type 1 4Rx.

Proposal 7: Type 4 is the default type. To allow a switching from type 4a/b to type 1 with 6Rx/8Rx. RAN2 should, either introduce a new NW IE or extend nonCollocatedTypeNR-CA/ nonCollocatedTypeMRDC to make it applicable for UEs configured with maxMIMO-Layers ≤ 4, too.

# Conclusions

Proposal 1: Introduce a single type 4 UE capability [interBandMRDC-WithOverlapDL-Bands-r19] and use the legacy UE capabilities to distinguish between type 4a and 4b in EN-DC configuration. No need to introduce two separate UE capabilities for type 4a and type 4b.

FeatureSetsEUTRA-r15> FeatureSetDL-PerCC-r15 > supportedMIMO-CapabilityDL-MRDC-r15 > MIMO-CapabilityDL-r10 ::= ENUMERATED {twoLayers, fourLayers, eightLayers}

Proposal 2: Introduce a single type 4 UE capability for NR-CA

Proposal 3: No need to have a special signalling to switch between type 4a and type 4b, as the NW can change the number of layers/CC using its legacy signallings easily.

Proposal 4: A type 4 UE should report type 2 capabilities to be able to operate on legacy networks. An update on the agreement 2.2.2 of the WF R4-2410686 is necessary.

Proposal 5: Either the descriptions of intraBandNR-CA-non-collocated-r18 and interBandMRDC-WithOverlapDL-Bands-r16 in TS 38.306 need to be updated to link them to type 2 UEs, directly, or type 4 UE requirements should be on different sections than type 2 in TS 38.101-1 and TS 38.133

Proposal 6: No new NW IE is needed to perform type 4 to type 2 switching. Set maxMIMO-Layers =2 to change from type 4 to type 2. Consequently to switch from type 2 to type 4, set maxMIMO-Layers =4

Observation 1: As type 4 is capable of performing in collocated scenarios, there is no need to switch from type 4 to type 1 4Rx.

Proposal 7: Type 4 is the default type. To allow a switching from type 4a/b to type 1 with 6Rx/8Rx, RAN2 should, either introduce a new NW IE or extend nonCollocatedTypeNR-CA/ nonCollocatedTypeMRDC to make it applicable for UEs configured with maxMIMO-Layers ≤ 4, too.

# Reference

[1]	“R4-2410686, WF on Rel-19 non-collocated UE RF requirements, KDDI.”

[2]	“R4-2404258, Introduction of type 4 UE architecture,Huawei, Hisilicon.”