3GPP TSG-RAN WG4 Meeting #112	R4-2412441

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.22.4.1

Source:	OPPO

Title:	Simulation assumptions and results for LP-WUS/WUR measurement

Document for:	Approval

- Introduction

RAN4 #111 meeting had an initial discussion on the simulation for LP-WUS/WUR measurement and the related simulation assumptions are captured in [1] for information. This contribution will give our simulation results for LP- RSRP and the assumptions used in our simulation platform.

- Discussion

Our simulation results for LP-RSRP measurements are shown below, where 4 different LP-SS patterns are compared. And no much performance difference is observed among these LP-SS patterns in the same channel conditions.

- LP-SS sequence length is 4 using OOK-4 with M=2, i.e. 2 OFDM symbols occupied

- LP-SS sequence length is 8 using OOK-4 with M=2, i.e. 4 OFDM symbols occupied

- LP-SS sequence length is 8 using OOK-4 with M=4, i.e. 2 OFDM symbols occupied

- LP-SS sequence length is 16 using OOK-4 with M=4, i.e. 4 OFDM symbols occupied

Taking TDL-C fading channel as the example, the measured LP-RSRP at Es/Iot=-3dB with 4 samples could achieve 4.5dB accuracy at 90%-tile, which is comparable to the 6dB accuracy for SS-RSRP intra-frequency for CA/DC Idle mode in clause 10.1.2B.

Observation 1: LP-RSRP measurement could achieve 4.5dB accuracy performance at Es/Iot=-3dB with 4 samples in TDL-C channel.

| Figure 1: Simulation results of RSRP accuracy for 4 cases with AWGN channel |  |
| --- | --- |
| Figure 2: Simulation results of RSRP accuracy for 4 cases with TDL-C channel |  |

Our simulation assumptions are given in the tables below, where the different parameters compared with that in WF [1] are highlighted in red.

Table 1: General parameters

| Simulation parameters | Comments/values |
| --- | --- |
| Carrier frequency for Cell 1 and Cell 2 | FR1: 2.6 GHz/700MHz |
| System bandwidth | 20/100 MHz; |
| Prior knowledge of Cell 1 / Cell 2 by the UE | No / Yes Cell 1: Yes (measured serving cell) Cell 2: No (neighbour cell as interference) |
| DRX | No |
| BS transmit antennas for LP-SS blocks | 1 tx or single layer transmissions |
| UE receive antennas | 1 rx |
| Data and control channel subcarrier spacing | OFDM based: The same as SS block subcarrier spacing OOK based: the same as one of the SCS(s) used for other NR transmissions in the same CP-OFDM symbol |
| Measurement period (in number of measurement samples) | OOK based: [5, other number of samples may also be studied upon a need] OFDM based: [5] |
| Subcarrier spacing | 2.6GHz: 15 kHz and 30 kHz; 700MHz: 15kHz |
| Number of LP-SS blocks per SS burst set, K | [1] |
| LP-SS/SSB burst periodicity | [320 ms] OFDM based: [320 ms] |
| Number of transmit antenna ports | 1 (the same port for NR-SSS, NR-PSS, NR-PBCH) |
| LP-SS block BW | [132 subcarriers for SCS=30kHz (based on RAN1 agreement);] [288 subcarriers for SCS=15kHz ] Note: May need be updated based on RAN1 conclusion |
| Actual LP-SS block transmissions | always transmitted |

Table 2: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| RF Channel number | - | Channel 1 | Channel 1 |
| NR-PSS,NR-SSS and LP-SS sequences | - | To be indicated by companies | To be indicated by companies |
| PBCH and DMRS power offset with respect to NR-PSS, NR-SSS and LP-SS | dB | 0 | 0 |
| Data and control PSD relative to NR-PSS,NR-SSS and LP-SS | dB | 0 | 0 |
| RB Utilization | % | 100 | 100 |
| LP-SS |  | OOK-1; OOK-4 with M = 2,4; | [when Cell 1 uses OOK-1; OOK-1 or NR signal is used for Cell 2] [when Cell 1 uses OOK-4, OOK-4 or NR signal is used for Cell 2] |
| LP-SS pattern |  | Company report | Company report |
| Data Modulation | - | [OOK based: OOK] [OFDM based: QPSK] | [OOK based: QPSK/OOK] [OFDM based: QPSK] |
| Slot length | - | 14 symbols | 14 symbols |
| CP Length | - | Normal | Normal |
| Frequency Offset relative to UE frequency reference | Hz | [TBD] | [TBD] |
| 1)Relative Delay of 1st Path (synchronous) | µs | 0 | [CP/2] |
| 2) Relative Delay of 1st Path (asynchronous): Fixed delay | Ms | 0 | [3 ms] |
| SNR | dB | -2.7 | -11.7dB |
| Es/IoT (calculated from SNR) | dB | N/A -3dB | OFDM based: [TBD] OOK based: [TBD] |
| Propagation conditions | - | FR1: AWGN TDL-A 30ns TDL-B 100ns TDL-C 300ns |  |
| UE speed |  | FR1: 3 km/h |  |
| NOTE: the companies are encouraged to state channel model parameters together with the results, the parameters are to be further discussed and aligned |  |  |  |

Table 3: UE-specific parameters

| [Receiver Filter] | [5th Order Butterworth with 4.32MHz bandwidth] |
| --- | --- |
| [Receiver ADC bit width] | [4/8-bitADC] |
| [Receiver Sampling Rate for LP-SS only] | [3.84/7.68MHz] |

Observation 2: For simulation assumptions, some parameters should be updated which are highlighted as red in the Table1/2/3 above.

- Conclusion

In this contribution the following observation based on the initial simulation results for LP-RSRP measurement are proposed.

Observation 1: LP-RSRP measurement could achieve 4.5dB accuracy performance at Es/Iot=-3dB with 4 samples in TDL-C channel.

Observation 2: For simulation assumptions, some parameters should be updated which are highlighted as red in the Table1/2/3 above.

References

[1] R4-2410296, WF for RRM core requirements for LP-WUS/WUR, vivo, RAN4 #111