3GPP TSG-RAN WG4 Meeting #112	R4-2412460

# Maastricht, Netherlands, 19 – 23 August, 2024

Agenda Item:	7.18.1

Source:	MediaTek Inc.

Title:	Discussion on NTN-TN UE-coexistence issue for an IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL)

Document for:	Discussion

- Introduction

In RAN#104 plenary meeting, the new spectrum WID on introduction of IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) was approved in [1]. One of the main objectives is to define RF requirements. Before defining the RF requirements, to discuss the NTN-TN UE-coexistence issue would be necessary.

In this contribution, our views about NTN-TN UE-coexistence issue in WI [1] are provided for discussion.

- Discussion

2.1		WI requirements

One of the main Rel-19 WID [1] objectives is shown below.

The objective of the core part is to: - Specify a new E-UTRA NTN FDD band for NB-IoT/eMTC NTN operation, with a UE transmitting at 2000 – 2020 MHz and SAN transmitting at 2180-2200 MHz; - Perform necessary co-existence analysis, for example UE-UE coexistence, for adjacent band per RAN4 scope; - Consider whether assumptions for coexistence should be re-evaluated for the NTN scenario; - Support UE Categories NB1, NB2, M1; - Support UE Power Class 3 (+23dBm) and Power Class 5 (+20dBm); - Introduce the corresponding SAN and UE RF core requirements; - Introduce the corresponding RRM requirements.

2.2		NTN-TN UE-coexistence

In TR 38.863, it indicates that there is isolation-distance protection between NTN-TN UEs co-existence as shown below. An isolation distance of 1500m as 2*ISD is considered.

6.2.1.1 Co-existence between NTN satellite and TN Cellular cell structure is considered for both NTN satellite and TN network layout. Referring to TR 38.811[5] Section 6.3 and Annex A, a 3D global coordinate system is considered (Earth-Centred Earth Fixed) for simulating NTN satellite beams direction and location on the earth surface. It means the NTN satellite beam location, TN randomly dropping location are generated with a set of three parameters (x,y,z). Deployment of NTN satellite and TN cells and UEs for co-existence study is listed in Table 6.2.1.1-1. NOTE: The NTN UE(s) shall be dropped at the edge of the “central 19 TN cells (cluster)”. For Case 1 (Urban scenario), an Isolation distance of 1500m as 2*ISD is considered to reflect the NTN-TN selection algorithm at the border, assuming TN connection will always be prioritized over NTN. As defined in Figure 6.2.1.1-1, isolation distance is the distance between the blue-dotted line which represents TN cell boarder and the red line. No UEs deployed in the isolation region is assumed to reduce the calculation complexity. Figure 6.2.1.1-1: Isolation distance for Case 1

Observation 1: Regarding the conventional isolation distance of 2m between UE-UE coexistence operation, the free space path loss between UEs is 44.5dB for 2GHz band. Thus, it seems reasonable to define the general spurious-emission requirement of -50dBm/MHz for UE-coexistence.

Observation 2: Regarding the isolation distance of 1500m between NTN and TN operation, the free space path loss from NTN-UE TX to TN-UE RX is 102dB for 2GHz band.

Proposal 1: Discuss whether the general spurious-emission requirement of -50dBm/MHz for UE coexistence would be conditionally relaxed for NTN-TN coexistence due to higher isolation distance.

3 Conclusion

In this paper, our views about NTN-TN UE coexistence issue are provided for discussion.

Observation 1: Regarding the conventional isolation distance of 2m between UE-UE coexistence operation, the free space path loss between UEs is 44.5dB for 2GHz band. Thus, it seems reasonable to define the general spurious-emission requirement of -50dBm/MHz for UE-coexistence.

Observation 2: Regarding the isolation distance of 1500m between NTN and TN operation, the free space path loss from NTN-UE TX to TN-UE RX is 102dB for 2GHz band.

Proposal 1: Discuss whether the general spurious-emission requirement of -50dBm/MHz for UE coexistence would be conditionally relaxed for NTN-TN coexistence due to higher isolation distance.

Reference

- RP-241662, " New WID on Introduction of IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL)", DISH Network, EchoStar, TerreStar, OmniSpace, Telus

- TR 38.863, “Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence aspects”