3GPP TSG RAN WG4 #112															  R4-2412461

Maastricht, NL, August 19th – 23rd, 2024

Agenda Item:		5.35

Source:				Inmarsat, Viasat

Title:	Essential correction to NB-IoT NTN Carrier Frequency to avoid breaking of forward and backward compatibility

Document for:		Discussion

# Background

The purpose of this paper is to provide background and justification for proposing to change the NB-IoT NTN channel frequency formula as currently specified in TS 36.102 and TS 36.108 during Release-18 as part of LTE_NBIoT_eMTC_NTN_req-Core WI.

Awareness of this issue arose as part of analysis for the purpose of introducing in-band support for NB-IoT NTN deployed with NR NTN within the same Satellite Access Node (SAN).

NB-IoT supports In-band and guard-band deployments with LTE carriers since Rel-13 and in-band with NR carriers since Rel-15.   Based on the conclusions of TR 37.824, of the approved TP in R4-2220812, the analysis in R4-2216637, and after further analysis it has been concluded that the agreement based on the approved TP R4-2220812 breaks both backwards compatibility with Terrestrial UE implementations and existing NB-IoT NTN UE implementations on the market, requiring a new behaviour of which the effects cannot be predicted, as well as forward compatibility with in-band deployment with NR NTN (or any potential EUTRA or eMTC NTN deployment).

# Discussion

## Context

NB-IoT NTN support was introduced late during Release 17 due to market pressure.  Due to lack of time, and in order to allow to complete the work within the release, the RAN4 work, including the specification of UE RF specifications was postponed to later releases, and addressed in Release 18 as a leftover item.

When the work on RAN4 specifications was eventually carried out in Release 18, it was also done under time pressure, and with yet limited engagement with satellite operators who were in the very early stages of deployment preparation.  In order to limit the scope of the work also within Release 18, in addition, the scope of the specification was limited to Standalone deployment, with in-band and guard-band deployments left unsupported in Release 18, to potentially be discussed in future releases.

The System Parameters for NB-IoT NTN, including Carrier Frequency, EARFCN, Channel Raster and Channel Offset values (e.g. MDL, MUL) were discussed at length during the RAN4#103-e, RAN4#104-e, RAN4#104-bis-e and ultimately agreed in RAN4#105 with limited consensus.

One of the main areas of discussion was how to ensure efficient deployment of standalone carriers without spectrum wastage.  The following excerpt from R4-2216637 summarizes the problem identified back then:

In the legacy NB-IoT specification, MDL is selected from {-10, -9, -8, -7, -6, -5, -4, -3, -2, -1, -0.5, 0, 1, 2, 3, 4, 5, 6, 7, 8, 9} for FDD, which results in unutilized spectrum resources for orthogonal arrangement between anchor and non-anchor carrier since between anchor and non-anchor carriers there are 8 unused subcarriers as the closest possible selection.

- Figure 1: Example of closest standalone anchor and non-anchor carriers deployment

The main solutions were ultimately captured by the following two options summarized in R4-2217747:

- Option 1: Reuse existing MDL values for non-anchor carriers

- Option 2: Non-anchor carrier MDL enhancement where limited to a general shift of MDL+0.5 for non-anchor carriers

Option 2 had been presented in R4-2213243 and R4-2216637 and discussed in several meetings including RAN4#104-e and RAN4#104-bis-e, and presented two possible solutions summarized in the following excerpt from R4-2216637:

Introducing a new set value of MDL -9.5, -8.5,-7.5, -6.5, -5.5, -4.5, -3.5, -2.5 -1.5, -0.5, 0.5, 1.5, 2.5, 3.5, 4.5, 5.5, 6.5, 7.5, 8.5, 9.5}. Alternatively, re-using the existing set introducing a new note for deriving a new MDL value defining MDL = MDL + 0.5.

In the end, however, a solution was agreed in RAN4#105 based on the Text Proposal (TP) contained in R4-2220812, which instead proposed to maintain the existing MDL value range, by modifying the base formula for Carrier Frequency (FDL) from its original form in TS 36.101 and TS 36.104 as below:

FDL = FDL_low + 0.1(NDL – NOffs-DL) + 0.0025*(2MDL+1)

To a reduced formula, by removing the “+1” term as shown in the following excerpt from R4-2220812:

Observation 2: There’s a reserved DC sub-carrier which does not belong to any PRB in the LTE DL, while there’s no DC sub-carrier in the UL. Hence, there’s an extra offset of 0.0025 for FDL compared with FUL, i.e. 0.0025*(2MDL+1) vs 0.0025*(2MUL).

Since the current WI targets for standalone deployment for NB-IoT NTN, either observation 1 or 2 do not apply. Therefore, it’s feasible to start on a clean slate to design the channel number offset for NB-IoT NTN.

First of all, the formula for FDL can have the same form as that for FUL. For example:

FDL = FDL_low + 0.1(NDL – NOffs-DL) + 0.0025*(2MDL)

FUL = FUL_low + 0.1(NUL – NOffs-UL) + 0.0025*(2MUL)

With these definitions, both MDL and MUL can have the same value range of {-10, -9, -8, -7, -6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5, 6, 7, 8, 9}, and in the meantime enable the same flexible spacing as proposed in [4].

Proposal 1: Define the formula for the DL carrier frequency FDL using the same form as for the UL carrier frequency FUL. And define the value range for both MDL and MUL as {-10, -9, -8, -7, -6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5, 6, 7, 8, 9}.

The above mentioned TDoc further stated that the choice made could have been potentially able to allow forward compatibility with in-band deployment with NR, as can be seen from the following excerpt:

Note that the above conclusion in TR 37.824 assumes that FDL takes the form below for in-band deployment.

FDL = FDL_low + 0.1(NDL – NOffs-DL) + 0.0025*(2MDL+1)

Observation 3: The value range of {-10, -9, -8, -7, -6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5, 6, 7, 8, 9} for MDL and MUL is potentially forward compatible with NR-NTN in-band deployment.

However, after further analysis, we have identified that this is not true, and in fact, the modification of the FDL formula from its original form in fact severely breaks compatibility with both existing UE and with support for in-band deployment.

## Identified Issue

The proposals to clarify the support for in-band deployment for NB-IoT NTN with NR NTN in the same SAN have been presented for several meetings including both as part of  Release 18 maintenance, as well as a potential scope for Release 19.

In RAN#103 it was agreed that the issue should be continued to be pursued as part of Release 18 maintenance.

Our original assumption and high level assessment, based on the conclusions from TR 37.824 and of the approved TP in R4-2220812 was that the existing spec would indeed be already forward compatible with deployment in-band with NR NTN.

However, after further analysis, we identified that in fact, the agreement in R4-2220812 breaks compatibility both backwards with existing terrestrial UE and NTN NB-IoT UE based on Release 17, which would have been based on pre-R18 modes of operation for the UE RF, as well as forwards compatibility with in-band deployment, both with NR, as well as with a hypothetical E-UTRA carrier.

This is because the DL frequency relationship to MDL is different between 36.101 and 36.102 due to the difference in terms of the equations and difference in base MDL value for anchor carriers. Effectively for the same channel center and same MDL values, the FDL value results in a 2.5 kHz shift in DL frequency between the two formulas.

For comparison, the formulas and MDL values respectively align with:

- 36.102: FDL = FDL_low + 0.1(NDL – NOffs-DL) + 0.0025*(2MDL).  MDL range: {-10, -9, -8, -7, -6, -5, -4, -3, -2, -1, -0.5, 0, 1, 2, 3, 4, 5, 6, 7, 8, 9}

- 36.101: FDL = FDL_low + 0.1(NDL – NOffs-DL) + 0.0025*(2MDL+1). MDL range: {-10,-9,-8,-7,-6,-5,-4,-3,-2,-1,-0.5,0,1,2,3,4,5,6,7,8,9}

- 36.102: NOTE 2: For the carrier including NPSS/NSSS for stand-alone operation, MDL = 0.

- 36.101: NOTE 4: For the carrier including NPSS/NSSS for stand-alone operation, MDL = -0.5.

For standalone operation, in the TS 36.102 version of the spec, this shift is compensated by applying a different MDL value restriction to the anchor carrier, and then the different formula with integer MDL values allows placing multiple NB-IoT non-anchor carriers on the 180 kHz grid using the combination of 100 kHz channel raster and 5 kHz granularity offered by the integer MDL steps.

However, for in-band operation, this is no longer true, as the change in the DL frequency formula is not compensated, due to the lack of a correspondingly offset set of MDL values.

Conversely, in the TN scenario, the in-band carrier center frequency would end up being shifted by either 2.5 or 7.5 kHz in respect to the nominal center frequency on the 100 kHz grid, depending on the integer value of MDL, thanks to the “+1” term of the FDL formula.

As a further comparison, an example with 1526 MHz as the base frequency shows the difference when applying integer MDL values to the two different equations:

|  | MDL |  |
| --- | --- | --- |
|  | 0 | 1 |
| (TS 36.101) FDL = FDL_low + 0.1(NDL – NOffs-DL) + 0.0025*(2MDL+1) | 1526.0025 | 1526.0075 |
| (TS 36.102) FDL = FDL_low + 0.1(NDL – NOffs-DL) + 0.0025*(2MDL) | 1526 | 1526.005 |

The root of the compatibility problem is that in NB-IoT, which inherits its EARFCN properties from E-UTRA, the carrier center frequency maps in between the two center subcarriers, while in NR the channel raster points either to subcarrier index 0 or 6 depending on whether there is an even or odd number or PRBs in the channel. In other words, there is an inherent 7.5 kHz shift between the resource grids, and enhanced NR channel raster only brings this down to 2.5 kHz.

This is well explained in TR 37.824, as captured in the following snippets:

#### 5.2.1.5	NB-IoT and NR coexistence

As mentioned before, NB-IoT carrier frequency and NR RF reference frequency refers to different locations in the frequency domain:

NB-IoT carrier frequency is referring to a frequency located in between 2 NB-IoT tones.

NR RF reference frequency is referring to a frequency located in the middle of a tone.

To align sub-carriers, considering NR 15 kHz SCS, a 7.5 kHz offset shall therefore be considered in between reference frequencies.

The specified carrier frequency is located in the middle of NB-IoT RB, i.e. the frequency in between tones 5 and 6 of the RB (for 15 kHz SCS), as shown on Figure 5.2.1.2-1.

Figure 5.2.1.2-1: NB-IoT carrier location

The RF reference frequency defined by this channel raster identifies the RF channel position depending on the number of specified RBs (NRB) by the spectrum utilization for a given SCS and channel bandwidth:

If this number is even, then it points to the subcarrier #0 (first one) of RB index #0, as shown in Figure 2.

If this number is odd, the it points to the subcarrier #6 of RB index #0, as shown in Figure 5.2.1.3-1.

Figure 5.2.1.3-1: NR RF reference frequency position

In conclusion, due to the change in formula adopted in RAN4#105 based on R4-2220812, it becomes apparent that it is not possible to align single standalone NB-IoT NTN anchor carrier (i.e. with MDL = 0) to the NR resource grid within the same IFFT, nor shift subsequent non-anchor carriers sufficiently to allow multiple carriers to be placed in-band within the NR PRB grid.

The only possible solutions to this issue appear to either revert the formula for NB-IoT NTN Carrier Frequency to its original form, or specify a new set of MDLs with a 0.5 value shift specifically for in-band deployment, effectively rendering the rationale for adoption of the solution in R4-2220812 completely useless. Given the above, it is difficult to understand the rationale and benefits that emerged from adopting this solution.

Observation 1: The current formula for Carrier Frequency (FDL) and MDL value set for Cat NB1 and NB2 specified in TS 36.102 and TS 36.108 as part of Rel-18 WI break forward compatibility with in-band deployment and backwards compatibility with Terrestrial spec.

Observation 2: The current Carrier Frequency and MDL value set specified in TS 36.102 and TS 36.108 prevent the introduction of in-band deployment support of NB-IoT NTN with NR NTN in the same SAN.

Observation 3: Restoring forward support for in-band deployment with NR would likely require the introduction of a new set of MDL, thus rendering the solution adopted in R4-2220812 completely moot.

Lastly, it has to be noted that the consequences of this situation can be very severe.  First of all, given that the initial NB-IoT NTN deployments in the market have been based on Rel-17, which did not contain an NTN-specific RF specification set, it is very likely that some, if not all, of the implementations in the field, would actually be based on the terrestrial version of the RF spec (e.g. TS 36.101) and thus be based on the legacy FDL equation and MDL value set.

This would mean that any new Release 18 based NB-IoT NTN UE would be non-backwards compatible with initial Release 17 based UE, causing some potentially significant issues and upredictable behaviours that may not be fixable.

Furthermore, it would break forward compatibility with upcoming systems that are planning to implement NB-IoT NTN in-band with NR, by preventing completely Rel-18 based UE from functioning properly or at all with such new systems.

As a silver lining, given that Rel-18 has just been concluded, there is a very good likelihood that the majority of UE in the field would not yet be based on Rel-18, therefore, the impact of this late change might still be relatively limited.

Observation 4: This issue with forwards and backwards compatibility can have massive impact on upcoming deployments of NB-IoT NTN, and furthermore may break backwards compatibility with existing R17 based deployments and UE populations in the field, therefore it needs to be urgently fixed before R18 UE are being implemented, this may well be our last opportunity to fix it.

Observation 5: Rel-17 NB-IoT implementations currently in the market are likely to be still based on previous pre-R17 releases (i.e. aligned with terrestrial R13-14 UE as per TS 36.101), due to the lack of an NB-IoT NTN UE RF spec in Release 17.  Therefore the impact of this late change should be limited.

On the other hand, the alternative that was originally proposed by R4-2213243 and R4-2216637, and identified as “Option 2” in WF R4-2217747 and other previous documents, appears to provide a better solution which would not break forwards compatibility with in-band deployment, nor backwards compatibility with terrestrial implementation, whilst still enabling the efficient placement of Non-Anchor Carriers in Standalone deployment.

Observation 6: The alternative Option 2 presented in R4-2213243 and R4-2216637 and discussed in several meetings including RAN4#104-e and RAN4#104-bis-e, and captured in WF R4-2217747 provides a better solution which does not break forwards compatibility with in-band deployment, nor backwards compatibility with terrestrial implementation, whilst still enabling the efficient placement of Non-Anchor Carriers in Standalone deployment.

## Proposed Solution

Based on the above, we think that the best approach in order to resolve the situation would be to revert back from the solutions agreed in RAN4#105, re-align the NTN specs for NB-IoT NTN Carrier Frequency and MDL in TS 36.102 and TS 36.108 with their original corresponding terrestrial specifications in TS 36.101 and TS 36.104 respectively and adopt a new set of MDL values to achieve feature parity with the agreements in Release 18.

We thus propose the following:

Proposal 1:  Revert the FDL formula for Carrier Frequency for Cat NB1 and NB2 to the original formula aligned with TS 36.101 (terrestrial spec) as follows:

- FDL = FDL_low + 0.1(NDL – NOffs-DL) + 0.0025*(2MDL +1)

Proposal 2: Adopt one of the solutions proposed in R4-2213243 and R4-2216637 and either change the MDL values for Standalone by introducing the additional MDL values or add a NOTE that introduces a 0.5 shift in MDL values for Standalone Non-Anchor carriers:

- Option 1)

  - Introduce a new set  of MDL as {-9.5, -8.5,-7.5, -6.5, -5.5, -4.5, -3.5, -2.5 -1.5, 0.5, 1.5, 2.5, 3.5, 4.5, 5.5, 6.5, 7.5, 8.5, 9.5}

- Option 2) – Preferred/simpler Option – Introduce a new NOTE5 as follows:

  - NOTE 5: For the carrier not including NPSS/NSSS for stand-alone operation, MDL = MDL + 0.5.

Proposal : Remove the restriction for in-band deployment by striking out NOTE1 as follows:

NOTE 1:  Guard-band operation and in-band operation for NB-IoT are not supported in this version of the specification.

Proposal 4: Modify existing NOTE2 to NOTE4 as follows and change the value of MDL for carrier including NPSS/NSSS for stand-alone operation as follows, aligned with TS 36.101:

- NOTE 24:  For the carrier including NPSS/NSSS for stand-alone operation, MDL = -0.5.

Proposal 5: Introduce a new NOTE2 aligned with TS 36.101, and modified as follows:

- NOTE 2:  For FDD MDL = -0.5 is not applicable for in-band operation.

Proposal 6: Introduce a new NOTE3, aligned with TS 36.101, and modified as follows:

- NOTE 3:	For the carrier including NPSS/NSSS for in-band operation, MDL is selected from {-2,-1,0,1}.

Observation 7: The additional MDL values {-2,-1,0,1,2} would allow alignment with NR NTN Enhanced Channel Raster and would provide more consistency with Terrestrial spec in TS 36.101

Proposal 7: As a compromise, and if necessary, we can consider to keep the same MDL value restrictions for Anchor Carrier aligned for both Standalone and in-band.

# Conclusions

Observation 1: The current formula for Carrier Frequency (FDL) and MDL value set for Cat NB1 and NB2 specified in TS 36.102 and TS 36.108 as part of Rel-18 WI break forward compatibility with in-band deployment and backwards compatibility with Terrestrial spec.

Observation 2: The current Carrier Frequency and MDL value set specified in TS 36.102 and TS 36.108 prevent the introduction of in-band deployment support of NB-IoT NTN with NR NTN in the same SAN.

Observation 3: Restoring forward support for in-band deployment with NR would likely require the introduction of a new set of MDL, thus rendering the solution adopted in R4-2220812 completely moot.

Observation 4: This issue with forwards and backwards compatibility can have massive impact on upcoming deployments of NB-IoT NTN, and furthermore may break backwards compatibility with existing R17 based deployments and UE populations in the field, therefore it needs to be urgently fixed before R18 UE are being implemented, this may well be our last opportunity to fix it.

Observation 5: Rel-17 NB-IoT implementations currently in the market are likely to be still based on previous pre-R17 releases (i.e. aligned with terrestrial R13-14 UE as per TS 36.101), due to the lack of an NB-IoT NTN UE RF spec in Release 17.  Therefore the impact of this late change should be limited.

Proposal 1:  Revert the FDL formula for Carrier Frequency for Cat NB1 and NB2 to the original formula aligned with TS 36.101 (terrestrial spec) as follows:

- FDL = FDL_low + 0.1(NDL – NOffs-DL) + 0.0025*(2MDL +1)

Proposal 2: Adopt one of the solutions proposed in R4-2213243 and R4-2216637 and either change the MDL values for Standalone by introducing the additional MDL values or add a NOTE that introduces a 0.5 shift in MDL values for Standalone Non-Anchor carriers:

- Option 1)

  - Introduce a new set  of MDL as {-9.5, -8.5,-7.5, -6.5, -5.5, -4.5, -3.5, -2.5 -1.5, 0.5, 1.5, 2.5, 3.5, 4.5, 5.5, 6.5, 7.5, 8.5, 9.5}

- Option 2) – Preferred/simpler Option – Introduce a new NOTE5 as follows:

  - NOTE 5: For the carrier not including NPSS/NSSS for stand-alone operation, MDL = MDL + 0.5.

Proposal : Remove the restriction for in-band deployment by striking out NOTE1 as follows:

NOTE 1:  Guard-band operation and in-band operation for NB-IoT are not supported in this version of the specification.

Proposal 4: Modify existing NOTE2 to NOTE4 as follows and change the value of MDL for carrier including NPSS/NSSS for stand-alone operation as follows, aligned with TS 36.101:

- NOTE 24:  For the carrier including NPSS/NSSS for stand-alone operation, MDL = -0.5.

Proposal 5: Introduce a new NOTE2 aligned with TS 36.101, and modified as follows:

- NOTE 2:  For FDD MDL = -0.5 is not applicable for in-band operation.

Proposal 6: Introduce a new NOTE3, aligned with TS 36.101, and modified as follows:

- NOTE 3:	For the carrier including NPSS/NSSS for in-band operation, MDL is selected from {-2,-1,0,1}.

Observation 7: The additional MDL values {-2,-1,0,1,2} would allow alignment with NR NTN Enhanced Channel Raster and would provide more consistency with Terrestrial spec in TS 36.101

Proposal 7: As a compromise, and if necessary, we can consider to keep the same MDL value restrictions for Anchor Carrier aligned for both Standalone and in-band.

# References

- R4-2213243, On IoT NTN raster (Ericsson)

- R4-2216637, On system parameter for IoT over NTN (Ericsson)

- R4-2217747, Way Forward on UE RF and System Parameters for IoT NTN (Mediatek Inc.)

- R4-2220812, TP on the channel number offset for NB-IoT NTN (Huawei, Hisilicon)