3GPP TSG-RAN WG4 Meeting # 112	R4-2412463

Maastricht City, Maastricht, NL, Aug 19 – Aug 23, 2024

Agenda Item: 8.8.2.1

Source: MediaTek Inc.

Title: Discussion on NTN coexistence study

Document for: Discussion

Introduction

This document provides simulation results focusing on adjacent channel interference for coexistence between NTN and TN based on the NTN coexistence study [1].

Simulation scenarios and assumptions

Simulation scenario

According to WID [2], RAN4 to evaluate the impact on the uplink performance of TN. The aggressor is High power UE (HPUE) for NTN (aggressor) in UL. The power class (PC) include PC2, PC1.5 and PC1. The Victim is TN UL from base station. The coexistence scenarios shown in Table 1. This document presents the simulation observations relating to the coexistence of NTN with TN in UL scenarios.

Table 1: Simulation scenarios

| Satellite | Aggressor | Victim |
| --- | --- | --- |
| GEO | NTN UL | TN UL |

Refer to 6.4.4 Scenario 4: 2GHz NTN UL interfering TN UL [1]. NR-NTN GEO UL interfering the NR UL equipped with both AAS and non-AAS antenna that deployed in urban environment as the most stringent case. In our understanding, the same most stringent case could be applied for both NR NTN and IoT NTN for the coexistence study.

Proposal 1: According to coexistence study in TR 38.863 for NTN UL interfering TN UL, we assume that the most stringent case is GEO UL interfering the NR UL equipped with both AAS and non-AAS antenna for both NR-NTN and IoT NTN.

Results presented are for urban environments described in TR38.901, using both AAS and non-AAS antenna models in TR38.821.

For GEO with NTN UL as an aggressor and TN UL as a victim scenario, we simulated all NTN UEs are transmitting at three set of PC level (31 dBm, 29 dBm and 26 dBm) for HPUE. We reused the simulation procedure and assumption refer to TR 38.863 [1] as below in details.

TR 38.863 [1] 6.2.2 System parameters Table 6.2.2.4-1: Simulation assumptions of TN respectively based on NB-IoT and NR NB-IoT standalone; NR. Carrier frequency in GHz; 2; 2. Size of each nominal channel BW in MHz; 0.2; 20. Transmission bandwidth in MHz; 0.18; N/A. Environment; Urban macro Rural; Deployment scenario related, see Table 6.2.2.4-2.. Network layout; 19-sites [57 sectors] with wrap-around; 19-sites 57 sectors with wrap-around. Inter-site distance in meter; 500 for 2GHz band for UMA TBD For Rural; Deployment scenario related, see Table 6.2.2.4-2. System loading and activity; Full buffer 100%; See Table 6.2.1.1-1. Network location; TBD; See Table 6.2.1.1-1. DL subcarrier spacing; 15kHz; 15kHz. UL; See RP-152284[7]; OFDMA. DL power control; No; No. UL power control; TR 36.942[8] section 5.1.1.6 (set 1) by bandwidth scale, target SNR at BS is 15 dB; TR 36.942[8]. Frequency reuse; 1; 1. Number of scheduled UE per cell (DL); 1; 1. Number of scheduled UE per cell (UL); 3 for multi-tone (60kHz per UE), 12 for 15kHz single-tone, 48 for 3.75kHz single-tone; 3. UE antenna height in meter; 1.5; 1.5m. UE TX power in dBm; -40 to 23; -40 to 23. UE antenna gain in dBi; 0; 0. Building penetration loss; TR 45.820[9] Annex D.1; In pathloss model, TR 38.901[10]. Cell selection margin in dB; 3; 3. BS-MS min distance in meters; 35; 35. BS noise figure in dB; 5; 5. UE noise figure in dB; 9; 9. BS-UE path-loss model; TR 36.942[8] macro urban; TR 38.901[10]. Standard deviation of BS-UE log-normal shadow fading in dB; 10; Deployment scenario related, referring to TR 38.901[10]. Shadowing correlation; Inter-cell 0.5 Intra-cell 1; Inter-cell 0.5 Intra-cell 1. Link-level performance model; See Section 6.2.8 Throughtput-SINR mapping. UE distribution; Uniform. Evaluation metrics; SINR vs ACS (as victim); See Section 6.2.8 Throughtput or SNR loss criteria. Table 6.2.2.4-2: Deployment-related parameters of TN (2 GHz) Urban Macro; Rural Macro; Remarks. ISD in meters; 750; 7500; ITU-R Report M.2292[11]. BS Antenna height in meters; 25; 30. UE Outdoor/indoor; 100% Outdoor. UE height in meter; 1.5; 1.5; RP-200559 [12] 3GPP LS to ITU-R WP5D and ITU-R WP5D [IMT_Parameters] [13]. 6.3.1 Simulation procedure Adopt following simulation steps. - Step 1: Generate aggressor and victim networks. - NTN central beam is at satellite nadir, surrounded with 6 co-frequency beams. NTN FRFs higher than 1 need to be considered. Assume one NTN aggressor as default. - Deployment of TN network (19 cells with wraparound) refers to Table 6.2.1.1-1 - Step2: UE associations - TN UE are generated randomly inside the TN network, make sure enough TN UEs are associated to each TN sectors based on coupling loss. - Deployment of NTN UE refers to Table 6.2.1.1-1. - Step 3: Once association is done, round robin scheduling is used. BF weights are adjusted to point to the LOS direction between BS/SAN-UE. This is done for both victim and aggressor networks. - Step 4: Throughput is computed in the victim systems without considering ACI as below: ThputNO ACIbpshz=fSINRICI=fSN+IICI, where: IICI is the inter-cell interference. For TN-NTN SINR calculation, the satellite receiver off angle should be considered in the satellite receiver gain calculation when calculating SINR. Note that such angle is not considered in TR 38.821[6] section 6.1.3 equations. Thus those equations should be used for SINR calculation. - Step 5: Throughput is computed considering ACI as below: ThputACIbpshz=fSINRICI+ACI=fSN+IICI+IACI, where: IACI is the adjacent channel interference. - Step 6: RF parameters are determined based on the degradation cause by ACI as below: LossACI=1-ThputACIThputSINGLE

Network layout model

The system level simulation deployment is implemented as Network layout model as below [1].

TR 38.863 [1] 6.2.1 Network layout model Figure 6.2.1.1-1: Isolation distance for Case 1 As defined in Figure 6.2.1.1-1, isolation distance is the distance between the blue-dotted line which represents TN cell boarder and the red line. No UEs deployed in the isolation region is assumed to reduce the calculation complexity. No.; Combination; Aggressor; Victim; Which NTN cell/UE to observe?; Which TN/UE to observe?; Which TN cells in a TN to observe?. 4; TN with NTN satellite; NTN UL; TN UL; NTN cell: Nadir point. NTN UE: NTN UEs dropped at the edge of TN clusters; TN randomly placed in this NTN beam; Option 1: All active TN clusters which has the NTN UE(s) at its edge. Option 2: Only the TN sectors which have NTN UE(s) at their edges. Option 1 is the baseline and it is not precluded companies can follow Option 2 to bring results.

In this deployment, we assumed there’re 3 TN UEs in each TN sector and total 9 NTN UEs in the system. The isolation shown in Figure 1. is assumed according to TS 38.863 Figure 6.2.1.1-1[1] between NTN UEs and the edge of TN cluster. The isolation is 1500 meters (2*ISD) and the ISD for Urban Macro is 750 meters. NTN UEs placed at the TN cluster edge, those are at the radius of inter-site distance (ISD)*2 metres (isolation) from the TN cluster edge, as shown in Figure 1.

Figure 1: System level deployment for NTN UE and TN UE.

Simulation results

The key performance metric as highlighted in TR 38.863 [1] is the throughput loss ratio, thus simulation results in this document will focus on this.

Throughput Loss Ratio vs ACIR

The throughput loss ratio of each scenario is determined across different ACIR values to determine the minimum required ACIR of the victim system to prevent impact from interference.

Table 2: ACIR requirements

|  | BS Antenna type | ACIR Requirement @ 5%-tile Throughput Loss Ratio |  |  |
| --- | --- | --- | --- | --- |
| Satellite |  | GEO |  |  |
| Aggressor |  | NR NTN UL |  |  |
| Victim |  | TN UL (FDD) |  |  |
| PC level |  | PC1 (31 dBm) | PC1.5 (29 dBm) | PC2 (26 dBm) |
| Urban | Non-AAS | 20.822 dB | 18.822 dB | 15.704 dB |
|  | AAS | 25.349 dB | 23.349 dB | 19.978 dB |

Table 2 above lists the ACIR requirements for each PC level of the scenario listed in Table 1. The ACIR requirement is determined based on the 5%-tile throughput loss which should be less than 5%. Plots are given in Annex 1, with TN as the victim.

Based on the results in Table 2, the following observations can be made:

- No tighten ACIR requirement since NTN UL interference to TN UL with a required ACIR of < 26 dB for HPUE (PC1, PC1.5, PC2).

Observation 1: Based on the simulation results in Table 2:

- No tighten ACIR requirement since NTN UL interference to TN UL with a required ACIR < 26 dB.

Proposal 2: Based on the simulation results in Table 2, there is no interference impact or tighten ACIR requirement from NTN UL to TN UL for NR NTN HPUE.

Annex – 1 Throughput Loss Ratio vs ACIR with TN as victim network

Figure 2: TN UL Throughput loss ratio with nAAS @ GEO NR-NTN

Figure 3: TN UL Throughput loss ratio with AAS @ GEO NR-NTN

Reference

[1] 3GPP TR 38.863 Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence aspects.

[2] RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN