3GPP TSG-RAN WG4 Meeting #112					R4- 2412464

19th Aug, 2024 – 23rd Aug, 2024

Source: 	Qualcomm Incorporated

Title: 	Views on 8Rx UE demodulation and CSI requirements with inter-cell and intra-cell interference

Agenda item:	 8.16.2

Document for:	Discussion

1. Introduction

- Define the following UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices:

  - PDSCH demodulation and CQI reporting requirements with inter-cell interference with MMSE-IRC receiver

    - Reuse the Rel-17 interference model for 2/4Rx UE as a baseline

  - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver

    - Reuse the existing interference model for 2/4Rx UE as a baseline

In Rel-18 topic NR_ENDC_ RF_FR1_enh2 WI, 8Rx performance requirements for SU-MIMO (single user) were discussed. In Rel-19, 8Rx performance requirements with inter-cell, and intra-cell inter-user interference are to be defined for below objectives [1].

# 2. PDSCH Demodulation Requirements

## 2.1 Receiver types

In Rel-18, two types of 8Rx receiver capabilities, viz. baseline receiver (joint 8Rx), and simplified receiver (joint 4Rx) are defined. Since the objective of the WID is to define requirements for larger spatial domain freedom with 8Rx, and requirements with 4Rx are already specified in 38.101-4, we propose to discuss requirements only for baseline receiver.

Observation 1: Requirements with 4Rx are already defined in 38.101.

Proposal 1: RAN4 to define requirements only for SU-MIMO baseline receiver.

## 2.2 MCS choice

Since MCS13 is used in Rel-17, it is a good starting point for further studies.

Proposal 2: RAN4 to use MCS13 (Table1) for simulation studies as a starting point.

## 2.2 PDSCH demodulation requirements for inter-cell interference

Observation 2: In 38.101-4 with 4Rx, rank1/rank2 interferers, and single rank for target UE are agreed.

Since 8Rx provides higher degree of freedom, higher rank can be considered for both target UE and interferers to demonstrate additional gains with joint 8Rx processing. Although rank2 can be considered, we believe rank4 for target UE (serving cell) is quite higher than rank1 and allows high rank to interferers too. To have minimal requirements, we prefer RAN4 to use rank4 than another choice such as rank2. Moreover, rank4 is the right candidate to show higher interference rejection capability of 8Rx. The rank of interference cell can be discussed further.

Proposal 3: For inter-cell interference, RAN4 to use rank4 for target UE for both TDD and FDD.

In Rel-17 requirements, TDLA30-10, and 2 x 4 antenna configuration were used. Since Rel-17 settings are expected to be the used as a baseline, the same propagation model shall be used for initial simulations. For rank4 8Rx scenario, 4 x 8 is sufficient.

Proposal 4: RAN4 to use TDLA30-10 as the propagation model with 4 x 8 configuration for initial simulations.

# 3. PDSCH Demodulation requirements for intra-user inter-cell interference

In Rel-17 38.101-4 spec., only rank1 for target UE is used with 2 x 4 antenna configuration. With higher processing gain available in 8Rx, for the MU-MIMO scenarios, we would like RAN4 to consider the scenario with 8 x 8 antenna with rank4 for target UE, and rank2 for co-scheduled UE. This scenario is closer to a near cell UE (rank4) and cell edge UE (rank2).

Proposal 5: For MU-MIMO, RAN4 to use 8 x 8 antenna configuration with rank4 for target UE for both FDD and TDD.

# 4. CQI requirements with inter-cell interference

The objective of CQI tests is to ensure that UE better performance with the interferer than with white noise. Extending the logic presented in earlier sections, rank4 with 4 x 8 antenna config. can be considered for wideband CQI reporting with inter-cell interference.

Proposal 6: RAN4 to consider rank4 with 4 x 8 antenna configuration for wideband CQI fading reporting with inter-cell interference for both FDD and TDD.

# 5. Interference model

Rel-17 interference models in terms of number of interferers, power levels etc. can be used as a starting point for simulation studies.

Proposal 7: RAN4 to use Rel-17 interference models for initial simulation studies.

# 6. Release independence

Since interference requirements are being discussed in Rel-19 feature, release independence can be considered from Rel-18 for

Proposal 8: Rel-19 8Rx performance requirements shall be independent from Rel-18.

# 7. Conclusions

Following is the summary of the proposals.

Proposal 1: RAN4 to define requirements only for SU-MMO baseline receiver.

Proposal 2: RAN4 to use MCS13 (Table1) for simulation studies as a starting point.

Proposal 3: For inter-cell interference, RAN4 to use rank4 for target UE for both TDD and FDD.

Proposal 4: RAN4 to use TDLA30-10 as the propagation model with 4 x 8 configuration for initial simulations.

Proposal 5: For MU-MIMO, RAN4 to use 8 x 8 antenna configuration with rank4 for target UE for both FDD and TDD.

Proposal 6: RAN4 to consider rank4 with 4 x 8 antenna configuration for wideband CQI fading reporting with inter-cell interference for both FDD and TDD.

Proposal 7: RAN4 to use Rel-17 interference models for initial simulation studies.

Proposal 8: Rel-19 8Rx performance requirements shall be independent from Rel-18.

# References

[1] RP-241297, “Revised WID on NR demodulation performance: Phase 5”, 3GPP TSG RAN meeting, #104, June 17– June 20, 2024, Shanghai, China.