3GPP TSG-RAN WG4 Meeting #112	R4-2412466

Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	China Telecom

Title:	Discussion on PC2 RedCap for TDD bands

Agenda Item:	8.7.2

Document for:	Discussion

- Introduction

In RAN#104, the new WID on NR power class 2 RedCap (Reduced Capability) UE in FR1 was approved [1] with the following objectives captured:

- Apply the PC2 RF requirements for non-RedCap UE to RedCap UE, based on 1Tx architecture with 26 dBm PA.

  - The PC2 RF requirements include: UE maximum output power, Tx power tolerance, MPR, A-MPR and RSD requirements if needed.

  - For TDD bands, PC2 RF requirements for non-RedCap UE can be re-used for PC2 RedCap UE in a band agnostic way.

    - Note: Not differentiate the applicable form factors for TDD bands in the specification.

  - Check at RAN #106 on whether to define PC2 RF requirements for FDD bands and the applicable form factors for FDD bands (if defined).

Note 1: Only the operating bands for which PC2 on non-RedCap UE has been introduced can be considered for RedCap UE.

Note 2: PC2 for Rel-17 Redcap and Rel-18 eRedcap are considered for this WI.

This contribution discusses enabling PC2 RedCap for TDD bands as highlighted in the objectives.

- Discussion

How to enable PC2 RedCap for TDD bands in the band agnostic way

Tx Requirements

In TS 38.101-1, the current specification on the power class for RedCap is as follows:

6.2I		Transmitter power for RedCap

6.2I.1	Maximum output power for RedCap

For Redcap UE, the requirements for power class 3 specified in clause 6.2.1 apply.

As seen, from the RAN4 specification perspective, only PC2 is targeted in the new WID, thus it is suitable to add one sentence in the specification stating that PC2 also applies for RedCap UE.

Proposal 1: Add one sentence in the clause of 6.2I.1 to state that PC2 also applies for RedCap UE.

Rx Requirements

In TS 38.101-1, the current specification on the reference sensitivity for RedCap is as follows:

- 7.3I.2	Reference sensitivity power level

For a RedCap UE equipped with 2 Rx antenna ports, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.2-1a and Table 7.3.2-1b for the applicable operating bands….

For a RedCap UE equipped with 1 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3.2-1a and in Table 7.3.2-1b shall be modified by the amount given in ΔR1R in Table 7.3I.2-1 for the applicable operating bands….

Table 7.3I.2-1: Single antenna port reference sensitivity allowance ΔR1R

| Operating band | Channel bandwidth (MHz) | ΔR1R (dB) |
| --- | --- | --- |
| TDD band | 5, 10, 15, 20 | 2.5 |
| FDD band | 5 | 2.5 |
| FDD band | 10, 15, 20 | 3.0 |

Regarding PC3 RedCap for TDD bands, the reference sensitivity requirements are specified in Table 7.3.2-1b for 2Rx antenna ports and specified in Table 7.3I.2-1 by the amount of ΔR1R for 1Rx antenna port. Because no additional emission is received by the Rx antenna for TDD bands although transmission power is increased from PC3 to PC2, no need to relax the Rx reference sensitivity requirements.

Proposal 2: Keep reference sensitivity for RedCap for TDD bands unchanged since no additional emission is received by the Rx antenna

- Conclusions

In this contribution, we discuss enabling PC2 RedCap for TDD bands and have the following proposals:

Proposal 1: Add one sentence in the clause of 6.2I.1 to state that PC2 also applies for RedCap UE.

Proposal 2: Keep reference sensitivity for RedCap for TDD bands unchanged since no additional emission is received by the Rx antenna

The corresponding draft CR is also prepared and submitted in [2].

- References

RP-241665, New WID on NR power class 2 RedCap (Reduced Capability) UE in FR1, China Telecom, MediaTek, RAN#104, June, 2024

R4-2412467, draft CR on power class 2 RedCap for TDD bands, China Telecom, ZTE