3GPP TSG-RAN WG4 Meeting #112	R4-2412489

Maastricht, Netherlands, 19-23 August 2024

Source:	Nokia

Title:	On eEMR test cases

Agenda item:	5.24.2

Document for:	Discussion

Introduction

In this contribution we discuss eEMR test cases.

Discussion

In the last meeting, RAN4 introduced eEMR FR1 and FR2 test cases for:

- EMR measurements without reporting

- Cell reselection measurements without reporting

- Cell reselection measurements with valid reporting

Issue 2-2: test purpose for solution based on existing measurement for validity check Agreement: - For EMR, TC will verify UE does not send measurement report which is measured exceeds X second before paging reception. - For non-EMR, TC will verify - UE send measurement report which is measured within X second before [paging reception]. - UE does not send measurement report which is measured exceeds X second before [paging reception] if it is feasible for the testing.

While we understand that RAN4 already agreed not to introduce test case for EMR measurements with valid reporting, because it was seen that the existing Rel-16 EMR test cases cover this case sufficiently, we would like the group to reconsider this agreement.

In our view the Rel-16 test case does not sufficiently cover the impact of X, because X applies even if T331 is still running. In Rel-16, the UE is expected to report measurement results if T331 is running. If the duration of T331 is longer than X, the UE would be allowed to report results from the time before X, which is not correct UE behavior according to Rel-18. Therefore, we would propose to consider adding a test case for EMR-based measurements, where the UE reports valid measurements from the duration of X, similar to the test cases that have been defined for cell reselection measurement based early reporting.

Introduce test cases with valid reporting for FR1 and FR2 also for the case when the UE is configured with EMR measurements.

If the group agrees to Proposal 1, we have prepared a CR to cover these test cases in [1]. In the test cases T331 is running throughout T2 and T3, and the UE shall report results from T3.

Conclusion

The following observations and proposals were made in this contribution:

Proposal 1: Introduce test cases with valid reporting for FR1 and FR2 also for the case when the UE is configured with EMR measurements.

- References

[1] R4-2412490, CR for Rel-18 eEMR test cases, Nokia, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, Netherlands, 19-23 August 2024.