3GPP TSG-RAN WG4 Meeting #112	R4-2412491

Maastricht, Netherlands, 19-23 August 2024

Source:	Nokia

Title:	On RRM requirements for Phase 4 mobility enhancements

Agenda item:	8.23.2

Document for:	Discussion

Introduction

NR mobility enhancements Phase 4 work in RAN4 is set to start in the RAN4#112 meeting. The scope of the work item is to specify RRM requirements for the WI objectives:

- Inter-CU LTM

- Measurement enhancements for LTM

  - Event-triggered L1 measurement reporting

  - CSI-RS measurements for LTM

- Conditional LTM

In this contribution we discuss our views of the RRM impact of these objectives.

Inter-CU LTM

RAN2 and RAN3 have been working on inter-CU LTM based on the following objectives:

- Specify support for inter-CU Layer 2 Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - As secondary priority, support the case when NR-DC is configured and CU is acting as SN and MCG is unchanged - As secondary priority, support the case when NR-DC is configured, CU is acting as MN and SCG is unchanged or SCG is released - Note: The case that LTM is configured in both MCG and SCG is excluded - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support

From RRM requirement point of view, we do not see an immediate need to define separate requirements for inter-CU LTM. The procedure is similar to intra-CU LTM and the latency that RAN4 defined for intra-CU LTM in Rel-18 is already allowing a lot of flexibility. Hence, we propose:

Rel-18 LTM RRM requirements apply for inter-CU LTM.

Measurement enhancements for LTM

The objectives of measurement enhancements for LTM are:

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]

Event-triggered L1-reporting

In Rel-18, RAN4 defined measurement requirements for SSB-based LTM L1 measurements in sections 9.14 and 9.15. Related reporting requirements were defined according to RAN2 support for periodic, semi-persistent and aperiodic reporting. In Rel-19, RAN4 needs to define reporting requirements also for event-triggered L1 reporting for LTM candidate cells.

Related to event-triggered L1 reporting for LTM, RAN2 has made the following agreements in RAN2#126 meeting:

1. Event triggered L1 measurement should be designed for the following LTM purposes: - Select the candidate beam/cell to trigger early synchronization. - Select the target beam/cell and trigger LTM cell switch procedure. 2. For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement. 3. Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events. - Event LTM2: Beam of serving cell becomes worse than absolute threshold; - Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell; - Event LTM4: Beam of candidate cell becomes better than absolute threshold; - Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2. FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1. 4. Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5. 5. RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation. 6. For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

According to RAN2 agreements, event-triggered L1 reporting covers both SSB and CSI-RS based reporting. Event triggered reporting delay requirements would comprise a new section under the existing sections 9.14.3 and 9.15.3, and new sections under CSI-RS based LTM L1 measurement requirements, which are to be defined.

RAN4 to define event-triggered L1 reporting delay requirement for SSB based and CSI-RS based measurements.

When looking at the existing L3 event triggered reporting delay principles, this comprises the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. For example, for intra-frequency L3 measurements, event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify intra with index = TPSS/SSS_sync_intra + T SSB_measurement_period_intra  ms or T identify intra without index = TPSS/SSS_sync_ntra + T SSB_measurement_period_intra + TSSB_time_index_intra ms. Additional delays may be expected e.g. due to L3 filtering, and the delay does not take into account uncertainties related to for example other RRC signalling on the DCCH or availability of UL resources.

For event-triggered L1 reporting, the principle of L3 event triggered reporting may be used as the starting point. However, as the reporting conditions are evaluated based on L1 measurements, the reporting delay would obviously comprise L1 measurement period instead of L3 cell identification period. Rel-18 LTM L1 measurement delay TL1-RSRP_Measurement_Period_SSB_intra, TL1-RSRP_Measurement_Period_SSB_inter or TL1-RSRP_Measurement_Period_SSB_inter_withoutGap can be used as the starting point for the reporting delay.

For CSI-RS based reporting, the starting point is the measurement delay to be defined within this WI.

Starting point for the SSB-based LTM L1 event triggered reporting delay is TL1-RSRP_Measurement_Period_SSB_intra, TL1-RSRP_Measurement_Period_SSB_inter or TL1-RSRP_Measurement_Period_SSB_inter_withoutGap. CSI-RS based delay is pending on Rel-19 work.

Any uncertainties in the reporting delay need to be discussed based on RAN1/RAN2 reporting design. For example, RAN1/RAN2 are currently discussing whether some type of L1 filtering is to be included in the event-evaluation. Whether this or other aspects of the reporting have an impact on how the event-triggered L1 reporting delay is defined depends on further RAN1/RAN2 agreements.

The impact of RAN1/RAN2 event evaluation and reporting design to the reporting delay is up to further RAN1/RAN2 agreements.

Furthermore, RAN4 needs to discuss the impact of event-triggered reporting on other LTM requirements. Rel-18 LTM discussion focused on the assumption of periodic and network-controlled reporting, and the requirements were written accordingly. With event-triggered reporting there may not be reports available regularly, and the network is not aware when the report will be triggered and sent. It is also still unclear whether event becoming fulfilled may lead to one or more event-triggered reports. In any case, we think RAN4 will need to discuss whether event-triggered reporting has an impact at least on known TCI state condition for early TCI state activation and cell switch and the applicability of cell switch delay requirements

RAN4 to discuss potential impact of event-triggered L1 reporting to other LTM requirements, for example:
- Known TCI state condition for early TCI state activation and cell switch
- Applicability of cell switch delay requirements

Even further, one benefit of event-triggered reporting is that it gives a direct indication to the network about a candidate cell being a potential target for cell switch. Hence, RAN4 may discuss whether event-triggered reporting condition becoming fulfilled may also be considered as a trigger for some of the preparation steps for LTM, for example early DL sync, if not valid at the time of reporting condition becoming fulfilled, and early ASN.1 decoding and validity check. RAN4 should also discuss whether LTM L1 measurement delay requirements are impacted by event triggered reporting. For example, are the requirements the same before and after a reporting condition is fulfilled, or should similar relaxation as in Rel-18 requirements due to TCI state activation be considered after the reporting is triggered.

RAN4 to consider the impact of L1 event triggered reporting to:
- early DL sync (Tfirst-RS = 0 conditions)
- early ASN.1 decoding and validity check
- LTM L1 measurement requirements

CSI-RS measurements for LTM

Rel-18 LTM only supports SSB based intra- and inter-frequency measurements. One of the objectives of Rel-19 is to expand the LTM L1 measurements to include also CSI-RS. One of the motivations for this work is that the support of CSI-RS may enable greater throughput on the target cell immediately after cell switch.

It is currently up to RAN1 to agree which type of CSI-RS measurements are supported by LTM. However, we assume that at least periodic CSI-RS measurements will be supported. Hence, RAN4 can already start to discuss the CSI-RS based measurement requirements for periodic CSI-RS measurements.

RAN4 can start the requirements discussion of CSI-RS on periodic measurements while RAN1 is discussing about aperiodic and semi-persistent measurement support.

Currently LTM supports both SSB based intra- and inter-frequency measurements. Similarly, we can expect CSI-RS measurements to be supported on both.

Both CSI-RS intra- and inter-frequency measurements shall be supported. RAN4 can start by discussing intra-frequency CSI-RS measurement requirements first.

From the RAN4 requirements point of view, similar to SSB based requirements, we expect that reporting requirements for CSI-RS measurements are needed as well as a definition of the number of CSI-RS frequency layers, number of cells and number of CSI-RS resources.

RAN4 to discuss at least the following set of requirements for CSI-RS based LTM measurements:
- Measurement delay
- Measurement reporting requirements
- Measurement restrictions
- Scheduling availability

Furthermore, RAN4 needs to discuss the impact of CSI-RS measurements on the existing LTM requirements. Whether CSI-RS is supported as a QCL source for TCI state activation and/or whether PDCCH order with CSI-RS is to be supported is pending further RAN1 discussion and agreements. Such agreements may have an impact on the corresponding RAN4 requirements.

Impact of CSI-RS support for early DL/UL sync procedures is to be discussed once RAN1 has made agreements on these aspects

Conditional LTM

Conditional LTM is based on the following objectives:

- Specify support of conditional LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Prioritise intra-CU LTM - Checkpoint to review objective at RAN#105. RAN WG work to not start before this checkpoint - Specify RRM requirements related to the above objectives as necessary [RAN4]

As the WID states, the work on conditional LTM shall not start before RAN#105 meeting.

Conclusion

The following Observations and Proposals were made:

Proposal 1: Rel-18 LTM RRM requirements apply for inter-CU LTM.

Proposal 2: RAN4 to define event-triggered L1 reporting delay requirement for SSB based and CSI-RS based measurements.

Proposal 3: Starting point for the SSB-based LTM L1 event triggered reporting delay is TL1-RSRP_Measurement_Period_SSB_intra, TL1-RSRP_Measurement_Period_SSB_inter or TL1-RSRP_Measurement_Period_SSB_inter_withoutGap. CSI-RS based delay is pending on Rel-19 work.

Proposal 4: The impact of RAN1/RAN2 event evaluation and reporting design to the reporting delay is up to further RAN1/RAN2 agreements.

Proposal 5: RAN4 to discuss potential impact of event-triggered L1 reporting to other LTM requirements, for example: 
- Known TCI state condition for early TCI state activation and cell switch 
- Applicability of cell switch delay requirements

Proposal 6: RAN4 to consider the impact of L1 event triggered reporting to: 
- early DL sync (Tfirst-RS = 0 conditions) 
- early ASN.1 decoding and validity check 
- LTM L1 measurement requirements

Proposal 7: RAN4 can start the requirements discussion of CSI-RS on periodic measurements while RAN1 is discussing about aperiodic and semi-persistent measurement support.

Proposal 8: Both CSI-RS intra- and inter-frequency measurements shall be supported. RAN4 can start by discussing intra-frequency CSI-RS measurement requirements first.

Proposal 9: RAN4 to discuss at least the following set of requirements for CSI-RS based LTM measurements: 
- Measurement delay 
- Measurement reporting requirements 
- Measurement restrictions 
- Scheduling availability

Proposal 10: Impact of CSI-RS support for early DL/UL sync procedures is to be discussed once RAN1 has made agreements on these aspects

Observation 1: As the WID states, the work on conditional LTM shall not start before RAN#105 meeting.