3GPP TSG-RAN WG4 Meeting #112	R4-2412494

Maastricht, Netherlands, Aug 19 – Aug 23, 2024

Source:	Nokia

Title:	On Rel-19 NR MIMO Phase 5 RRM core requirements

Agenda item:	8.18.3

Document for:	Discussion

Introduction

MIMO is one of the key technologies in NR systems, and Rel-19, that started with RAN1#116, will bring important enhancements to both FR1 and FR2. Among the enhancements to be studied, the following is listed in the WID [1].

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

In this document, we present Nokia’s views regarding the core requirements that RAN4 need to discuss in relation to the WID, in particular about:

- Objective 1: UE-initiated beam management;

- Objective 5: Asymmetric DL sTRP/UL mTRP deployment scenarios.

UE-initiated beam management

Rel-15 to Rel-18 has specified periodic, semi-persistent and aperiodic CSI reporting as described in TS 38.214..In the scope of Rel-19, some use cases where UE could benefit from initiating the beam management/reporting are being identified. The UE-initiated beam management (UEIBM) feature refers to the case where the UE may be configured with at least one event/condition, and then the UE may start beam reporting if this at least one event/condition occurs or is satisfied. The improvements aimed by this feature are twofold:

- Reduce overhead, such that beam reports are sent by the UE only when needed, avoiding unnecessary beam reports that we may have in case of periodic reporting configured with small periodicities.

- Reduce latency, such that, as soon as certain events/conditions are met, a beam report can be sent, to avoid long delay that we may have in case of periodic reporting configured with large periodicities.

Regarding which event(s) may trigger a UEIBM report, the following has been agreed in RAN1:

Agreement(RAN1#116bis Changsha) On UE-initiated/event-driven beam reporting, regarding trigger-event detection for beam reporting, at least support Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam. - At least L1-RSRP is supported as quality metrics used for Event-2 - FFS: How the L1-RSRP is used to determine the triggering event (e.g. timer, counter, filter coefficient) - FFS: Whether the network controls how the L1-RSRP is used to determine the triggering event - Regarding RS measurement for the new beam for Event-2, down-select one or more of the following: - Option-3a (explicit manner): The RS(s) for new beam(s) are explicitly configured by RRC (e.g., reusing legacy configuration of RS measurement or in TCI-State) or MAC-CE - Option-3b (implicit manner): The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of activated TCI state(s). - Option-3c (implicit manner): The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of configured TCI state(s). - Note-1: ‘New/current beam’ is for discussion purpose. - Note-2: Other trigger events/quality metrics (e.g., L1-SINR) are not precluded. Note-3: For above implicit manner(s), if there are two QCL RSs in a TCI state, the measurement RS is derived from RS w.r.t. QCL-TypeD, if applicable. Agreement(RAN1#116bis Changsha) On UE-initiated/event-driven beam reporting, regarding Event-2, the threshold value is RRC configured. Agreement(RAN1 #117 Fukuoka) On UE-initiated/event-driven beam reporting, regarding trigger events, the following Event-1 and 7a/7b, are provided for down-selection or combination in RAN1#118 (possible outcome is that no new event is supported) - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7a: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the worst quality. - Event-7b: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the best quality.

From the RAN1 agreements, so far Event-2 has been agreed as an event that can trigger a UEIBM report. Event-2 is the event where the quality of at least one new beam becomes a “threshold value” better than the current beam. For example, the network may configure the UE via RRC with a certain threshold, e.g. 6 dB, and when the UE measures a new beam to have a L1-RSRP which is 6 dB better than the L1-RSRP of the current beam, then a UEIBM report is triggered.

Besides Event-2, some other events are currently under discussion, like Event-1, Event-7a and Event-7b.

On the other hand, for all events currently under discussion, L1 measurements are considered to trigger events and eventually reports.

For UEIBM, L1 measurements are considered for triggering events and reports.

Currently, L1-RSRP has been agreed as the baseline quality metric for trigger events, although other quality metrics like L1-SINR are under discussion in RAN1. However, such measurements can be performed by the UE as done in legacy, so there is no need for RAN4 to define new measurement requirements specific for UEIBM.

RAN4 to reuse legacy measurement requirements for measurement reports triggered by UEIBM.

Regarding the procedures that allow the UE to send back to the network the UEIBM reports, we have the following RAN1 agreements and working assumptions:

Agreement(RAN1#116bis Changsha) On beam report transmission procedure for UE-initiated/event-driven beam reporting, following modes are supported: Mode A (dynamically scheduling UCI by gNB): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) to request a resource for a second UL channel to carry beam report - FFS: Request format, e.g., SR or a new UCI type. - Step 2: UE detects the DCI format to indicate a resource for a second UL channel to carry beam report. - Step 3: Beam report is transmitted in second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - This mode is basic UE capability (i.e. all UE supporting UE-initiated/event-driven beam reporting should support this feature). - No new DCI format is introduced. Mode B (UCI in pre-configured resource(s) for second UL channel): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) notifying a second UL channel to carry beam report - FFS: Notification format, e.g., SR or a new UCI type. - Step 2: UE transmits the beam report in the second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - The notification in Step1 is in a separate reporting instance from the beam report in Step 2. FFS: Whether UE receives acknowledge information with response to each step for all modes For above procedures, cross-CC beam reporting is supported for both modes. - FFS: Details. Working Assumption(RAN1 #117 Fukuoka) On beam report transmission procedure for UE-initiated/event-driven beam reporting - For mode-A, at least support one-bit indication in the first PUCCH channel to request a resource for a second UL channel to carry beam report. - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling. - For mode-B, at least support one-bit indication in the first PUCCH channel to notify a second UL channel to carry beam report. - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling. - FFS: Whether/how to support multi-bit indication in the first PUCCH for mode-A and mode-B, e.g., when multi-event(s) are approved. - FFS: details on the dedicated RRC signaling - Above applies at least for the single CC case.

In summary, RAN1 is discussing two procedures for actually sending back the UEIBM report:

- Mode A, where the second UL channel for the UEIBM report is dynamically scheduled by the gNB; in such procedure the following steps are implemented:

  - The UE sends in a first PUCCH channel an UL indication to request to the gNB resources in a second UL channel to carry the UEIBM report;

  - The gNB indicates via DCI to the UE a resource in a second UL channel to carry the UEIBM report;

  - The UE sends the UEIBM report on the second UL channel.

- Mode B, where the second UL channel for the UEIBM report is pre-configured by the gNB; in such procedure the following steps are implemented:

  - The UE sends in a first PUCCH channel an UL indication to notify to the gNB that a UEIBM report will be transmitted in a second UL channel;

  - UE sends the UEIBM report on the second UL channel.

RAN1 has defined Mode A as the baseline, whereas Mode B is optional and may be supported only by some UEs.

For both Modes A and B,  the UE sends an UL indication in the first PUCCH channel once an event has been triggered. This indication is used in Mode A to request UL resources for the report, whereas in Mode B it simply notifies the gNB that the next preconfigured UL resource will be used for the UEIBM report.

With UEIBM, once an event has been triggered, the UE first sends an UL indication in the first PUCCH channel.

Event-triggered reporting requirements have already been defined for mobility use cases e.g. in clause 9.2.4 of TS 38.133 for “Event-triggered Reporting” and “Event-triggered Periodic Reporting”. In the existing requirements, the measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. For intra-frequency measurements, the delay shall be less than either Tidentify_intra_with_index or Tidentify_intra_without_index. These requirements cannot be applied in a straightforward way to the UEIBM feature, which involves L1 measurements and reporting.

Event triggered measurement reporting delay defined in 9.2.4 of TS 38.133 are not directly applicable to UEIBM.

To define event triggered beam reporting delay for L1, RAN4 first needs to consider L1 measurement delay in the reporting delay instead of L3 measurement delay. Second, RAN4 needs to discuss how to take into account the RAN1 reporting modes in the delay definition. There may also be some other uncertainties that need to be taken into account in the delay definition, and those may depend on further RAN1 agreements. In fact, considering that RAN1 defined when the UE is supposed to transmit a beam report for periodic and semi-persistent reporting in clause 5.2.1.4 of TS 38.214 and for aperiodic reporting in clause 6.1.2.1 of TS 38.214, we expect further work by RAN1 on that same issue also for the UEIBM feature in the coming meetings.

RAN4 to define new measurement reporting delay requirements for UEIBM. The details of the reporting delay depend on further RAN1 agreements.

Regarding the beams that the UE has to monitor and is allowed to report, we have the following RAN1 agreements and working assumptions:

Agreement(RAN1 #117 Fukuoka) Regarding RS measurement for the current beam for Event 2, for Option-2a, support the both schemes as follows. - Scheme-1: RS for current beam is the QCL RS in the indicated TCI state - FFS: Whether/How to handle the case if only one TRS is configured in the indicated TCI state. - Scheme-2: the RS for current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state. - Enabling one of either Scheme-1 or Scheme-2 is selected by NW. - FFS: The above selection is via an explicit RRC parameter or an implicit manner, e.g., if the RS(s) for new beam are CSI-RS, Scheme-1 is enabled; otherwise, Scheme-2 is enabled. - (Working Assumption) Enabling of either Scheme-1 or Scheme-2 should ensure the same RS type for RS measurement for current beam and new beam. The above QCL RS is the RS w.r.t. QCL-TypeD, if there are two QCL RSs in the indicated TCI state. Agreement(RAN1 #117 Fukuoka) Regarding explicit RS configuration for new beam measurement for Event 2, down-select the following options in the RAN1#118: - Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration; - FFS: The RS in the RS resource set can be updated by MAC-CE. - Option-2: A list of RS(s) for new beam measurement can be configured by RRC, and a subset can be activated for new beam measurement by MAC-CE. - FFS: If a list size is small, MAC-CE activation is not needed - Option-3: A list of RS resource (s) for new beam measurement can be configured by RRC, and a subset of RS resource(s) in the list can be provided for new beam measurement by indicated TCI state. - Others are not precluded. - FFS: Each RS for new beam measurement should be associated with a configured joint/DL TCI state which can be used as the indicated TCI state Agreement(RAN1 #117 Fukuoka) On UE-initiated/event-driven beam reporting, regarding UL signaling content(s) of L1-RSRP report depending on Event-2, in a report instance, at least Option-3 is supported - Option-3: N ≥ 1 beam(s) are reported in the report instance, - At least one of N reported beam(s) should satisfy the condition of Event-2 - N is configured by gNB - FFS: candidate value of ‘N’. - FFS: RRC can enable or disable whether current beam is always reported in addition to the N beams - FFS: Option-1/1a/1b/2. Above applies at least for the single CC case Agreement(RAN1#116) On UE-initiated/event-driven beam reporting, regarding signaling content(s), at least support DL RS resource indicator and L1-RSRP - FFS: Study and decide whether additional contents can be supported. - FFS: L1-RSRP format, e.g., absolute and/or differential value. - Note: Above does not imply to preclude discussion on L1-RSRP filtering. - The actual reported content depends on the triggering event - Support of one or multiple events will be discussed separately

For Event-2, the UE is supposed to monitor the current beam and a certain number of new beams. More specifically:

- The RS for the current beam is related to the indicated TCI state, and there are two options under discussion, in particular if the RS is the actual RS in the indicated TCI state or if the RS is the SSB which is QCLed with the actual RS in the indicated TCI state;

- The RS(s) for the new beam(s) are explicitly configured by the network via RRC, with few sub-options under discussion still on how exactly to do that.

The RS(s) for current beam and new beams that the UE is supposed to monitor are configured by the network.

Once Event-2 has been triggered, the UE starts either Mode A or Mode B procedure to send the UEIBM report to the network. The UE then reports N beams, with N≥1 configured by the network via RRC, where at least one of those N beams satisfies Event-2. The background is that, once the event is triggered, the UE reports the top-N beams, although not all such N beams may satisfy the conditions of Event-2. Note that however some other options are still under discussion, where the network configures the maximum number of beams Nmax that can be reported by the UE, and then the UE reports N≤ Nmax beams.

As part of the UEIBM report, the UE may be allowed/configured to report also beams that do not satisfy the conditions to trigger the event.

Moreover, the following two metrics/parameters can be included for a beam part of the UEIBM report:

- DL RS resource indicator, in the form for instance of CRI or SSBRI;

- L1-RSRP (absolute or differential).

However, RAN1 has not explicitly agreed yet that L1-RSRP is reported for every single beam that is included in the UEIBM report. In fact, although always reporting the L1-RSRP would be the simplest solution, L1-RSRP could be skipped, thus saving some bits in the UEIBM report, for beams that meet the conditions to trigger Event-2 because such beams are then a threshold value better than the current beam, and for the network that could be enough information to switch toward such new beam, without the need to exactly know the precise L1-RSRP of such new beam. On the other hand, in such scenario, we may have situations where the network decides, based on the UEIBM report, to send a MAC-CE TCI switching command toward a TCI state whose RSRP has not been reported (although it has been measured by the UE): thus, based on the known conditions for TCI states described in clause 8.15.2 of TS 38.133, such TCI state is unknown with longer TCI switching requirements that apply.

Therefore, RAN4 needs to discuss the impact of UEIBM on known/unknown conditions for TCI states.

RAN4 to discuss the impact of UEIBM on known/unknown conditions for TCI states depending on RAN1 reporting design.

Asymmetric DL sTRP / UL mTRP

Figure 1 shows examples of the asymmetric DL sTRP / UL mTRP scenario. In such scenario, the DL transmission is from only a single TRP, depicted in the figure as “DL/UL capable TRP”. When using this feature, one or more “UL-only TRPs” may be deployed, potentially close to the cell edge in order to improve coverage. A UE may be scheduled for UL transmissions toward more than one TRP. UL transmissions are scheduled by the DL/UL capable TRP, which is the only TRP sending data and control information to each UE. Moreover, there is no restriction for a certain UE on which TRPs the UE is scheduled for UL transmissions, as we may have with reference to the figure below:

- UE1/UE2 transmitting toward an UL-only TRP and the DL/UL capable TRP,

- UE3 transmitting only toward the DL/UL capable TRP (legacy UE),

- UE4 transmitting only to an UL-only TRP,

- UEs (not depicted in the figure) transmitting to even more than one UL-only TRP.

Under these assumptions, the asymmetric DL sTRP / UL mTRP scenario can be identified as a sDCI mTRP scenario, and therefore sDCI mTRP requirements should be considered as baseline requirements.

The asymmetric DL sTRP / UL mTRP scenario is a sDCI mTRP scenario.

Figure 1. Asymmetric DL sTRP/UL mTRP scenario

Regarding asymmetric DL sTRP / UL mTRP scenario, we have the following RAN1 agreements and working assumptions:

Agreement(RAN1 #116 Athens) For the asymmetric DL sTRP/UL mTRP deployment scenarios, support to associate a UL TCI state with a PL offset: - When a UL TCI state associated with a PL offset is applied for the PUSCH/PUCCH/SRS transmission, the UE shall calculate the Tx power of the PUSCH/PUCCH/SRS based on the DL PL RS and PL offset associated with this UL TCI state. - Reuse the legacy uplink power control formulation by replacing legacy PL with UL PL which is derived from the DL PL RS and the PL offset. - FFS: The UE can update UL PL in a way that new UL PL = current UL PL + an update delta indicated by the NW. - Note: it does not intend to increase the number of maintained PLs per cell. - FFS: whether to support associating joint TCI state (if supported) with a PL offset. Agreement(RAN1 #116 Athens) To facilitate the asymmetric DL sTRP/UL mTRP deployment scenarios, support two closed-loop PC adjustment states for SRS in one CC, both of which are separate from that of the PUSCH. Conclusion(RAN1 #117 Fukuoka) For the asymmetric DL sTRP/UL mTRP deployment scenario, reuse the rel-17 unified TCI/ICBM and rel-18 unified TCI framework: - When rel-17 unified TCI/ICBM is configured: - For FR1: one joint TCI state or {one DL TCI state + one UL TCI state} can be applied. - For FR2: {one DL TCI state + one UL TCI state} can be applied. - When rel-18 unified TCI is configured: - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied. - For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

Most of the work by RAN1 up to now has been on defining two closed-loop power control adjustments for SRS. In this scenario, as the UL-only TRPs do not send any RS. The UE monitors only one DL pathloss RS, which is transmitted by the DL/UL capable TRP. The solution is implemented:

- by having the power control loop with the DL/UL capable TRP as in legacy, and then

- by introducing for the power control loop with an UL-only TRP a “pathloss offset”, which is shared with the UE by the DL/UL capable TRP and used by the UE for setting the power of the UL transmission toward such UL-only TRP; such “pathloss offset” is computed at the network side based on the UE UL transmission toward the UL-only TRP and DL/UL capable TRP, which are assumed to have good backhaul connection (as eventually this is a sDCI mTRP deployment).

The Rel-18 MIMO WI included the two TA enhancements for mDCI mTRP operation. Such enhancements enable the UE to transmit with two different timings toward two different TRPs. The main motivation was that the propagation delays may become large when compared to the CP length, in particular in FR2, where different UE beams/panels are used for UL transmission toward two non-collocated TRPs.

As a numerical example to highlight the limitations of single TA with non-collocated mTRP, let us consider a worst-case scenario where, with reference to the simple deployment in Figure 1, UE1 is close to the DL/UL capable TRP and UE2 is close to UL-only TRP1, and both UEs transmit in UL toward both the TRPs. Let us also assume that a single TA is used, with UL transmissions of the two UEs timely aligned at the DL/UL capable TRP, for a system configured with 120 kHz SCS, and with therefore CP length of 590 ns. In such deployment, assuming that the UL transmissions from the two UEs should be received at each TRP with a time difference not exceeding 50% of the CP length, when we also take into account some margin for the UL transmit timing error (i.e., 2xTe). We obtain that the inter-TRP distance must be less than about 8.5 m, strongly limiting the deployment opportunities.

Inter-TRP distance is strongly limited without two TA operation in asymmetric DL sTRP / UL mTRP scenario.

Maximum inter-TRP distance in worst-case scenarios using single TA operation in asymmetric DL sTRP / UL mTRP scenario is less than 8.5 m in FR2.

In fact, although Rel-18 focus was on mDCI because a) inter-cell TRP deployment (note that in sDCI intra-cell is assumed) and b) non-ideal backhaul make the issues related in having a single TA much more severe when compared to sDCI, we think that using single TA for sDCI has very limited applicability in FR2. Therefore, the two TAs enhancements should be introduced for sDCI to enable asymmetric DL sTRP / UL mTRP also in FR2.

RAN4 to not progress on FR2 asymmetric DL sTRP / UL mTRP until there is decision on two TAs enhancements for sDCI by RAN plenary.

Regarding the unified TCI framework applicability, both joint DL/UL TCI states and separate DL and UL TCI states configurations are supported, although joint DL/UL have been agreed up to now only for FR1. With separate DL and UL TCI states, the UE is indicated with one separate DL TCI state associated with the DL/UL capable TRP, and up to two UL separate TCI states associated to the TRPs (the UL-only and/or the DL/UL capable) toward which UL transmissions are scheduled.

Rel-18 specified in TS 38.133 the TCI switching requirements for sDCI in clauses 8.21 for the downlink and 8.23 for the uplink, considering both joint DL/UL and separate DL and UL configurations.

When using separate DL and UL TCI states, as DL transmission happens only from the DL/UL capable TRP that is also sending the TCI switching command, either via MAC-CE or via DCI, DL TCI switching requirements in 8.21 apply in a straightforward way. To be even more precise, for example with MAC-CE from clause 8.21.3, we have that “When a MAC CE for DL TCI state switch indicates single TCI state switch, requirements specified in clause 8.15.3 apply”, meaning that eventually DL TCI switching requirements for sTRP defined in 8.15 apply for asymmetric DL sTRP / UL mTRP.

On the other hand, as UL-only TRPs do not transmit any RS such as SSBs, there may be some impact on:

- UL TCI switching requirements in case a UE is configured with separate DL and UL TCI states,

- Joint DL/UL TCI switching requirements in case a UE is configured with joint DL/UL TCI states.

RAN4 to discuss TCI switching requirements for asymmetric DL sTRP / UL mTRP.

Conclusion

In this paper, the following Observations and Proposals were made:

Observation 1: For UEIBM, L1 measurements are considered for triggering events and reports.

Proposal 1: RAN4 to reuse legacy measurement requirements for measurement reports triggered by UEIBM.

Observation 2: With UEIBM, once an event has been triggered, the UE first sends an UL indication in the first PUCCH channel.

Observation 3: Event triggered measurement reporting delay defined in 9.2.4 of TS 38.133 are not directly applicable to UEIBM.

Proposal 2: RAN4 to define new measurement reporting delay requirements for UEIBM. The details of the reporting delay depend on further RAN1 agreements.

Observation 4: The RS(s) for current beam and new beams that the UE is supposed to monitor are configured by the network.

Observation 5: As part of the UEIBM report, the UE may be allowed/configured to report also beams that do not satisfy the conditions to trigger the event.

Proposal 3: RAN4 to discuss the impact of UEIBM on known/unknown conditions for TCI states depending on RAN1 reporting design.

Observation 6: The asymmetric DL sTRP / UL mTRP scenario is a sDCI mTRP scenario.

Observation 7: Inter-TRP distance is strongly limited without two TA operation in asymmetric DL sTRP / UL mTRP scenario.

Observation 8: Maximum inter-TRP distance in worst-case scenarios using single TA operation in asymmetric DL sTRP / UL mTRP scenario is less than 8.5 m in FR2.

Proposal 4: RAN4 to not progress on FR2 asymmetric DL sTRP / UL mTRP until there is decision on two TAs enhancements for sDCI by RAN plenary.

Proposal 5: RAN4 to discuss TCI switching requirements for asymmetric DL sTRP / UL mTRP.

- References

- RP-232915, Proposal for Rel-19 MIMO Objectives, RAN#102, Edinburgh, Scotland, December 11-15, 2023.