3GPP TSG-RAN WG4 Meeting #112	R4-2412497

Maastricht, Netherlands, 19th – 23rd August, 2024

Title:	On performance metric for NTN OTA

Source:	OPPO

Agenda item:	8.12.2.2

Document for:	Approval

- Introduction

In the last RAN4 #111 meeting in Fukuoka, the topic of the performance metric for NR-NTN handheld was discussed. 5 categorized directions were summarized in the way forward, with Direction 1 excluded [1].

This contribution further discusses our views and considerations on the performance metric for NTN OTA, and provides proposals on the down selection of performance metric.

- Discussion

From the agreed directions, generally, there are two issues which should be considered, i.e. one issue is whether single point requirement is needed, the other issue is which part of sphere is required for NTN communication.

Issue 1, whether single point requirement is needed?

Before answer the question, some background information is considered. From link budget perspective, usually minimum radiated power/EIRP is taken into account, which guarantees the basic wireless connection, regardless the connect direction. However, in some satellite communication usage scenarios, with the software assistant, the UE can point to the communication satellite with its peak radiated direction, automatically or manually. For such usage scenarios, peak radiated EIRP/EIS is more meaningful. To summarize, whether single point requirement (peak EIRP/EIS) is needed depends on the usage scenarios which require the UE to point to the communication satellite with its peak radiated direction. In the other word, Peak EIRP/EIS is not needed if the UE is not required to point to certain direction during the satellite communication.

Observation 1: Peak EIRP/EIS is not needed to be measured if the UE is not required to point to certain direction during the satellite communication.

Proposal 1: If the UE is not required to point to certain direction during the satellite communication, single point requirement is not specified, i.e. exclude Direction 2 and Direction3.

Issue 2, whether partial or full of sphere is required for NTN communication?

During NTN communication, clearly, the satellites that UE connect to are distributed in the sky, i.e. upper sphere of UE. Because of large transmission range, the reflected energy in the communication can hardly be collected. Therefore, partial sphere of UE can satisfy the requirement is enough for NTN communication.

Proposal 2: Partial sphere of UE is a propriate performance matric for NTN OTA. The range of partial sphere can be further studied.

- Conclusions

This contribution further discusses our views and considerations on the performance metric for NTN OTA. And provide the following proposal.

Observation 1: Peak EIRP/EIS is not needed to be measured if the UE is not required to point to certain direction during the satellite communication.

Proposal 1: If the UE is not required to point to certain direction during the satellite communication, single point requirement is not specified, i.e. exclude Direction 2 and Direction3.

Proposal 2: Partial sphere of UE is a propriate performance matric for NTN OTA. The range of partial sphere can be further studied.

- References

[1] R4-2409931 WF for [111][338] TRP_TRS_MIMO_OTA vivo