# 3GPP TSG-RAN WG4 Meeting # 112	R4-2412501

# Maastricht, Netherlands, Aug. 19th – Aug. 23rd, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4831 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_MG_enh2-Core) CR on 38.133 MG enh2 on Pre-MGs Core |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In RAN4 #111 meeting, the UE behaviour when the UE doesn’t support dynamic collision is agreed. For UE not supporting dynamic collision, the MG will be drop if overlapped with Pre-MG, regardless whether Pre-MG (with higher priority) is activated or deactivated, including the case when the MG overlaps with the Pre-MG activation/deactivation procedure. - The UE’s behaviour when two gaps are collision is unclear |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify the UE’s behaviour when UE not supporting dynamic collision but supporting concurrentMeasGapsPreMG-r18. - When collision happens, UE will perform measurement in non-dropped gap and can schedule data in dropped gap. - Update the wording |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec. is unclear and incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.12.3, 9.1.12.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------NEXT CHANGE---------------------------

#### 9.1.12.3	Collisions involving Pre-MG(s)

A collision happens between occasions of two measurement gaps where the higher priority gap is a Pre-MG and the lower priority gap may or may not be a Pre-MG is called as dynamic collision.

For a UE that supports [dynamic collision capability]:

- Collisions between a Pre-MG and a measurement gap may occur only when the Pre-MG is activated and satisfy the collision rule defined in clause 9.1.8.3. No collisions can occur between a per-FR Pre-MG and a per-FR measurement gap when they are configured in different FRs.

- Collisions between two Pre-MGs may occur only when both Pre-MGs are activated and satisfy the collision rule defined in clause 9.1.8.3. No collisions can occur between per-FR Pre-MGs when they are configured in different FRs.

When the collision between a Pre-MG and a measurement gap or two Pre-MGs satisfy the collision rule defined in clause 9.1.8.3, the UE shall perform measurements in the occasion of the non-dropped gap except the scenario specified in clause 9.1.8.4.

For a UE that does not support [dynamic collision capability]:

- Collision between a Pre-MG and a measurement gap or two Pre-MGs may occur when they satisfy the collision rule defined in clause 9.1.8.3 regardless of whether Pre-MG is activated or deactivated.

When the collision between a Pre-MG and a measurement gap or two Pre-MGs satisfy the collision rule defined in clause 9.1.8.3, the UE shall perform measurements in the occasion of the non-dropped gap regardless of whether it collides with the Pre-MG activation or deactivation procedure.

No collisions can occur between a per-FR Pre-MG and a per-FR measurement gap when they are configured in different FRs.

No collisions can occur between per-FR Pre-MGs when they are configured in different FRs.

The UE is expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI in the corresponding NR serving cells in the slots of the configured Pre-MG that are dropped according to the requirements in clause 9.1.8.4.

The requirements for concurrentMeasGapsPreMG-r18 apply provided that the two measurement gaps colliding with each other are configured with different priorities.

#### 9.1.12.4	Collision between Pre-MG activation/deactivation and measurement gap

A measurement gap occasion and a Pre-MG activation/deactivation procedure collide when the ending point of the Pre-MG activation/deactivation procedure occurs anywhere within a time period starting 4ms before the starting point of the gap occasion and ending 4ms after the ending point of the gap occasion. The ending point of the Pre-MG activation/deactivation procedure is defined in clause 8.19.5.3.

For a UE that supports [dynamic collision capability]:

- When a collision occurs between a measurement gap occasion and a Pre-MG activation procedure, and the Pre-MG is configured with higher priority, the UE shall perform measurements during the measurement gap occasion and the activation of the Pre-MG is delayed until 5ms after the ending point of the measurement gap occasion.

- When a collision occurs between a measurement gap occasion and a Pre-MG deactivation procedure, and the Pre-MG is configured with higher priority, the measurement gap occasion shall be (or remain) dropped if the measurement gap occasion collides with an occasion of the Pre-MG.

When the activated Pre-MG and measurement gap satisfy the collision rule defined in clause 9.1.8.3 and the Pre-MG is configured with lower priority, the UE shall perform measurements in the occasion of the measurement gap regardless of whether it collides with the Pre-MG activation procedure or collides with the Pre-MG deactivation procedure.

The UE is expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI in the corresponding NR serving cells in the slots of the configured Pre-MG that are dropped according to the requirements in clause 9.1.8.4.

--------------------END OF CHANGES--------------------------