3GPP TSG RAN WG4 #112	R4-2412505

Maastricht, Netherlands, Aug. 19th – Aug. 23rd, 2024

Agenda Item:	8.22.4.2

Source: 	Ericsson

Title:	Discussion on LP-WUS RRM requirement

Document for:	Discussion

# Introduction

In last RAN4 meeting, RAN4 had discussed the LP-WUS/WUR WI and made the following agreement [1]. In this contribution, we continue the discussion on the RRM impact of LP-WUR operation.

Issue 1-1-2: Cases/states to be considered for RRM relaxation Agreement: Discuss the RAN4 requirements first for the following case #1, and FFS for case #2 to #5. RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #1 Fully offloading case; Off; Off: FFS the condition and the details; ON. RAN4 to further discuss case #2 to #4: RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #2 Relaxed case a; On with relaxation measurement; Off; ON. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. #4 Relaxed case c; Off; On, FFS the condition and the details; ON.

- MR RRM relaxation

Based on RAN1 and RAN2’s discussion, different scenarios to support the RRM relaxation are proposed as shown below:

- MR relaxation with LR

- MR offloading by LR

In last RAN4 meeting, RAN4 agreed the MR fully offloading case, but the other MR relaxation case is FFS.

RAN4 to further discuss case #2 to #4: RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #2 Relaxed case a; On with relaxation measurement; Off; ON. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. #4 Relaxed case c; Off; On, FFS the condition and the details; ON.

When both NW and UE supports LP-WUS feature, different scenarios are valid by the NW using different thresholds and the UE will need to adapt its measurement behaviour to the respective zones. Adapting herein comprises turning ON/OFF the MR or LR receiver chains.

In legacy release, when neighbour cell measurement relaxation criteria were introduced, UE may perform neighbour cell measurements from normal measurement, relaxed measurement till stop the measurement. Now in Rel-19 LP-WUS, different serving cell measurement relaxation scenarios will be introduced. As shown in the figure below, different thresholds for neighbour cell measurement and serving cell measurement can be introduced for different serving cell and neighbour cell combinations.

Figure 1 Serving and neighbour cell measurement scenarios

For example, when serving cell measurement quality is larger than a threshold(other criteria may also introduced in RAN2), UE may enter the MR+LR relaxation serving cell measurement(UE enters LR measurement together with MR measurement relaxation). At the same time, both neighbour cell measurement relaxation and neighbour cell measurement ceased scenarios are possible(shown in figure scenario 3 and 2). One of the reasons to have such hybrid measurement is to achieve more power saving gain for MR relaxation compared with legacy serving cell measurement. Another reason is the LP-SS receiver’s performance may not good enough, MR is used to assist retuning the measurement by LP-SS. Furthermore, when NW configures high priority neighbour cells and UE fulfils both MR fully offloading and neighoubr cell offloading criteria, MR serving cell measurement will be OFF but neighbour cell measurement will be ON.  It is also important that NW can skip the MR+LR relaxation scenarios and only configure MR measurement and LR measurement by different threshold configurations. For example, NW will only configure one threshold to differentiate the MR measurement and LR measurement.

Figure 2 MR relaxation with LR measurement

Observation 1: LP-WUR may need MR assistance to retune the timing/frequency when the serving cell quality is not very well(LR fully offloading).

Observation 2: MR measurement can be relaxed by LR due to good quality.

From network’s perspective, all the mentioned scenarios are possible and can be easily controlled by NW’s thresholds’ indication. Furthermore, it is worth noting that whether UE enters any power saving mode is also up to UE’s implementation. In other words, NW can only configure/activate a sub-set of scenarios to UE.

Proposal 1: When UE supports Rel-19 LP-WUS capability, the following scenarios are possible.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #2 Relaxed case a | On with relaxation measurement | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |
| #4 Relaxed case c | Off | On | ON |

Proposal 2: Whether any LR based serving cell RRM relaxation cases are valid is fully up to NW’s configuration, such as NW can enable/disable any RRM relaxation scenario based on the configured thresholds.

Proposal 3: Serving cell measurement relaxation/offloading with LR measurement is independent with neighbour cell measurement relaxation/offloading, such as NW can control serving and neighbour cell measurement with different thresholds.

Proposal 4: MR RRM further relaxation without LR measurement is out of scope in Rel-19.

Proposal 5: RAN4 to send LS to RAN2 to discuss the criteria to enter/exit each RRM relaxation scenario which should be captured in RAN2 spec.

- Entry/exit condition for LP-WUS monitoring in RRC-IDLE and INACTIVE

In legacy NR, UE will use the SSB to measure and evaluate the serving cell’s quality with the cell selection S criterion. If UE has evaluated that serving cell’s quality doesn’t fulfil the cell selection S criterion in N consecutive DRX cycles, UE will initiate all neighbour cells’ measurement.

When LP-WUS operation introduced, UE will use LP-WUS to monitor serving cell’s quality. The conditions to enter/exit the LP-WUS monitoring shall be defined, e.g. based on serving cell RSRP or/and RSRQ.

To avoid negative impact on paging or cell re-selection when using WUR, the following entry/exit pricinples should be followed.

- The entry should be made carefully and delay is not critical.

- The exit should be made fast because LP-SS measurement and mobility detection may delay for the exit condition to be fulfilled.

Proposal 6: To minimize the negative impact on paging and cell reselection, the following entry/exit principles should be followed if RAN4 decides to define entry/exit requirement.

- The entry should be made carefully, and delay is not critical.

- The exit should be made quickly because LP-SS measurement and mobility detection may delay for the exit condition to be fulfilled.

However, considering this issue may be discussed in different WGs at the same time, it’s better to avoid duplicate discussion among the WGs. In our understanding, RAN4 can focus on defining the requirement based on measurement simulation evaluation.

Proposal 7: RAN4 to discuss the requirement about exit the LR based on the LP-RSRP/LP-RSRQ simulation results.

Proposal 8: RAN4 to discuss the requirement about entry the LR based on both MR SSB RSRP/RSRQ and whether includes LR evaluation.

- RRM measurement

LP-RSRQ definition

In RAN1 117, the LP-RSRQ definition was agreed as follow. RAN4 can reuse the definition from RAN1 to start the evaluation on LP-RSRQ.

Agreement For LP-SS based LP-RSRQ, LP-RSRP and LP-RSSI are measured within the same bandwidth. Agreement For LP-RSSI definition for LP-RSRQ, the following is agreed - Option 1: LP-RSSI is the linear average of total received power in ON and OFF LP-SS OOK symbols. Note: Above does not constrain LP-SS sequence design for OOK Working Assumption From RAN1 perspective, for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively. - Above is applicable for both time-domain processing or frequency-domain processing - Above does not imply that RAN1 will introduce LP-SSS-RSRP and LP-SSS-RSRQ in the specifications

Proposal 9: RAN4 to follow RAN1’s LP-RSRP and LP-RSRQ definition to evaluate the LP-SS performance.

SSB periodicity

UE can also use PSS/SSS for LP-WUR to evaluate the serving cell quality to decide whether to exit/enter the LP-WUR. In this case, to compare the performance with LP-SS, we propose to follow the same periodicity as LP-SS, such as 320ms to evaluate the PSS/SSS based LP-WUR exit condition. At the same time, we also notice that the power consumption due to LP-WUR is very less than the legacy MR. Thus, the power consumption of fast PSS/SSS detection(compared with 1 measurement sample per DRX in legacy IDLE mode) is negligible.

Proposal 10: RAN4 to discuss the PSS/SSS periodicity to evaluate the LP-WUR exiting condition in LR.

MR relaxation factor

As we mentioned above, a hybrid MR and LR measurement to evaluate the serving cell quality is possible. One of the reasons is LR performance may not good enough and need MR’s assistance. Thus, when RAN4 evaluates the scaling factor of MR relaxation, RAN4 should not only consider the MR relaxation power saving gain but also consider LR’s performance.

Proposal 11: RAN4 should consider both the MR relaxation power saving gain and LR’s performance to define MR relaxation scaling factor.

Proposal 12: RAN4 to evaluate how long the MR wake up is needed to assist LR measurement.

- Accuracy

In last meeting, RAN4 spent a long time to discuss how to define the accuracy requirement. Two possible way forwards were proposed.

- Define a new LP-SS accuracy directly in performance section

- Follow the legacy IDLE mode requirements to reflect accuracy in reselection margin

We prefer the second option to reuse the same approach where the legacy SSB-based measurements in IDLE mode are used with a margin other than creating a new LP-SS performance part. The NR SSB performance is also defined because the SSB performance can be verified in CONNECTED mode, but LP-SS is only used in IDLE mode which cannot be verified due to no measurement reporting.

Proposal 13: No dedicated accuracy requirement in the performance section are defined for LP-SS based measurement and reflect the accuracy performance as a margin in the IDLE/INACTIVE mode core requirement.

- LP-WUR in RRC-CONNECTED mode

RAN4 shall focus on developing the RRM requirements for IDLE and INACTIVE mode during this meeting since not many agreements have been reached in other WGs. Discussions on CONNECTED mode shall be postponed until any agreements are achived in other WGs regarding, and if so RAN4 can check the RRM requirements for those at later stage.

Proposal 14: LP-WUS CONNECTED mode requirements is postoponed until more progress is achieved in other WGs.

- Summary

In this contribution, we have initially discussed the LP-WUS operation. Based on the discussions, we have made following proposals and observations:

Proposal 1: When UE supports Rel-19 LP-WUS capability, the following scenarios are possible.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #2 Relaxed case a | On with relaxation measurement | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |
| #4 Relaxed case c | Off | On | ON |

Proposal 2: Whether any LR based serving cell RRM relaxation cases are valid is fully up to NW’s configuration, such as NW can enable/disable any RRM relaxation scenario based on the configured thresholds.

Proposal 3: Serving cell measurement relaxation/offloading with LR measurement is independent with neighbour cell measurement relaxation/offloading, such as NW can control serving and neighbour cell measurement with different thresholds.

Proposal 4: MR RRM further relaxation without LR measurement is out of scope in Rel-19.

Proposal 5: RAN4 to send LS to RAN2 to discuss the criteria to enter/exit each RRM relaxation scenario which should be captured in RAN2 spec.

Proposal 6: To minimize the negative impact on paging and cell reselection, the following entry/exit principles should be followed if RAN4 decides to define entry/exit requirement.

Proposal 7: RAN4 to discuss the requirement about exit the LR based on the LP-RSRP/LP-RSRQ simulation results.

Proposal 8: RAN4 to discuss the requirement about entry the LR based on both MR SSB RSRP/RSRQ and whether includes LR evaluation.

Proposal 9: RAN4 to follow RAN1’s LP-RSRP and LP-RSRQ definition to evaluate the LP-SS performance.

Proposal 10: RAN4 to discuss the PSS/SSS periodicity to evaluate the LP-WUR exiting condition in LR.

Proposal 11: RAN4 should consider both the MR relaxation power saving gain and LR’s performance to define MR relaxation scaling factor.

Proposal 12: RAN4 to evaluate how long the MR wake up is needed to assist LR measurement.

Proposal 13: No dedicated accuracy requirement in the performance section are defined for LP-SS based measurement and reflect the accuracy performance as a margin in the IDLE/INACTIVE mode core requirement.

Proposal 14: LP-WUS CONNECTED mode requirements is postoponed until more progress is achieved in other WGs.

- References

[1]. R4-2410296, “WF for RRM requirements for LP-WUS/WUR”, vivo

[2]. R2-2403551, “RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE”, Ericsson