3GPP TSG RAN WG4 #112	R4-2412506

Maastricht, Netherlands, Aug. 19th – Aug. 23rd, 2024

Agenda Item:	8.22.4.1

Source: 	Ericsson

Title:	Simulation on LP-WUS RRM measurement

Document for:	Discussion

# Introduction

In last RAN4 meeting, RAN4 had discussed the LP-WUS/WUR WI and made the following agreement [1]. In this contribution, we share our views on simulation assumption on LP-SS RRM measurement.

Issue 1-4-2: LP-SS periodicity for evaluation Agreement: - As starting point for RAN4 simulation purpose, uses 320 ms as the LP-SS periodicity in IDLE/Inactive mode, and other values based on RAN1/2 discussion are not precluded. - Not consider the MR DRX configuration in the simulation. Issue 1-4-5: Simulation assumptions Agreement: First start the simulation for FR1, and FFS for FR2 pending on the RF progress. Recommendations: Companies are encouraged to provide simulation assumptions based on framework in the appendix. FFS on topics listed in [1] for sub-topic 1-4

- Simulation

RAN4 needs to agree the simulation assumption to evaluate the LP-SS RSRP/RSRQ measurement metrics similar as legacy NR SSB measurement. Some initial simulation results for LP-SS based RSRP/RSRQ measurement are shown in Figure 1 and Figure 2 below. Based on the simulation results, it is clearly observed that the LP-SS measurements performance will be degraded once the interference signal is large.

Observation 1. LP-SS measurement performance has significant decreased when the interference signal is higher than the serving signal.

Figure 1. RSRP and RSRQ accuracy based OOK-1 LP-SS and inter-cell NR interference

Figure 2. RSRP and RSRQ accuracy based OOK-4 LP-SS and inter-cell OOK-4 interference

- Simulation assumption

Proposal 1: RAN4 to agree the LP-SS simulation assumption.

Table 1: General parameters

| Simulation parameters | Comments/values |
| --- | --- |
| Carrier frequency for Cell 1 and Cell 2 | FR1: 2.6 GHz/700MHz |
| System bandwidth | 20 MHz; |
| Prior knowledge of Cell 1 / Cell 2 by the UE | Yes / No |
| DRX | No |
| BS transmit antennas for LP-SS blocks | 1 tx |
| UE receive antennas | 1 rx |
| Data and control channel subcarrier spacing | OFDM based: The same as SS block subcarrier spacing OOK based: the same as one of the SCS(s) used for other NR transmissions in the same CP-OFDM symbol |
| Measurement period (in number of measurement samples) | - |
| Subcarrier spacing | 30 kHz; |
| Number of LP-SS blocks per SS burst set, K | 1 |
| LP-SS/SSB burst periodicity | 320 ms OFDM based: 320 ms |
| Number of transmit antenna ports | 1 (the same port for NR-SSS, NR-PSS, NR-PBCH) |
| LP-SS block BW | 132 subcarriers for SCS=30kHz; |
| Actual LP-SS block transmissions | always transmitted |

Table 2: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| RF Channel number | - | Channel 1 | Channel 1 |
| PBCH and DMRS power offset with respect to NR-PSS, NR-SSS and LP-SS | dB | 0 | 0 |
| Data and control PSD relative to NR-PSS, NR-SSS and LP-SS | dB | 0 | 0 |
| RB Utilization | % | 100 | 100 |
| LP-SS |  | OOK-1; OOK-4 with M = 2,4; | when Cell 1 uses OOK-1, NR signal is used for Cell 2 when Cell 1 uses OOK-4, OOK-4 or NR signal is used for Cell 2 |
| LP-SS binary sequence pattern |  | OOK-4: [1010] | OOK-4: [0101] |
| Data Modulation | - | QPSK | QPSK |
| Slot length | - | 14 symbols | 14 symbols |
| CP Length | - | Normal | Normal |
| Frequency Offset relative to UE frequency reference | Hz | OFDM: 5 ppm OOK: 5 or 10 ppm | - |
| 1)Relative Delay of 1st Path (synchronous) | µs | 0 | CP/2 |
| 2)Relative Delay of 1st Path (asynchronous): Fixed delay | ms | 0 | [3] ms |
| SNR | dB | 3: 6: 9 | 3 |
| Es/IoT (calculated from SNR) | dB | -1.7:1.2:4.2 |  |
| Propagation conditions | - | AWGN, TDL-C 300ns |  |
| UE speed |  | 3 km/h |  |
| NOTE: the companies are encouraged to state channel model parameters together with the results, the parameters are to be further discussed and aligned. |  |  |  |

- Summary

In this contribution, we have initially discussed the LP-WUS RRM measurement simulation assumption. Based on the discussions, we have made following proposals and observations:

Observation 1. LP-SS measurement performance has significant decreased when the interference signal is higher than the serving signal.

Proposal 1: RAN4 to agree the LP-SS simulation assumption.

- Reference

[1]. R4-2410296, “WF for RRM requirements for LP-WUS/WUR”, vivo