3GPP TSG RAN WG4 #112	R4-2412507

Maastricht, Netherlands, Aug. 19th – Aug. 23rd, 2024

Agenda Item:	8.21.2

Source: 	Ericsson

Title:	Initial discussion on Rel-19 NES

Document for:	Discussion

# Introduction

In RP #102, enhancements of network energy savings for NR[1] are introduced. In last RP meeting, the WID is updated with the following objectives.

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] - Triggering method by uplink wake-up-signal using an existing signal/channel. - Wake-up-signal configuration provisioning to UE - Note: No modification of SSB will be discussed under this objective - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. - Checkpoint for normative work in RAN#105 - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

In this contribution, we present our initial view on the Rel-19 NES operation from RAN4 perspective.

- On-demand SSB(OD-SSB)

General

In RAN4 RRM, from Rel-15 to Rel-18, multiple requirements about SCell activation delay are defined. In our understanding, there is no restriction to apply OD-SSB to all these kinds of SCell activation from RAN1.

| Release | Scell activation type |
| --- | --- |
| Rel-15 | Single Scell activation |
| Rel-16 | Multiple Scell activation |
| Rel-16 | Direct Scell activation |
| Rel-17 | PUCCH Scell activation |
| Rel-17 | Fast Scell activation |
| Rel-18 | L3 reporting enh Scell activation |
| Rel-18 | SSB-less inter-band co-located Scell activation |

Considering the workload and complexity, we propose RAN4 to first study the OD-SSB impact from the Rel-15 single SCell activation. At the same time, we understood that applying of OD-SSB to multiple SCell activation and direct SCell activation are two valid use cases. For example, NW can configure multiple OD-SSBs to multiple SCells to identify the qualified SCells to be activated. Furthermore, RRC-based OD-SSB indication is agreed in RAN1 which can be used together with direct SCell activation by RRC. Thus, in our opinion, RAN4 can further study whether and how to define other type of SCell activation requirement in phase 2 which can start on Feb. 2025.

RAN1 #117 Agreement - For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell. - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell. - This DCI signaling does not provide SCell activation/deactivation. - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc. - FFS: Scenarios where the above signalings are applicable

Proposal 1: RAN4 to study OD-SSB impact based on Rel-15 single Scell activation requirement as a start point.

Proposal 2: RAN4 to study whether and how to define requirement for other type of SCell activation in phase 2 after baseline case (Rel-15 single SCell activation) is completed, such as multiple SCells activation, direct SCell activation etc.

- Note: Phase 2 will start from RAN4 #114 meeting (Feb., 2025)

In last RAN1 meetings, the following agreements were made. In our understanding, currently, both case 1 and case 2 defined in RAN1 are valid scenarios based on the agreements. RAN4 needs to study the requirement about OD-SSB based SCell activation for both cases.

RAN1 #116 Agreement Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study: - Case #1: No always-on SSB on the cell - Case #2: Always-on SSB is periodically transmitted on the cell - FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted. - FFS: Which scenario the above applies for

In RAN1 #116 and #116bis meeting, the scenarios #2 and #2A are agreed for OD-SSB indication. The OD-SSB can be used to trigger deactivated SCell measurement before the SCell activation command. In this case, the SCell can be a known Scell during SCell activation. On the contrary, the OD-SSB can be triggered together with SCell activation. In this case, UE will perform SCell activation as an unknown Scell.

RAN1 #116 Agreement For the following identified scenarios for on-demand SSB SCell operation, focus future RAN1 discussion to down-select (both may be selected) between the two scenarios. - Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321) - Scenario #3: After UE receives SCell activation command (e.g., as defined in TS 38.321) - This does not preclude SCell for which activation is completed - FFS: The case where SCell activation is completed FFS: Application timing between NW triggering message and on demand SSB transmission RAN1 #116bis Agreement For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed.

Proposal 3: RAN4 to study OD-SSB SCell activation for both Case 1 and Case 2 as follow.

- Case 1-1: No always-on SSB on the known cell

- Case 1-2: No always-on SSB on the unknown cell

- Case 2-1: Always-on SSB periodically transmitted on the known cell

- Case 2-2: Always-on SSB periodically transmitted on the unknown cell

Proposal 4: RAN4 to study OD-SSB based deactivated Scell measurement and Scell activation requirement first.

Typical OD-SSB based SCell scenario

From a SCell addition configuration to the SCell deactivated, a typical OD-SSB activation with SCell activation procedure includes five possible phases which is shown as follow.

Figure 1. OD-SSB based known SCell activation(Case 1)

In our understanding, NW can stop transmitting the OD-SSB and achieve the power saving gain during the phase 1. During phase 2, NW indicates the OD-SSB activation for deactivated SCell measurement. After NW receiving the L3 MR, NW can decide whether to stop the OD-SSB transmission in phase 3. When NW indicates both OD-SSB transmission and SCell activation, UE performs SCell activation procedure in phase 4. After UE finishing the SCell activation and transmitting the valid CQI report, UE will stay in an active serving cell as phase 5.

Phase 2: Deactivated SCell measurement

To maximize the NES gain, NW is expected to extend the duration of phase 1 and phase 3 as much as possible. When NW indicates the OD-SSB transmission in phase 2, UE shall follow the OD-SSB configuration as expected by the NW. In other words, the OD-SSB transmission in phase 2 shall be limited to a shorter duration as possible. At the same time, to guarantee the SCell activation as a known cell, the OD-SSB transmission duration should be longer than the required deactivated SCell measurement period.

Observation 1: To guarantee the known condition, OD-SSB transmission time duration should be longer than deactivated Scell measurement period.

Observation 2: To maximize the NES gain, OD-SSB transmission time duration in phase 2 shall be limited to a shorter duration.

Proposal 5: In Case 1, RAN4 to discuss UE’s measurement behaviour during the deactivated SCell stage to maximize the NES gain.

Proposal 6: In Case 1, UE is assumed to perform deactivated SCell measurement based on OD-SSB when NW indicates the OD-SSB transmission during the deactivated SCell stage.

Phase 3: OD-SSB transmission muting after L3 MR

When multiple OD-SSB transmission is supported, NW can either configure a single OD-SSB indication before SCell activation or configure multiple OD-SSB transmission/detransmission. In other words, considering whether phase 3 is existing before the SCell activation, there are two possible scenarios:

- Scenario 1: Single OD-SSB indication for both deactivated SCell measurement and SCell activation

- Scenario 2: Two OD-SSB indications for OD-SSB transmission in deactivated SCell measurement and SCell activation separately

Proposal 7: RAN4 to support the following scenarios for OD-SSB based known SCell activation.

- Scenario 1: Single OD-SSB indication for OD-SSB transmission in both deactivated SCell measurement and SCell activation

- Scenario 2: Two OD-SSB indications for OD-SSB transmission in deactivated SCell measurement and SCell activation separately

Phase 4: SCell activation

In Case 1-1, based on current known condition in SCell activation, the UE needs to send a valid L3 measurement report (MR) for the SCell being activated. Thus, during the deactivated SCell, UE needs to perform deactivated SCell measurement. After NW receiving the L3 MR, the SCell can be believed as a known SCell. When NW indicates the SCell activation command, UE is assumed to perform SCell activation based on known SCell requirement. As agreed in RAN1, OD-SSB transmission can be before or together with SCell activation. We understand that UE can follow the legacy Rel-15 SCell activation requirement in these two scenarios if the first OD-SSB transmission time instance A(will be defined in RAN1) is no longer than the TX defined in legacy SCell activation. RAN4 can study the impact if OD-SSB transmission is after SCell activation command once RAN1 agreed such scenario.

Proposal 8: In Case 1, when OD-SSB transmission is before/together with the SCell activation command, UE follows the legacy Rel-15 known SCell activation requirement if the known cell condition is met.

- FFS: the OD-SSB transmission time instance A

Case 1-2: OD-SSB unknown SCell activation

In Case 1-2, the SCell is an unknown SCell which means no L3 MR is transmitted to NW. Thus, the following two sub-scenarios are both possible.

- OD-SSB transmission at the same time as SCell activation command

- OD-SSB is transmitted before the SCell activation command but UE hasn’t finished the deactivated SCell measurement or L3 MR.

However, in our understanding, UE is assumed to follow the same unknown SCell activation delay requirement regardless the OD-SSB transmission scenarios.

Proposal 9: In Case 1, when OD-SSB transmission is before/together with the SCell activation command, UE follows the legacy Rel-15 unknown SCell activation requirement once the unknown cell condition is met.

- FFS: the OD-SSB transmission time instance A

Case 2: SCell procedure

In Case 2, NW transmits the legacy always-on SSB together with the OD-SSB as follows: (Green SSB is legacy always-on SSB, Orange SSB is the OD-SSB). Before NW indicating the OD-SSB transmission, UE is assumed to use legacy always-on SSB to perform deactivated SCell measurement. UE will switch to use OD-SSB to perform deactivated SCell measurement once UE receiving the OD-SSB transmission indication. RAN4 can further discuss UE’s behaviour and requirement for deactivated SCell measurement and SCell activation with the OD-SSB transmission.

Figure 2. OD-SSB based known SCell activation(Case 2)

Observation 3: OD-SSB and legacy SSB transmission can have different configurations, such as SMTC periodicities.

Proposal 10: In Case 2, the UE is assumed to switch to OD-SSB to perform measurement/Scell activation when OD-SSB transmission is indicated.

Proposal 11: In Case 2, the UE is assumed to perform measurement based on legacy SSB before UE receiving OD-SSB transmission indicated during deactivated Scell stage.

# On-demand SIB1(OD-SIB1)

The second item in Rel-19 NES is about OD-SIB1 in IDLE/INACTIVE mode. It is clearly specified in the WID that only RAN 1/2/3 WG are involved in this topic’s discussion. Thus, we propose to deprioritize the OD-SIB1 requirement discussion unless any RAN4 RRM impact is identifiedin the group.

Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] - Triggering method by uplink wake-up-signal using an existing signal/channel. - Wake-up-signal configuration provisioning to UE - Note: No modification of SSB will be discussed under this objective - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. Checkpoint for normative work in RAN#105

Proposal 12: RAN4 to deprioritize the OD-SIB1 requirement discussion unless any RRM impact is identified.

# Adaptation of common signal/channel transmissions (CS/CC adaptation)

There are several sub-topics in this item, including SSB adaptation, PRACH adaptation and paging occasions adapatation.

Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

Paging Occasion/RACH adaptation

In our understanding, the RRM impact of Paging Occasion and RACH adaptation introduced in RAN1 is unclear in current stage. In the phase 1, RAN4 can wait RAN1’s further progress on Paging Occasion/RACH adaptation.

Proposal 13: RAN4 to wait RAN1’s further progress on Paging Occasion/RACH adaptation.

SSB adaptation

In current NR, SSBs are always on-transmissions, and their periodicities can only be changed semi-statically. Legacy UE performing initial access on a cell may assume a default SSB periodicity of 20ms. While most other transmission activities (e.g. SIB1, etc) can be minimized (e.g. periodicity of 160ms or larger), SSB transmissions on such cells must continue every 20ms even when the cell has no/light traffic.

The TR 38.864 (in [2]) captures the evaluation results for adaptation of SSB periodicity (and other common signals/channel) as part of technique A-1. It shows that significant NW energy savings gain be obtained by SSB periodicity adaptation, such as from 20ms to 40 ms in below Figure 2.

- Observation 4: Dynamic adaptation of SSB periodicity provides additional network energy savings, e.g., 16% when SSB periodicity changes from 20ms to 40ms.

Figure 3. NW energy consumption when SSB periodicity is increased from 20ms to 40ms

In RAN1 #116 meeting, RAN1 agrees to apply such SSB adaptation in time domain to both PCell and SCell. Thus, RAN4 can define SSB adaptation requirement for both PCell and SCells.

RAN1 #116 Agreement For the adaptation mechanisms of SSB in time-domain, study further applicable scenarios and associated legacy UE impact/handling (if any) based on the following: - Applicability to UE in idle/inactive and/or connected mode - Applicability to PCell and/or SCell(s)

Proposal 14: RAN4 to define SSB adaptation requirement for both PCell and/or SCell(s).

In RAN1 #116bis meeting, NW controlled SSB adaptation is agreed. The UE triggered SSB adaptation is FFS. Thus, we propose RAN4 to focus on NW controlled SSB adaptation firstly.

RAN1 #116 Agreement For the adaptation mechanisms of SSB in time-domain, study further following mechanisms: - Adaptation mechanism indicated or configured by gNB without UE trigger - Adaptation triggered by UE (if any) FFS: Details of associated signaling/indication/configuration RAN1 #116bis Agreement For indication of adaptation of SSB in time-domain, - Support at least SSB adaptation provided by gNB without UE trigger

Proposal 15: RAN4 to focus on NW controlled SSB adaptation firstly.

In RAN1 #116bis, it is agreed to apply SSB adaptation in time-domain in CONNECTED mode PCell and SCells firstly. Considering legacy UE impact, whether to apply the SSB adaptation in IDLE/INACTIVE mode is FFS. Thus, from our perspective, RAN4 can prioritize the discussion on SSB adaptation requirement in CONNECTED mode for L1/L3 measurement first.

RAN1 #116bis Agreement Adaptation mechanism(s) of SSB in time-domain is supported at least for one of the following scenario(s): - For cell with both legacy UEs and Rel-19 NES-capable UEs - Rel-19 NES-capable UE’s PCell (Connected mode) - Study from the following options: - Option A1: adaptation for CD-SSB - Option A2: adaptation for SSB that is not CD-SSB - Option A3: adaptation for SSB not on sync raster - Rel-19 NES-capable UE’s SCell - Study from the following options: - Option B1: adaptation for CD-SSB - Option B2: adaptation for SSB that is not CD-SSB - Option B3: adaptation for SSB not on sync raster - FFS: Rel-19 NES-capable UE in idle/inactive mode - Note: Impact to idle/inactive UEs shall be minimized

Proposal 16: RAN4 to prioritize the study of SSB adaptation requirement in CONNECTED mode, such as L1/L3 measurement impact.

- Summary

In this contribution, we have discussed the MUSIM gaps requirements. Based on the discussions, we have made following proposals and observations:

Observation 1: To guarantee the known condition, OD-SSB transmission time duration should be longer than deactivated Scell measurement period.

Observation 2: To maximize the NES gain, OD-SSB transmission time duration in phase 2 shall be limited to a shorter duration.

Observation 3: OD-SSB and legacy SSB transmission can have different configurations, such as SMTC periodicities.

Observation 4: Dynamic adaptation of SSB periodicity provides additional network energy savings, e.g., 16% when SSB periodicity changes from 20ms to 40ms.

Proposal 1: RAN4 to study OD-SSB impact based on Rel-15 single Scell activation requirement as a start point.

Proposal 2: RAN4 to study whether and how to define requirement for other type of SCell activation in phase 2 after baseline case (Rel-15 single SCell activation) is completed, such as multiple SCells activation, direct SCell activation etc.

- Note: Phase 2 will start from RAN4 #114 meeting (Feb., 2025)

Proposal 3: RAN4 to study OD-SSB SCell activation for both Case #1 and Case #2 as follow.

Proposal 4: RAN4 to study OD-SSB based deactivated Scell measurement and Scell activation requirement first.

Proposal 5: In Case 1, RAN4 to discuss UE’s measurement behaviour during the deactivated SCell stage to maximize the NES gain.

Proposal 6: In Case 1, UE is assumed to perform deactivated SCell measurement based on OD-SSB when NW indicates the OD-SSB transmission during the deactivated SCell stage.

Proposal 7: RAN4 to support the following scenarios for OD-SSB based known SCell activation.

Proposal 8: In Case 1, when OD-SSB transmission is before/together with the SCell activation command, UE follows the legacy Rel-15 known SCell activation requirement if the known cell condition is met

- FFS: the OD-SSB transmission time instance A

Proposal 9: In Case 1, when OD-SSB transmission is before/together with the SCell activation command, UE follows the legacy Rel-15 unknown SCell activation requirement once the unknown cell condition is met.

Proposal 10: In Case 2, the UE is assumed to switch to OD-SSB to perform measurement/Scell activation when OD-SSB transmission is indicated.

Proposal 11: In Case 2, the UE is assumed to perform measurement based on legacy SSB before UE receiving OD-SSB transmission indicated during deactivated Scell stage.

- FFS: the OD-SSB transmission time instance A

Proposal 12: RAN4 to deprioritize the OD-SIB1 requirement discussion unless any RRM impact is identified.

Proposal 13: RAN4 to wait RAN1’s further progress on Paging Occasion/RACH adaptation.

Proposal 14: RAN4 to define SSB adaptation requirement for both PCell and/or SCell(s).

Proposal 15: RAN4 to focus on NW controlled SSB adaptation firstly.

Proposal 16: RAN4 to prioritize the study of SSB adaptation requirement in CONNECTED mode, such as L1/L3 measurement impact.

- References

- RP-234065, “Rel-19 WID: Enhancements of network energy savings for NR”, Ericsson, Apple

- RP-241650, “Rel-19 Revised WID: Enhancements of network energy savings for NR”, Ericsson, Apple

- 3GPP TR 38.864 V18.1.0, “Study on network energy savings for NR”