3GPP TSG RAN WG4 #112	R4-2412509

Maastricht, Netherlands, Aug. 19th – Aug. 23rd, 2024

Agenda Item:	4.4

Source:	Ericsson

Title:	(NR_MG_enh-Core) Remaining issues on R17 NCSG

Document for:	Discussion

# Introduction

The Rel-17 and Rel-18 MG enh. WI are already closed. However, in RAN4 #111 meeting, some remaining issues about NCSG were identified in WF[1].

In this paper, we will discuss these remaining issues of Rel-17 NCSG.

# The remaining issues of Rel-17 NCSG

In RAN4 #111 meeting, some remaining issues for NCSG were identified as follow.

Issue 1: Rel-17 NCSG clarification - FFS whether and how to update VIL requirements in case of per-UE NCSG pattern #12-23 is configured - FFS whether and how to distinguish NCSG pattern #0-11 and NCSG pattern #12-23. - Option 1: clarify VIL in Table 9.1.9.3-1: NCSG Configurations supported by the UE. - Option 2: modify VIL specific to NCSG patterns, e.g. Table 9.1.9-1 for NCSG pattern #0-11 and Table 9.1.9-2 for NCSG pattern #12-23.

In Rel-15, the measurement gap patterns were defined based on MGL and MGRP. Due to different SMTC and RF retuning time, MGL will be different in FR1 and FR2. However, in Rel-17 NCSG, the NCSG patterns were defined based on ML and VIRP. The ML which equals SMTC duration is the same for FR1 and FR2 patterns, and the difference is the VIL in FR1 and FR2. Thus, the NCSG patterns look the same for pattern #0-11 and pattern #12-23. However, when UE reports to support or NW configures one specific pattern in #0-11 or #12-23, both NW and UE can understand that the VIL, VIRP and ML for the pattern.

Observation 1: NCSG patterns’ parameters for pattern #0-11 and pattern #12-23 are the same in RAN4 spec, but they are clear from RAN2 signalling.

Proposal 1: In Rel-17, RAN4 to update the NCSG patterns with a clarification.

# Summary and Conclusion

In the contribution, we discuss the remaining issues for Rel-17 MG enh with the following proposals:

Observation 1: NCSG patterns’ parameters for pattern #0-11 and pattern #12-23 are the same in RAN4 spec, but they are clear from RAN2 signalling.

Proposal 1: In Rel-17, RAN4 to update the NCSG patterns with a clarification.

# References

- R4-2410326, “WF on clarification of R17 NCSG pattern”, OPPO