3GPP TSG-RAN WG4 Meeting #112 	R4-2412515

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	vivo

Title: 	(NR_UE_pow_sav_enh-Core) Discussion on maintenance of RRM requirements for RLM and BFD relaxation

Agenda Item:	4.5

Document for:	Discussion

Introduction

In RAN4 #111, an LS [1] is received from RAN2 and a CR [2] is discussed. In [2] the following revision is proposed.

In this contribution, we provide our views on the maintenance issue raised by [2].

Discussion

In RAN4 #111, an LS form RAN2 is received ant the following agreement in RAN2 is enclosed in the LS.

RAN4 captured in TS 38.133 that the UE is not allowed to perform RLM and/or BFD relaxation when the configured DRX cycle is larger than 80 ms. Furthermore TS 38.133 is agnostic about the configuration of both a short and long DRX cycle. For the case when short DRX cycle is configured, i.e. both a short and long DRX cycle are configured, RAN2 captured the following in TS 38.331: When the short DRX cycle is smaller or equal to 80 ms and the long DRX cycle is larger than 80 ms then the UE is allowed to perform RLM and/or BFD relaxation while drx-ShortCycleTimer is running and the relaxed measurement criteria are met.

As stated in the RAN2 LS, RAN4 RRM requirements are agnostic about the configuration of short or long DRX cycle. Taken a further look in TS 38.321, the following about short DRX is stated.

1> if drx-InactivityTimer for a DRX group expires: 2> if the Short DRX cycle is configured: 3> start or restart drx-ShortCycleTimer for this DRX group in the first symbol after the expiry of drx-InactivityTimer; 3> use the Short DRX cycle for this DRX group. 2> else: 3> use the Long DRX cycle for this DRX group. 1> if a DRX Command MAC CE indicated by PDCCH addressed to C-RNTI or CS-RNTI, or by a configured downlink assignment for unicast transmission is received: 2> if the Short DRX cycle is configured: 3> start or restart drx-ShortCycleTimer for each DRX group in the first symbol after the end of DRX Command MAC CE reception; 3> use the Short DRX cycle for each DRX group. 2> else: 3> use the Long DRX cycle for each DRX group. 1> if drx-ShortCycleTimer for a DRX group expires: 2> use the Long DRX cycle for this DRX group. 1> if a Long DRX Command MAC CE is received: 2> stop drx-ShortCycleTimer for each DRX group; 2> use the Long DRX cycle for each DRX group. 1> if the Short DRX cycle is used for a DRX group, and [(SFN × 10) + subframe number] modulo (drx-ShortCycle) = (drx-StartOffset) modulo (drx-ShortCycle): 2> start drx-onDurationTimer for this DRX group after drx-SlotOffset from sthe beginning of the subframe.

According to above specified UE behaviour in TS 38.321, in most cases, UE uses short DRX only once, when drx-InactivityTimer expires. On the other hand, R17 RLM/BFD relaxation mainly focuses on the scenarios where UE has quite rare DL/UL data, but quite sensitive on the latency of DL data. In such case UE is not expected to be scheduled every time during each DRX. In other word, UE would not be continuously using short DRX cycle for a long duration in case of R17 UE power saving.

Observation 1   Short DRX is only used once when there is UL/DL data during the on-duration of each DRX cycle. However, R17 UE RLM/BFD relaxation mainly focuses on the scenarios where in most DRX cycles there is no UL/DL data. UE shall not be expected in the continuous short DRX cycles for R17 RLM/BFD relaxation.

Therefore, considering the concerned scenario for R17 RLM/BFD relaxation, normally short DRX is only used for quite a short period, i.e. during the transition period between “small data” and “no data”. TS 38.133 is already clear on the requirements for RLM/BFD detection during transition periods, as cited below.

### 8.1.4 Minimum requirement at transitions When the UE transitions between DRX and no DRX or when DRX cycle periodicity changes, for each RLM-RS resource, for a duration of time equal to the evaluation period corresponding to the second mode after the transition occurs, the UE shall use an evaluation period that is no less than the minimum of evaluation period corresponding to the first mode and the second mode. Subsequent to this duration, the UE shall use an evaluation period corresponding to the second mode for each RLM-RS resource. This requirement shall be applied to both out-of-sync evaluation and in-sync evaluation of the monitored cell. When the UE transitions from a first configuration of RLM resources to a second configuration of RLM resources that is different from the first configuration, for each RLM resource present in the second configuration, for a duration of time equal to the evaluation period corresponding to the second configuration after the transition occurs, the UE shall use an evaluation period that is no less than the minimum of evaluation periods corresponding to the first configuration and the second configuration. Subsequent to this duration, the UE shall use an evaluation period corresponding to the second configuration for each RLM resource present in the second configuration. This requirement shall be applied to both out-of-sync evaluation and in-sync evaluation of the monitored cell. When the UE transitions from a first configuration of active TCI state of the CORESET to a second configuration of active TCI state of the CORESET, for each CSI-RS for RLM present in the second configuration, the UE shall use an evaluation period corresponding to the second configuration from the time of transition. This requirement shall be applied to both out-of-sync evaluation and in-sync evaluation of the monitored cell.

Therefore, most likely UE would not even need to enter the short-DRX-based RLM/BFD mode the since it is only considered for one or two DRX cycles. UE implementation based on the long DRX cycle may still meet the requirements.

Observation 2  Considering the target scenario for R17 RLM/BFD relaxation are the cases when UE has no UL/DL data in most of the DRX cycles, short DRX shall only be used during transition period, and it is already clear in TS 38.133 that UE does not need to meet RRM requirements during transitions for RLM/BFD.

As written in RAN2 LS, TS 38.133 is always agnostic on DRX cycles. Although “configured DRX cycle” might be ambiguous about whether short or long DRX it is referring to, it is actually allowing UE to considering the longer one between them, so as to simplify UE implementation, while no negative impact on network side would be assumed. In case UE has continuous data in consecutive DRX cycles, it is up to gNB implementation to enable or disable RLM/BFD relaxation feature via RRC signalling, since those are not the scenarios where UE power saving gain can be achieved. Given RAN2 spec is already clear, no further RAN4 spec impact is needed.

Observation 3  In case UE has continuous data in consecutive DRX cycles, it is up to gNB implementation to enable or disable RLM/BFD relaxation feature via RRC signaling.

Proposal 1  For R17 RLM/BFD relaxation, no further spec impact is needed.

Conclusions

Based on above analysis, we have following observations and proposals.

Observation 1   Short DRX is only used once when there is UL/DL data during the on-duration of each DRX cycle. However, R17 UE RLM/BFD relaxation mainly focuses on the scenarios where in most DRX cycles there is no UL/DL data. UE shall not be expected in the continuous short DRX cycles for R17 RLM/BFD relaxation.

Observation 2  Considering the target scenario for R17 RLM/BFD relaxation are the cases when UE has no UL/DL data in most of the DRX cycles, short DRX shall only be used during transition period, and it is already clear in TS 38.133 that UE does not need to meet RRM requirements during transitions for RLM/BFD.

Observation 3  In case UE has continuous data in consecutive DRX cycles, it is up to gNB implementation to enable or disable RLM/BFD relaxation feature via RRC signaling.

Proposal 1  For R17 RLM/BFD relaxation, no further spec impact is needed.

- References

[1]  R4-2407012  LS on RLM and BFD relaxation when short DRX is configured, RAN2.  RAN4 #111

[2]  R4-2410220  [NR_UE_pow_sav_enh-Core] Clarification to RLM/BFD relaxation with short DRX, Nokia. RAN4 #111