3GPP TSG-RAN WG4 Meeting #112 	R4-2412517

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	vivo

Title: 	Discussion on maintenance of RRM core requirements for R18 NR_Mob_enh2

Agenda Item:	5.21.1

Document for:	Discussion

Introduction

In RAN4 #111, the WF and Big CR [1], [2] are approved.

Based on all above information, we provide our views on maintenance of LTM core requirements.

Discussion

<On relations between L1 measurements configured by LTM-CSI-ResourceConfig and CSI-ResourceConfig>

Based on latest outcome of RAN1/RAN2 discussion, the LTM L1 measurement configuration is one separate CSI measurement configuration. For the LTM CSI reporting, based on TS 38.331, it is impossible to configure legacy L1 reporting and LTM L1 reporting in the same CSI-ReportConfig, as the CSI reporting for LTM is configured in a separate IE. RAN1 has also specified the priority rule in TS 38.214:

TS 38.214 v18.2.0 (Agreed by RAN1 in 02/2024) A CSI report configured with LTM-CSI-ReportConfig has a higher priority over all CSI report(s) configured with CSI-ReportConfig irrespective of PriiCSIy,k,c,s value in case of collision with CSI report(s) configured with CSI-ReportConfig.

Therefore, normally, to ensure CSI measurement and reporting performance of both legacy L1-RSRP/L1-SINR/CQI/PMI/RI/CRI reporting, and LTM L1 reporting, it is nature that gNB configures/schedules LTM CSI report and legacy CSI report in different slots. In contrast, for R17 ICBM L1 measurement on the additional cell, RRC configuration framework shares the legacy CSI configuration framework.

Observation 1  RRC configurations of LTM L1 measurement and reporting are separate from legacy L1 measurement and reporting.

Observation 2  RAN1 has agreed that LTM CSI reports have higher priority than all legacy CSI Reports configured under CSI-ReportConfig in case of collision. Hence, it is nature that gNBs will prevent collisions in CSI-reporting occasions as far as possible so as to ensure performance of legacy CSI reporting.

As shown in observation 1 and 2, LTM L1 measurement and reporting is separate and has higher priority than legacy L1 measurement and reporting. Therefore generally, LTM CSI and legacy CSI are never reported in the same slot. From reporting perspective, they are totally separated.

Note that the following is also captured in RAN1 spec for CSI-ReportConfig.

### TS 38.214 ##### 5.2.1.4.3 L1-RSRP Reporting … If the higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to "Configured", the UE shall derive the channel measurements for computing L1-RSRP reported in uplink slot n based on only the most recent, no later than the CSI reference resource, occasion of SS/PBCH or NZP CSI-RS (defined in [4, TS 38.211]) associated with the CSI resource setting.

RAN4 has also agreed to restrict M = 1 for intra-f L1 measurement. However, there is no restriction on the CSI measurement occasion for LTM L1 measurement. Hence, the spec does not assume UE always perform LTM L1 measurement on serving cell at the exactly same occurrence when UE performs legacy L1 measurements.

Observation 3  For legacy serving cell L1 measurement, the CSI measurement occasion is restricted to the most recent occasion according to TS 38.214. However, such restriction does not apply to LTM L1 measurement.

Hence UE shall not be assumed to always perform LTM L1 measurement on serving cell and legacy L1 measurement on serving cell at exactly the same time.

Currently, delay requirements for LTM L1 measurement of serving cell is captured in clause 9.5.4.1. The current clause 9.5.4.1 of TS 38.133 is cited as follows.

TS 38.133 v18.6.0 9.5.4.1 SSB based L1-RSRP Reporting … When there is no intra-frequency L1-RSRP measurement on LTM neighbor cell(s) to measure, the value of TL1-RSRP_Measurement_Period_SSB is defined in Table 9.5.4.1-1 for FR1, the value of TL1-RSRP_Measurement_Period_SSB is defined in Table 9.5.4.1-2 for FR2 when highSpeedMeasFlagFR2-r17 is not configured and if groupBasedBeamReporting-r17 is not configured, and defined in Table 9.5.4.1-2A for FR2-1 when highSpeedMeasFlagFR2-r17 is not configured and if groupBasedBeamReporting-r17 is configured, and defined in Table 9.5.4.1-3 for FR2 power class 6 UE when highSpeedMeasFlagFR2-r17 is configured, where - M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise - The value of N in Table 9.5.4.1-2 is [reducedRxBeamNum] in FR2-1 for UE supporting [faster beam switching capability], according to the conditions described in clause 3.6.x, and activated with multi-Rx operation, otherwise the value of N in Table 9.5.4.1-2 is 8. The UE is activated with multi-Rx operation when the following conditions are met: - UE is configured with group-based beam reporting (GBBR) report - The value of N in Table 9.5.4.1-2A is 8. - The value of N in Table 9.5.4.1-3 is 8. … The value of TL1-RSRP_Measurement_Period_SSB is defined in Table 9.5.4.1-4 for UE incapable of [capability of measurement with RTD>CP] in FR1, Table 9.5.4.1-5 for UE capable of [capability of measurement with RTD>CP] in FR1, Table 9.5.4.1-6 for UE incapable of [capability of measurement with RTD>CP] in FR2 and Table 9.5.4.1-7 for UE capable of [capability of measurement with RTD>CP] in FR2 when there is intra-frequency L1-RSRP measurement on neighbor cell(s) to measure, where - M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise - N= 8 in Table 9.5.4.1-6 and Table 9.5.4.1-7. … Table 9.5.4.1-1: Measurement period TL1-RSRP_Measurement_Period_SSB for FR1 Configuration; TL1-RSRP_Measurement_Period_SSB (ms). non-DRX; max(TReport, ceil(M*P)*TSSB). DRX cycle ≤ 320ms; max(TReport, ceil(K *M*P)*max(TDRX,TSSB)). DRX cycle > 320ms; ceil(M*P)*TDRX. Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1 when TSSB ≤ 40 ms and highSpeedMeasFlag-r16 or highSpeedMeasCA-Scell-r17 are configured; otherwise K = 1.5. Note 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16. or measurementEnhancementCA-r17. … Table 9.5.4.1-4: Measurement period TL1-RSRP_Measurement_Period_SSB in FR1 for UE incapable of [capability of measurement with RTD>CP] Configuration; TL1-RSRP_Measurement_Period_SSB_intra (ms). non-DRX; max(TReport, ceil(M*P)*TSSB). DRX cycle ≤ 320ms; max(TReport, ceil(K *M*P)*max(TDRX,TSSB)). DRX cycle > 320ms; ceil(M*P)*TDRX. Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1.5. Note 3: The requirements apply if the actual RTD between serving cell and neighbor cell is not larger than CP.. Table 9.5.4.1-5: Measurement period TL1-RSRP_Measurement_Period_SSB in FR1 for UE capable of [capability of measurement with RTD>CP] Configuration; TL1-RSRP_Measurement_Period_SSB_intra (ms). non-DRX; max(TReport, ceil(M*P)*TSSB). DRX cycle ≤ 320ms; max(TReport, ceil(K *M*P)*max(TDRX,TSSB)). DRX cycle > 320ms; ceil(M*P)*TDRX. Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1.5..

Note that applicability rules of above requirements are captured in 9.5.1 and 9.5.2, cited as follows.

TS 38.133 v18.6.0 9.5 L1-RSRP measurements for Reporting 9.5.1 Introduction … The UE shall be able to measure all CSI-RS resources and/or SSB resources of the nzp-CSI-RS-ResourceSet and/or csi-SSB-ResourceSet within the CSI-ResourceConfig settings configured for L1-RSRP for the active BWP, provided that the number of resources, including the number of SSB resources of the cell with PCI different from serving cell configured for L1-RSRP measurements in 9.13, does not exceed the UE capability indicated by beamManagementSSB-CSI-RS. If the UE supports bwpOperationMeasWithoutInterrupt-r18, [UE shall be able to measure the SSB reources of csi-SSB-ResourceSet within the CSI-ResourceConfig configured outside active BWP], provided that the SSB is within the configured UE-specific CBW. … 9.5.2 Requirements applicability The requirements in clause 9.5 apply, provided: - The CSI-RS or SSB or CSI-RS and SSB resources configured for L1-RSRP measurements are measurable. An SSB resource configured for L1-RSRP shall be considered measurable when for each relevant SSB the following conditions are met: - L1-RSRP related side conditions given in clauses 10.1.19.1 and 10.1.20.1 for FR1 and FR2, respectively, for a corresponding band, - SSB_RP and SSB Ês/Iot according to Annex B.2.4.1 for a corresponding band. A CSI-RS resource configured for L1-RSRP shall be considered measurable when for each relevant CSI-RS the following conditions are met: - L1-RSRP related side conditions given in clauses 10.1.19.2 and 10.1.20.2 for FR1 and FR2, respectively, for a corresponding band, - CSI-RS_RP and CSI-RS Ês/Iot according to Annex B.2.4.2 for a corresponding band. A CSI-RS and SSB resource configured for L1-RSRP shall be considered measurable when the measurable resource conditions are met for both CSI-RS resource and SSB resource. Requirements are defined for periodic, semi-persistent and aperiodic resources. In case the SSB resources configured for L1-RSRP measurements outside SMTCs are overlapped with the PDSCH/PDCCH that is associated to a PCI different from serving cell, L1-RSRP measurement performance degradation is expected. If groupBasedBeamReporting-r17 is configured, the requirements in clause 9.5 apply when all SSB resources of csi-SSB-ResourceSet or CSI-RS resources of NZP-CSI-RS-ResourceSet within the CSI-ResourceConfig settings are configured for L1-RSRP for the active BWP are from a serving cell, including a single PCell, PSCell, or SCell configured for L1-RSRP measurements, when the UE is configured with a single serving cell (PCell, PSCell, or SCell) in FR2-1. For a UE supports [two TAs and RTD > CP capability], the requirements apply when Rx timing difference is up to MRTD as specified in clause [7.X1.X]. For a UE which do not support [two TAs and RTD > CP capability], the requirements apply when Rx timing difference is less than CP.

Firstly, from UE measurement behaviour perspective, LTM L1 measurement is for mobility purpose, in which RTD>CP would be an important feature for UE to support. However, for legacy serving cell L1 measurement, including ICBM, RTD>CP is not that important because the scenarios are limited to the cases where legacy MIMO operation are applied. In R18 MIMO, RTD>CP is introduced for legacy serving cell L1 measurement only for the case of 2-TA, and the applicable condition and the UE feature granularity there, are different from R18 LTM feature for the RTD>CP L1 measurements. The existing requirements in 9.5 shows that, RRM requirements specified in 9.5 are applicable when CP < RTD < MRTD as specified in 7.6.8 of TS 38.133 if UE supports R18 MIMO capability rxTimingDiff-r18, no matter Multi-TRP transmission is configured or not. Mixing UE requirements under two similar but separate UE capabilities would cause confusion here. For example, considering different pre-requisite features, it is quite possible that UE only supports RTD > CP for MIMO 2-TA, but not support RTD>CP for LTM, and vice versa.

Observation 4  L1 measurement configured by LTM-CSI-ResourceConfig are performed for the RTD>CP scenario only if UE supports RAN4 FG39-1. However, requirements specified in 9.5.4.1 are applicable for the RTD>CP scenario if UE support rxTimingDiff-r18 defined in R18 MIMO WI. Actually rxTimingDiff-18 is only applicable to the L1 measurement configured by CSI-ResourceConfig.

Secondly, the max number of cell/layer/resources where UE can measure for the LTM L1 measurement are different from legacy L1 measurement, i.e. L1 measurement configured by CSI-ResourceConfig. The existing requirements in 9.5 shows that, RRM requirements specified in 9.5 are applicable if the total number of resources to be measured does not exceed R15 UE capability ‘beamManagementSSB-CSI-RS’. However, LTM L1 measurement capabilities are discussed and defined in RAN4 FG 39-3-1/2/3/4/5/6. These RAN4 FGs are not applied to legacy L1measurements. In other word, in case the total number of cells or resources exceed the number indicated by UE capability of LTM, it shall not impact legacy L1 measurement configured by CSI-ResourceConfig.

Observation 5  L1 measurement configured by CSI-ResourceConfig are performed in case the total number of resource to measure does not exceed beamManagementSSB-CSI-RS. If UE is configured with LTM L1 measurement and the total number of cells or resources exceed the number indicated by UE FG 39-3-1/2/3/4/5/6, it shall not impact the L1 measurement configured by CSI-ResourceConfig.

Thirdly, the L1 measurement configured by CSI-ResourceConfig shall be applicable to FR1 HST and FR2 HST not matter LTM is configured or not. However, the existing requirements in 9.5.4.1 shows that, if a UE is configured with LTM L1 measurements on neighbour cell, enhanced serving cell L1 measurement requirements for FR1 HST are no longer applicable even if UE supports and is configured with highSpeedMeasFlag-r16 and/or highSpeedMeasCA-Scell-r17. We do not think this is a necessary restriction.

Observation 6  RRM requirements for L1 measurement configured by CSI-ResourceConfig shall be applicable to FR1 HST and FR2 HST no matter whether LTM L1 measurement is configured or not.

With M = 1 for intra-f. L1 measurement, the typical UE behaviour based on typical RRC configuration, i.e. PUCCH based periodic reporting, is shown in Fig. 1.

Figure 1 L1 measurements of serving cell and LTM L1 measurements are performed on different OFDM symbols.

Fig. 1 could be one kind of implementation. It would be up to UE implementation whether LTM L1 measurement and legacy L1 measurement can be performed at the same time or using exactly the same UE behaviour or not, but for FR1 they could be performed simultaneously. For FR2, the situation would be similar. If different Rx beams of the UE are used for LTM L1 and legacy L1, respectively, the measurement on serving cell shall not be performed simultaneously. However, in RAN4 #108bis, companies discussed to perform legacy serving cell L1 measurement and LTM serving cell L1 measurement with the same Rx beam simultaneously. Agreements were achieved and sharing factor is defined. Actually, PL1_sharing can be used to capture the impact of LTM L1 measurement to impact to legacy serving cell L1 measurements, and it is not necessary to impact all the legacy features including requirement applicability.

Observation 7  For FR1 and FR2, there is no technical issue if LTM L1 measurement and legacy L1 measurement are performed simultaneously on serving cell SSBs.

Observation 8  For FR2, RAN4 has agreed to define sharing factor between all serving cell L1 measurements (including LTM L1 and legacy L1) and neighbour cell(s) LTM L1 measurement, considering different UE Rx beams could be used. It shall be understood as measurement relaxation to legacy L1 measurements if LTM L1 measurements on neighbour cell(s) are configured, and PL1_sharing would be enough to capture this impact.

Observation 9  It is not always necessary to assume UE are performing legacy L1 measurement and LTM L1 measurement simultaneously with exactly the same UE behaviour.

Hence, it is natural to separate the requirements defined for SSB based L1 RSRP reporting for serving cell (configured by CSI-ReportConfig) under 9.5.4.1, and the requirements for SSB-based L1-RSRP reporting for LTM (configured by LTM-CSI-ReportConfig), including serving cell under 9.14. or 9.15. When UE performs LTM L1 measurement,

Based on above analysis, we have the following proposals

Proposal 1  Capture all intra-frequency LTM L1 measurement requirements, including serving cell L1 measurement requirements, in 9.14.

Proposal 2  The impact to serving cell L1 measurement, which is configured by CSI-ResourceConfig, due to Rx beam sharing with neighbour cell LTM L1 measurement is captured by PL1_sharing in 9.5.4.1

The proposed change is provided in our CR [3].

<On L1 report for unmeasured candidate cells >

Based on TS 38.214 and TS 38.331, the number of cells reported in L1 reporting are configured by gNB. Hence, although it might not be a RAN4 RRM issue to discuss the reported value, it is possbile that gNB has configured LTM L1 resource in LTM-CSI-ResourceConfig-r18 but UE may not identify any SSB configured in the CandidateID+SSB_ID list in LTM-CSI-ResourceConfig-r18 based on the side condition for detectable cell/SSB in RRM specs.

Observation 10  Based on the SSB detectable condition defined in TS 38.133, it is possible for LTM L1 measurement that, the number of SSBs identified by UE in the configured list of candidate SSBs in LTM-CSI-ResourceConfig-r18, is less than the number of SSBs configured to report, i.e. nrOfReportedCells-r18 * nrOfReportedRS-PerCell-r18.

Therefore, we think it is necessary to define a rule for the L1 reporting of unmeasured cell/SSB in R18, since the physical size of L1 reporting is fixed, which is different from L3 reporting. This was discussed in last meeting and the following issue is left.

Issue 2-4-1: L1 report for unmeasured candidate cells <Way Forward >: Further discuss the following options: - Option 1 (vivo): - In L1-RSRP measurement report, for unmeasured candidate cells, UE sends invalid L1-RSRP in PUCCH if needed, i.e. the reported value corresponds to one of the invalid codepoints for L1-RSRP in Table 10.1.6.1-1 or DIFFRSRP_15 in Table 10.1.6.1-2. - In this case, the SSB or the cell it reflects remains unknown to the UE. - Option 2 (CTC): - In L1-RSRP measurement report, for unmeasured candidate cells, UE reports measured quantity value corresponding to any of the invalid codepoints in Table 10.1.6.1-1. - Option 3 (Nokia): RAN4 to send LS to RAN1 - Question: From RAN1 point of view, is it acceptable for the UE to report invalid values (from table 10.1.6.1-1 of TS 38.133) or value DIFFRSRP_15 (from table 10.1.6.1-2 of TS 38.133) for LTM candidate cells that are not measured by the UE? At least the following should be considered: - When none of the candidate cells are measured within the given periodicity, UE reports a 7 bit “Not valid” value. - DIFFRSRP_15 is reported when at least one LTM candidate cell was measured and at least one configured candidate cell was unmeasured. - The reported values for unmeasured cells do not meet any measurement requirements as they are unmeasured. - Option 4 (Ericsson, QC): - In L1-RSRP measurement report, for unmeasured candidate cells, UE reports DIFFRSRP_15 in Table 10.1.6.1-2.

We support option 1 as it is the simplest approach in R18. Any revisions to RAN1 rules for the L1 reporting would be too late in R18. Moreover, we think the proposal would also impact the known condition of a candidate cell and/or a candidate SSB/TCI. In case the reported resource ID defined in TS 38.212 is associated to the invalid codepoint, the cell/SSB/TCI it reflects cannot be regarded as known based on this reporting.

Proposal 3  In L1-RSRP measurement report, for unmeasured candidate cells, UE sends invalid L1-RSRP in PUCCH if needed, i.e. the reported value corresponds to one of the invalid codepoints for L1-RSRP in Table 10.1.6.1-1 or DIFFRSRP_15 in Table 10.1.6.1-2.

- In this case, the SSB or the cell it reflects remains unknown to the UE.

The impact of above proposal is reflected in our CR [3].

< On ‘time gap’ for TCI activation and PDCCH-ordered RACH>

Issue 1-3-2: The definition of Tfirst-SSB in early candidate cell’s TCI state activation delay for inter-frequency with gap for known TCI state case Ad hoc agreement < Agreement>: - In early candidate cell’s TCI state activation delay for known TCI state case: Tfirst-SSB is the time to the first SSB occasion overlapped with MGL after slot n + THARQ +3Nslotsubframe,µ + [2ms] if the SSB needs to be measured with MG. - FFS: this is only applicable to UE supporting inter-frequency L1 measurement with MG. - Same agreement applies to the definition of Tfirst-SSB in PDCCH order RACH delay requirement.

In our view, time-frequency tracking for the early TCI activation is actually L1 measurement behaviour. It is already agreed that UE can only perform L1 measurement within gap if UE supports the corresponding capability. In last meeting, RAN4 has agreed that L3 measurement and reporting can also be considered as one applicable condition for RRM requirements in FR1. Therefore, RRM requirements are applicable if the SSB of the target cell (for either PDCCH-order-RACH or early TCI activation) is within active BWP and L3 MR with beam index is reported. For the case SSB of target cell is not within active BWP, no matter L3 or L1 reporting has been done, UE needs to perform time-frequency tracking again on this SSB with similar behaviour as L1 measurement. UE not supporting L1 measurement with gaps may not do this. It has no relation on whether L1 measurement or L3 measurement has been used to trigger this.

Proposal 4  Confirm the FFS bullet:

- In early candidate cell’s TCI state activation delay for known TCI state case: For UE supporting inter-frequency L1 measurement with gap, Tfirst-SSB is the time to the first SSB occasion overlapped with MGL after slot n + THARQ +3Nslotsubframe,µ + [2ms] if the SSB needs to be measured with MG.

- In PDCCH ordered RACH delay, when additional time for T/F tracking is needed: For UE supporting inter-frequency L1 measurement with gap, TSSB is the time to the first SSB occasion overlapped with MGL after [2]ms and [1slot from] the end of the slot of the PDCCH, plus 2ms (SSB processing time), if the SSB needs to be measured with MG.

Conclusions

Based on above analysis, we have following observations and proposals.

Observation 1  RRC configurations of LTM L1 measurement and reporting are separate from legacy L1 measurement and reporting.

Observation 2  RAN1 has agreed that LTM CSI reports have higher priority than all legacy CSI Reports configured under CSI-ReportConfig in case of collision. Hence, it is nature that gNBs will prevent collisions in CSI-reporting occasions as far as possible so as to ensure performance of legacy CSI reporting.

Observation 3  For legacy serving cell L1 measurement, the CSI measurement occasion is restricted to the most recent occasion according to TS 38.214. However, such restriction does not apply to LTM L1 measurement.

Observation 4  L1 measurement configured by LTM-CSI-ResourceConfig are performed for the RTD>CP scenario only if UE supports RAN4 FG39-1. However, requirements specified in 9.5.4.1 are applicable for the RTD>CP scenario if UE support rxTimingDiff-r18 defined in R18 MIMO WI. Actually rxTimingDiff-18 is only applicable to the L1 measurement configured by CSI-ResourceConfig.

Observation 5  L1 measurement configured by CSI-ResourceConfig are performed in case the total number of resource to measure does not exceed beamManagementSSB-CSI-RS. If UE is configured with LTM L1 measurement and the total number of cells or resources exceed the number indicated by UE FG 39-3-1/2/3/4/5/6, it shall not impact the L1 measurement configured by CSI-ResourceConfig.

Observation 6  RRM requirements for L1 measurement configured by CSI-ResourceConfig shall be applicable to FR1 HST and FR2 HST no matter whether LTM L1 measurement is configured or not.

Observation 7  For FR1 and FR2, there is no technical issue if LTM L1 measurement and legacy L1 measurement are performed simultaneously on serving cell SSBs.

Observation 8  For FR2, RAN4 has agreed to define sharing factor between all serving cell L1 measurements (including LTM L1 and legacy L1) and neighbour cell(s) LTM L1 measurement, considering different UE Rx beams could be used. It shall be understood as measurement relaxation to legacy L1 measurements if LTM L1 measurements on neighbour cell(s) are configured, and PL1_sharing would be enough to capture this impact.

Observation 9  It is not always necessary to assume UE are performing legacy L1 measurement and LTM L1 measurement simultaneously with exactly the same UE behaviour.

Proposal 1  Capture all intra-frequency LTM L1 measurement requirements, including serving cell L1 measurement requirements, in 9.14.

Proposal 2  The impact to serving cell L1 measurement, which is configured by CSI-ResourceConfig, due to Rx beam sharing with neighbour cell LTM L1 measurement is captured by PL1_sharing in 9.5.4.1

Observation 10  Based on the SSB detectable condition defined in TS 38.133, it is possible for LTM L1 measurement that, the number of SSBs identified by UE in the configured list of candidate SSBs in LTM-CSI-ResourceConfig-r18, is less than the number of SSBs configured to report, i.e. nrOfReportedCells-r18 * nrOfReportedRS-PerCell-r18.

Proposal 3  In L1-RSRP measurement report, for unmeasured candidate cells, UE sends invalid L1-RSRP in PUCCH if needed, i.e. the reported value corresponds to one of the invalid codepoints for L1-RSRP in Table 10.1.6.1-1 or DIFFRSRP_15 in Table 10.1.6.1-2.

- In this case, the SSB or the cell it reflects remains unknown to the UE.

Proposal 4  Confirm the FFS bullet:

- In early candidate cell’s TCI state activation delay for known TCI state case: For UE supporting inter-frequency L1 measurement with gap, Tfirst-SSB is the time to the first SSB occasion overlapped with MGL after slot n + THARQ +3Nslotsubframe,µ + [2ms] if the SSB needs to be measured with MG.

- In PDCCH ordered RACH delay, when additional time for T/F tracking is needed: For UE supporting inter-frequency L1 measurement with gap, TSSB is the time to the first SSB occasion overlapped with MGL after [2]ms and [1slot from] the end of the slot of the PDCCH, plus 2ms (SSB processing time), if the SSB needs to be measured with MG.

- References

[1]  R4-2410302  WF on NR mobility enhancements, MediaTek.  RAN4 #111

[2]  R4-2410399  Big CR to TS 38.133 on core requirement maintenance for Further NR mobility enhancements, MediaTek, Apple.  RAN4 #111

[3]  R4-2412518  CR on R18 LTM RRM core requirements, vivo. RAN4 #112