3GPP TSG-RAN WG4 Meeting #112 	R4-2412519

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	vivo

Title: 	Discussion on maintenance of RRM test cases for R18 NR_Mob_enh2

Agenda Item:	5.21.2

Document for:	Discussion

Introduction

In RAN4 #111, the WF and Big CR [1], [2] are approved.

Based on all above information, we provide our views on maintenance of LTM Test cases.

Discussion

<On TCI related test case coverage and Test configurations>

In [2], the following is captured for LTM cell switch test cases including FR1 and FR2.

The test consists of 4 tests, and UE is required to pass one among Test 1A, Test 1B, Test 2A and Test 2B. - Test 1: for a UE supporting ltm-MAC-CE-JointTCI-r18 and/or ltm-MAC-CE-SeparateTCI-r18 - Test 1A: for a UE supporting ltm-MAC-CE-JointTCI-r18. - Test 1B: for a UE supporting ltm-MAC-CE-SeparateTCI-r18 and does not support ltm-MAC-CE-JointTCI-r18. - Test 2: for a UE not supporting ltm-MAC-CE-JointTCI-r18 and ltm-MAC-CE-SeparateTCI-r18 - Test 2A: for a UE supporting ltm-BeamIndicationJointTCI-r18. - Test 2B: for a UE supporting ltm-BeamIndicationSeparateTCI-r18 and does not support ltm-BeamIndicationJointTCI-r18.

As in RAN1 feature list, 45-3a and 45-4a are generally not correlated to each other. However, the R17 pre-requisite for 45-3a is FG 23-1-1, while 45-4a is FG 23-10-1. In [3], it is clearly state that FG 23-1-1 is the pre-requisite of FG 23-10-1.

Observation 1  Since R17, supporting joint TCI configuration is the pre-requisite for supporting separate TCI.

Another issue is for FR1, how joint TCI or separate TCI would be configured. Actually, there was no ever test cases in FR1 that refers to R17 joint/separate TCI configuration. In TS 38.214, we see the following restrictions for UL TCI.

TS 38.214 v18.1.0 6.1 UE procedure for transmitting the physical uplink shared channel … The UE can be configured with a list of up to 64 TCI-UL-State configurations within the higher layer parameter BWP-UplinkDedicated. Each TCI-UL-State configuration contains a parameter for configuring one reference signal, if applicable, for determining UL TX spatial filter for dynamic-grant and configured-grant based PUSCH and PUCCH resource in a CC, and SRS. … When the UE is configured dl-OrJointTCI-StateList or ul-TCI-StateList, the UE shall perform PUSCH transmission corresponding to a Type 1 configured grant or a Type 2 configured grant or a dynamic grant according to the spatial relation, if applicable, with a reference to the RS for determining UL Tx spatial filter. The RS is determined based on an RS configured with qcl-Type set to 'typeD' of the indicated TCI-State or an RS in the indicated TCI-UL-State. The reference RS in the indicated TCI-State can be a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info. …

In RAN1 spec, the reference RS configured in UL TCI would be effective only if it is applicable. Normally, the common understanding is that in FR2 the ‘QCL-D’ and UL TCI would be applicable, since the spatial Rx/Tx information, i.e. UE beam information, are necessary. For FR1, the meaning of UL TCI and QCL-D is ambiguous. Different UE implementation might have different interpretation. From test case design perspective, they should not be configured in FR1 test cases.

Observation 2  According to RAN1 spec, the meaning of configuring ‘QCL-D’ and UL TCI to UE would be ambiguous in FR1.

Note that ltm-MAC-CE-JointTCI-r18 and ltm-MAC-CE-SeparateTCI-r18 are per-band UE capabilities and normally UE report supporting of ltm-MAC-CE-SeparateTCI-r18 in a FR1 band does not expect network to configure UL TCI state. Hence allow test coverage to this case would be redundant.

Proposal 1  In RRM test case design for LTM, RAN4 avoid TCI configuration with ‘QCL-D’ or ‘UL TCI’ to UE in FR1, which means only DL or Joint TCI with QCL-A/C configuration, and pathloss RS configuration if necessary, shall be provided to UE in FR1, even if UE supports ltm-MAC-CE-SeparateTCI-r18.

Conclusions

Based on above analysis, we have following observations and proposals.

Observation 1  Since R17, supporting joint TCI configuration is the pre-requisite for supporting separate TCI.

Observation 2  According to RAN1 spec, the meaning of configuring ‘QCL-D’ and UL TCI to UE would be ambiguous in FR1.

Proposal 1  In RRM test case design for LTM, RAN4 avoid TCI configuration with ‘QCL-D’ or ‘UL TCI’ to UE in FR1, which means only DL or Joint TCI with QCL-A/C configuration, and pathloss RS configuration if necessary, shall be provided to UE in FR1, even if UE supports ltm-MAC-CE-SeparateTCI-r18.

- References

[1]  R4-2410302  WF on NR mobility enhancements, MediaTek.  RAN4 #111

[2]  R4-2410400  Big CR to TS 38.133 on performance requirements for Further NR mobility enhancements, MediaTek, Apple.  RAN4 #111

[3]  R4-2412520  CR on R18 LTM RRM test cases, vivo. RAN4 #112