3GPP TSG-RAN WG4 Meeting #112 	R4-2412521

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	vivo

Title: 	Discussion on test cases for R18 NES

Agenda Item:	5.29.2

Document for:	Discussion

Introduction

In RAN4 #111, the WF and big CR for NES test cases [1], [2] are approved.

Based on all above information, we provide our views on maintenance of R18 NES test cases.

Discussion

<On maintaining uplink timing in test case for cell DTX>

In RAN4 #110bis, the follow issue was discussed and the agreements are achieved as follows.

Issue 3-1-1: Whether to define TC for Cell DTX Agreement: - Define TC: SA event triggered reporting test without gap under UE DRX and Cell DTX. - A UE which fulfils the requirements in Cell DTX test case can skip the test case in SA event triggered reporting tests without gap under UE DRX.

The test configuration and test case are already agreed in [2].

For uplink timing, in Change #1 of [2], TAT is set to infinity. However, in Change #8 of [2], the following description is still provided.

A.3.x.1 Cell DTX Configuration 1: Cell DTX cycle = 640 ms and TAT = Infinity Table A.3.x.1-1: DTX.1: Cell DTX cycle = 640 ms and time alignment timer (TAT) = Infinity Field; Value. cellDTXDRX-onDurationTimer; 1 ms. cellDTXDRX-CycleStartOffset; 640 ms. cellDTXDRXconfigType-r18; dtx. cellDTXDRXactivationStatus; activated. Note: This Cell DTX configuration is applicable for NR serving cell. The Cell DTX cycle and time alignment timer parameters are specified in clause 6.3.2 in TS 38.331 [2]. … A.6.6.1.x.2 Test parameters … UE needs to be provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment. Furthermore, UE is allocated with PUSCH resource at every DRX cycle.

This could be an error in the test case description. However, setting TAT = infinity might not be good for UE to manage UE uplink timing. For UE who has valid L3 measurement result to report, but waiting for UL grants for the L3 MR, in Cell DTX inactive time this UE may only send SR or PRACH to request for the uplink resources. Note that in DRX.1 setting, 40ms DRX periodicity is used. However, cell DTX period in above table is 640ms. That means UE has much higher possibility in needing SR or PRACH for uplink scheduling than in legacy test case for SA event-triggered reporting.

UE needs to have precise enough uplink timing before sending SR or PRACH. Therefore, the description in A.6.6.1.x.2 as highlighted is still needed. At least UE shall be allocated with PUSCH at every cell DTX cycle, and the TAT is preferred to be set to a value larger than 640ms, e.g. 1280ms.

Proposal 1  For cell DTX test case, the TAT is set to 1280ms, and the UE shall be scheduled with PUSCH at every cell DTX cycle.

Moreover, it is also preferred to test UE behaviour with a 40ms Cell DTX cycle. As analysed above, 640ms would be too long and may have big impact on UE reporting behaviour.

Proposal 2  Test 40ms Cell DTX periodicity instead of 640ms.

Conclusions

Based on above analysis, we have following proposals.

Proposal 1  For cell DTX test case, the TAT is set to 1280ms, and the UE shall be scheduled with PUSCH at every cell DTX cycle.

Proposal 2  Test 40ms Cell DTX periodicity instead of 640ms.

- References

[1]  R4-2410269	WF on RRM requirements for NR network energy saving, Huawei, HiSilicon.  RAN4 #111

[2]  R4-2410435	Big CR to TS 38.133 on performance requirements for NR Network energy saving, Huawei, HiSilicon.  RAN4 #111