3GPP TSG-RAN WG4 Meeting #112 	R4-2412523

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	vivo

Title: 	Discussion on RRM requirement impacts for R19 NR MIMO Phase 5

Agenda Item:	8.18.3

Document for:	Discussion

Introduction

In RAN #102 and #103, the WID for NR MIMO phase 5 [1], [2] are approved.

Based on all above information, we provide our views on RRM requirement impact for UE-initiated/event-driven beam management in R19 NR MIMO WI. For all other parts, in our view more RAN1 input are needed.

Discussion on RRM requirements impact for UE-initiated /event-driven beam management

The WID scope for this is written as follows.

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

According to WID, the WI design is targeting FR2, but not precluding FR1. Moreover, both intra-cell scenario and inter-cell scenario are considered. Hence, we propose to specify RRM requirements at least for FR2 and covering both intra-cell and inter-cell cases.

Proposal 1  Specify RRM requirements for event-triggered L1 beam reporting for the serving cell and the cell with different PCI at least for FR2.

RAN1 has also agreed to support 2 Modes for the event-triggered beam reporting, i.e. Mode-A and Mode-B. The related agreements are cited as follows.

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, following modes are supported: - Mode A (dynamically scheduling UCI by gNB): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) to request a resource for a second UL channel to carry beam report - FFS: Request format, e.g., SR or a new UCI type. - Step 2: UE detects the DCI format to indicate a resource for a second UL channel to carry beam report. - Step 3: Beam report is transmitted in second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - This mode is basic UE capability (i.e. all UE supporting UE-initiated/event-driven beam reporting should support this feature). - No new DCI format is introduced. - Mode B (UCI in pre-configured resource(s) for second UL channel): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) notifying a second UL channel to carry beam report - FFS: Notification format, e.g., SR or a new UCI type. - Step 2: UE transmits the beam report in the second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - The notification in Step1 is in a separate reporting instance from the beam report in Step 2. FFS: Whether UE receives acknowledge information with response to each step for all modes For above procedures, cross-CC beam reporting is supported for both modes. - FFS: Details.

The main difference between these 2 Modes for UE initiated beam reporting (EVENT-TRIGGERED L1 REPORTING) is how UE reports the generated L1-RSRP/L1-SINR. As agreed in RAN1, Mode-A is the basic UE capability. In our view RRM requirements and test cases shall focus on Mode-A first. RAN1 has not yet come to the conclusions whether the one-bit/multi-bit PUCCH is SR or not. If it is SR, RAN4 may follow existing logic specified in 9.2.4 of TS 38.133 for defining delay requirements of event-triggered L1 reporting, i.e. RRM requirements are defined assuming UL resource for the reporting is always available. However, if multi-bit PUCCH is adopted in RAN1, it quite unlikely to be SR, and the latency for sending step-1 PUCCH must be counted in the delay requirements for the event-triggered L1-RSRP/L1-SINR reporting.

Proposal 2  RAN4 specifies RRM requirements at least for Mode-A event-triggered L1 reporting.

Proposal 3  To determine how to count latency for the uncertainty in acquiring UL resource for event-triggered L1 reporting, RAN4 waits further conclusions from RAN1 on whether the first PUCCH is SR or not.

RAN1 has also agreed on the resource type of event-triggered L1 reporting. The related agreements are cited below.

Agreement in RAN1 #116 On UE-initiated/event-driven beam reporting, at least support L1-RSRP as a measurement quantity on SSB for intra-cell and inter-cell, and periodic CSI-RS for beam management - Notes: measurement results may be contained in the beam report and/or used as quality metric(s) to initiate/trigger the reporting. - FFS: Semi-persistent CSI-RS and aperiodic CSI-RS. - FFS: Whether/how to support L1-SINR measurement, assuming legacy RS or RS combination (e.g., CMR only, CMR+ZP/NZP-IMR) for Rel-16 SINR is reused. - FFS: Whether/how to specify filtering operation for L1-RSRP. Agreement in RAN1 #116bis On UE-initiated/event-driven beam reporting, regarding trigger-event detection for beam reporting, at least support Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam. - At least L1-RSRP is supported as quality metrics used for Event-2 - FFS: How the L1-RSRP is used to determine the triggering event (e.g. timer, counter, filter coefficient) - FFS: Whether the network controls how the L1-RSRP is used to determine the triggering event - Regarding RS measurement for the new beam for Event-2, down-select one or more of the following: - Option-3a (explicit manner): The RS(s) for new beam(s) are explicitly configured by RRC (e.g., reusing legacy configuration of RS measurement or in TCI-State) or MAC-CE - Option-3b (implicit manner): The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of activated TCI state(s). - Option-3c (implicit manner): The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of configured TCI state(s). - Note-1: ‘New/current beam’ is for discussion purpose. - Note-2: Other trigger events/quality metrics (e.g., L1-SINR) are not precluded. - Note-3: For above implicit manner(s), if there are two QCL RSs in a TCI state, the measurement RS is derived from RS w.r.t. QCL-TypeD, if applicable. Agreement On UE-initiated/event-driven beam reporting, regarding Event-2, ‘current beam’ is a beam corresponding to the indicated TCI state. - Regarding RS measurement for the current beam for Event-2, Option-2a is supported: - Option-2a (implicit manner): The RS for current beam is implicitly derived from a QCL RS of indicated TCI state. - FFS: The RS for current beam can be either the QCL RS in the indicated TCI state or the SSB which is QCLed with the QCL RS in the indicated TCI state. - FFS: Option-2c (explicit manner): The RS for current beam is explicitly configured by RRC or MAC-CE. - Note: SSB or CSI-RS can be configured Agreement Regarding RS measurement for the current beam for Event 2, for Option-2a, besides for scheme-1 and scheme-2, further study the following for handling the case that only one TRS is configured in the indicated TCI state. - Option-1: Introducing additional scheme: the RS for current beam can be a CSI-RS for beam management derived from the QCL RS in the indicated TCI state; - Option-2: Further support TRS as measurement RS of current beam for determining L1-RSRP - Option-3: Introducing additional scheme: The RS for current beam is explicitly configured by RRC or MAC-CE (Option-2C in RAN1 116b agreement). - Option-4: No further enhancement (i.e., in such case, Scheme-2 is used) - Others are not precluded.

Both SSB and CSI-RS are agreed to be supported by RAN1. For CSI-RS, UE needs to at least support the CSI-RS that is configured as the QCL-D source of any TCI state. Note that as specified in TS 38.214, for unified TCI, QCL-D source RS of PDSCH/PDCCH can only be either TRS or CSI-RS with repetition on. As cited above, since TRS based L1 measurement is not yet concluded in RAN1, it is proposed to specify RRM requirements at least for SSB based L1 measurements and CSI-RS measurement with repetition on.

Proposal 4  For event-triggered L1 reporting, RRM requirements are specified at least for the following RS types:

- SSB of serving cell or cell with different PCI.

- CSI-RS with repetition on.

Regarding how UE will trigger the event, related RAN1 agreements are as follows.

Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification.

In our view, for beam management purposes, since the reported L1-RSRP/L1-SINR is one type of CSI which may also impact UL/DL scheduling, it would be OK if no filtering is applied before the reporting. Therefore, RAN4 shall focus on the basic UE feature. Moreover, RAN4 may also further discuss the impact to requirement applicability of TCI state switching in case of EVENT-TRIGGERED L1 REPORTING. For example, if UE reports for multiple times that a target TCI state is worse than a current TCI, (e.g. meeting the event-leaving condition), then if gNB still sends TCI state switching command indicating UE to switch to such target TCI, whether it could be considered as unknown TCI or an error case where RRM requirements are not applicable?

Proposal 5  RAN4 specify RRM requirements for event-triggered L1 reporting without time-window-based filtering or other filtering configured.

Proposal 6  RAN4 further discuss whether this is an error case where RRM requirements would be not applicable:

- A UE reports for N times that a target TCI state is worse than a current TCI, (e.g. meeting the event-leaving condition), but gNB still sends TCI state switching command indicating UE to switch to such target TCI. FFS value of N.

Conclusions

Based on above analysis, we have following proposals.

Proposal 1  Specify RRM requirements for event-triggered L1 beam reporting for the serving cell and the cell with different PCI at least for FR2.

Proposal 2  RAN4 specifies RRM requirements at least for Mode-A event-triggered L1 reporting.

Proposal 3  To determine how to count latency for the uncertainty in acquiring UL resource for event-triggered L1 reporting, RAN4 waits further conclusion from RAN1 on whether the first PUCCH is SR or not.

Proposal 4  For event-triggered L1 reporting, RRM requirements are specified at least for the following RS types:

- SSB of serving cell or cell with different PCI.

- CSI-RS with repetition on.

Proposal 5  RAN4 specify RRM requirements for event-triggered L1 reporting without time-window-based filtering or other filtering configured.

Proposal 6  RAN4 further discuss whether this is an error case where RRM requirements would be not applicable:

- A UE reports for N times that a target TCI state is worse than a current TCI, (e.g. meeting the event-leaving condition), but gNB still sends TCI state switching command indicating UE to switch to such target TCI. FFS value of N.

- References

[1]  RP-234007  New WID: NR MIMO Phase 5, Samsung.  RAN #102

[2]  RP-240087  WID revision: NR MIMO Phase 5, Samsung.  RAN #103