3GPP TSG-RAN WG4 Meeting #112 	R4-2412524

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	vivo

Title: 	Discussion on RRM requirement impacts for R19 NR Mobility Phase 4

Agenda Item:	8.23.2

Document for:	Discussion

Introduction

In RAN #103 and RAN # 104, the WID for R19 NR Mobility Phase 4 [1], [2] are approved.

Based on all above information, we provide our views on RRM requirement impact analysis.

Discussion

In [2], the measurement related enhancement is stated as follows.

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]

Moreover, in last two RAN2 meetings, the following agreements are achieved.

- Event triggered L1 measurement should be designed for the following LTM purposes: - Select the candidate beam/cell to trigger early synchronization. - Select the target beam/cell and trigger LTM cell switch procedure. - For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement. - Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events. - Event LTM2: Beam of serving cell becomes worse than absolute threshold; - Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell; - Event LTM4: Beam of candidate cell becomes better than absolute threshold; - Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2. - FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1. - Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5. - For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

RAN2 has already agreed to support both SSB based and CSI-RS based L1 measurement reporting for the event-triggered case, and same RS type will be assumed for LTM3 and LTM5. Therefore, RAN4 shall define RRM requirements for both of them.

However, for CSI-RS based L1 measurement and reporting on LTM candidate cells, RRM requirements for periodic reporting are not yet specified. The configuration of CSI-RS for an LTM candidate cell is already introduced in R18 LTM, and in our view RAN1 will not introduce additional mechanism in configuring NZP CSI-RS resources on top of existing R18 LTM. The remaining issue in RAN1/2 for this would be how to associate the cell and the CSI-RS resource to measure, while those will not impact much on RRM requirements. Hence RAN4 may start to discuss how to specify RRM requirements for R19 CSI-RS based L1 measurement.

On the other hand, for aperiodic reporting and semi-persistent reporting of LTM candidate cell, we are not yet sure whether the associated measurement RS can be also semi-persistent or aperiodic or not. Such measurement triggering across DU or CU would be challenging. Without supporting aperiodic or Hence, we would like to wait more progress from RAN1.

Proposal 1  RAN4 specify RRM requirements for CSI-RS based L1 measurement on candidate cell with periodic reporting in R19 mobility. RAN4 may start to discuss RRM requirements assuming R18 NZP CSI-RS configurations are re-used for R19 LTM candidate cells.

For SSB based L1 measurements, in R18 RAN4 had a long discussion on inter-frequency with gap. For CSI-RS based L1 measurements, in our view gap-based measurement is more difficult since it is only supposed to measure one cell at a time with the cost of interrupting current serving cell for several miliseconds every 40 or 80 ms. We do not think benefit of gap-based CSI-RS based measurement would match such huge cost. Hence, we think only CSI-RS based L1 measurement without gap shall be considered in R18.

Proposal 2  For CSI-RS based L1 measurement on candidate cell with periodic reporting, RRM requirements are specified for the following cases:

- CSI-RS based L1 measurement without gap

Another point is that according to WID, CSI-RS based L1 measurement is for BM purpose. For this issue, co-ordination with MIMO WI shall be done at plenary before RAN4 goes into too many details. But at least based on the WID we do not think RAN4 needs to consider any more about intra-f/inter-f definition. In case of cell switch, intra-f/inter-f definition based on SSB is already enough.

Proposal 3  For CSI-RS based L1 measurement, RAN4 not to introduce definition of intra-frequency/inter-frequency candidate cell measurement.

RAN1 and RAN2 has both discussed event-triggered L1 reporting, but in different WIs. In our view RRM requirements shall be specified. However, it is not yet clear how UE will trigger the event and how UE will generate the reports. Hence we think more RAN1/2 input on this part is needed.

Proposal 4  For L1 measurements on candidate cells with event-triggered reporting, RRM requirement are specified for both SSB-based and CSI-RS based L1 measurements. Further conclusions from RAN1/2 are needed before RAN4 start discussion on this part.

Conclusions

Based on above analysis, we have following proposals.

Proposal 1  RAN4 specify RRM requirements for CSI-RS based L1 measurement on candidate cell with periodic reporting in R19 mobility. RAN4 may start to discuss RRM requirements assuming R18 NZP CSI-RS configurations are re-used for R19 LTM candidate cells.

Proposal 2  For CSI-RS based L1 measurement on candidate cell with periodic reporting, RRM requirements are specified for the following cases:

- CSI-RS based L1 measurement without gap

Proposal 3  For CSI-RS based L1 measurement, RAN4 not to introduce definition of intra-frequency/inter-frequency candidate cell measurement.

Proposal 4  For L1 measurements on candidate cells with event-triggered reporting, RRM requirement are specified for both SSB-based and CSI-RS based L1 measurements. Further conclusions from RAN1/2 are needed before RAN4 start discussion on this part.

- References

[1]  RP-240299  Work Item: NR mobility enhancements Phase 4, Apple, China Telecom.  RANP #103

[2]  RP-241515  Revised Work Item: NR mobility enhancements Phase 4, Apple, China Telecom.  RAN4 #104