3GPP TSG-RAN WG4 Meeting #112 	R4-2412525

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	vivo

Title: 	Discussion on RRM requirement impacts for R19 NES enhancements

Agenda Item:	8.21.2

Document for:	Discussion

Introduction

In RANP #102 and #104, the WID for R19 NES [1], [2] are approved.

Based on all above information, we provide our views on RRM requirement impacts for R19 NES enhancements.

Discussion

<On RRM requirement impacts from On-demand SSB >

In [2], the following objective is listed.

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

The related RAN1 agreements for this bullet are cited below.

Agreements in RAN1 #116bis Agreement For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Case #1: No always-on SSB on the cell - Case #2: Always-on SSB is periodically transmitted on the cell - Scenario #2 refers to - SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321) - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed. Agreement - For SSB burst(s) indicated by on-demand SSB SCell operation, study at least the following options. - Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A. - Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB - Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B. - Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted. - Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B. - FFS: The combination of above options - FFS: How to define time instance A/B and the value of N per option - FFS: Each option is applicable to which Cases or Scenarios (as per the previous agreement) Agreement - For a cell supporting on-demand SSB SCell operation, - L1 and/or L3 measurement based on on-demand SSB is supported for the cell. - FFS further details on L1 and/or L3 measurement Agreements in RAN1 #117 Agreement - For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell. - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell. - This DCI signaling does not provide SCell activation/deactivation. - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc. - FFS: Scenarios where the above signalings are applicable Agreement - At least support L1 measurement based on on-demand SSB - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported. - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB

Based on RAN1 agreements, the following can be observed:

- OD-SSB can be used for SCell measurement before or after SCell activation. (As defined in Scenario #2 and #2A)

- OD-SSB can be used for SCell measurement on an SCell either with always-on SSB or not. (As defined in case #1 and #2)

- RRC or MAC CE has been agreed as triggering signaling for OD-SSB. DCI is still FFS.

- How to indicate termination of OD-SSB transmission is still FFS.

- L1 measurement and reporting has been agreed for OD-SSB. FFS it is only for scenario #2A or not.

- FFS L3 measurement for OD-SSB

Therefore, from RRM requirement impact perspective, we have the following proposals:

Proposal 1  Specify RRM requirements for OD-SSB based cell identification, i.e. SSB-based rough sync, and RSRP/SINR measurement, on an intra-f. layer for de-activated SCell and activated SCell.

Proposal 2  Specify RRM requirements for OD-SSB based SCell activation

For cell identification, normally searcher will be used. For an OD-SSB layer, where SSB based cell search is not always done once RRC configuration is provided, the searcher assumption would be much more complicated than legacy L3 measurements that semi-statically configured by MO. In our view RAN4 needs clarify the searcher assumption for the OD-SSB based rough sync. For example, whether UE count the OD-SSB frequency layer as one candidate in calculating CSSF when OD-SSB is activated, or when OD-SSB is configured, would need more discussion.

Proposal 3  RAN4 to discuss the searcher assumption of OS-SSB based cell identification, which would impact the CSSF of intra-frequency L3 or L1 measurements.

<On RRM requirement impacts from On-demand SIB1 >

In the WID, the following is listed.

- Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] - Triggering method by uplink wake-up-signal using an existing signal/channel. - Wake-up-signal configuration provisioning to UE - Note: No modification of SSB will be discussed under this objective - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. - Checkpoint for normative work in RAN#105

The details about uplink wake-up-signal is still being discussed in RAN1/2. RAN4 may further discuss timing requirements for such uplink transmission, if needed, after more RAN1/2 conclusions are achieved.

Proposal 4  For RRM requirements impact from on-demand SIB1 and the related uplink WUS desgin, RAN4 discuss the necessity of new RRM requirements after more conclusions from RAN1/2 is achieved.

<On RRM requirement impacts from SSB/PRACH adaptation >

In the WID, the following is listed.

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

So far, the RAN1 conclusion for SSB adaptation is like dynamic adjustment of SSB periodicity. Actually, RRM requirements for transition period, when UE switches between DRX and non-DRX, is already clear for all kinds of RRM requirements, e.g. RLM/BFD/CBD, L1-RSRP/L1-SINR measurement and L3 measurement. RAN4 may further check whether no additional impact is foreseen.

Proposal 5  For SSB periodicity adaptation, RAN4 further check whether it could be covered by existing transition period requirements for RLM/BFD/CBD, L1-RSRP/L1-SINR measurement and L3 measurement.

Conclusions

Based on above analysis, we have following proposals.

Proposal 1  Specify RRM requirements for OD-SSB based cell identification, i.e. SSB-based rough sync, and RSRP/SINR measurement, on an intra-f. layer for de-activated SCell and activated SCell.

Proposal 2  Specify RRM requirements for OD-SSB based SCell activation

Proposal 3  RAN4 to discuss the searcher assumption of OS-SSB based cell identification, which would impact the CSSF of intra-frequency L3 or L1 measurements.

Proposal 4  For RRM requirements impact from on-demand SIB1 and the related uplink WUS desgin, RAN4 discuss the necessity of new RRM requirements after more conclusions from RAN1/2 is achieved.

Proposal 5  For SSB periodicity adaptation, RAN4 further check whether it could be covered by existing transition period requirements for RLM/BFD/CBD, L1-RSRP/L1-SINR measurement and L3 measurement.

- References

[1]  RP-234065   New WID: Enhancements of network energy savings for NR, Ericsson.  RAN #102

[2]  RP-241650   Revised WID: Enhancements of network energy savings for NR, Ericsson, Apple.  RAN #104