# 3GPP TSG-RAN WG4 Meeting #112	R4-2412529

Maastricht, NETHERLANDS, 19th – 23th August, 2024

Agenda Item:	8.8.3.1

Source: 	Samsung

Title: 	System parameters for Less than 5MHz supporting in NTN

Document for:	Discussion

1		Introduction

For Rel-19 Enhanced requirements and conductive test methodology for NR NTN and IoT NTN WI, supporting 3MHz CHBW for NTN operation is one of major objective with detailed WID as following:

Less than 5MHz for NTN - Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands - Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Focus on power class 3 (PC3) for Tx RF requirements - Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary signalling to support the above objectives

In this contribution, we analyse the expected specification impact on supporting 3MHz CHBW from system parameter aspect.

2		Discussion

Channel bandwidth

3MHz CHBW with 15kHz SCS and corresponding new PBCH channel was introduced in Rel-18 for RMR system operation under specific TN operation bands.

In Rel-19, we can leverage the design on 3MHz CHBW from TN to NTN operation.

Proposal 1: Introduce 3MHz (15kHz) CHBW for NTN FR1 bands (n256, n255 and n254) and reuse Maximum transmission bandwidth configuration NRB and minimum guard band from TN specification for 3MHz (15kHz)

- Maximum transmission bandwidth configuration NRB :15RB

- Minimum Guard band: 142.5 kHz

- Applicable NTN bands: band n256, n255 and n254

Sync raster

In Rel-18, RAN1 introduced dedicated PBCH channel (12 RB) for 3MHz CHBW and corresponding sync raster was introduced in RAN4 for below 1GHz RMR operated bands.  To support new CHBW and dedicated PBCH channel, new channel raster with 600kHz step size and additional 300kHz offset (to maximize the offset between legacy sync raster and new 3MHz CHBW dedicated sync raster).

We can reuse the 3MHz CHBW sync raster design from TN specification and extend to 3GHz for supporting target FR1 NTN bands.

Proposal 2: Reusing TN  3MHz CHBW sync raster design and extending to 3GHz for FR1 NTN bands.

GSCN parameters for the global frequency for 3 MHz channel bandwidth

| Range of frequencies (MHz) | SS block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 0 – 3000 | N * 600 kHz + M * 50 kHz + 300 kHz, N = 1:1665, M ϵ {1,3,5} (Note 1) | 26638+3N + (M-3)/2 | 26640 – 41630 |
| NOTE 1: Only applicable for 15 PRB transmission bandwidth configuration within 3 MHz channel bandwidth with punctured PBCH defined in TS 38.211 [6] clause 7.4.3.1. |  |  |  |

Proposal 3: The detailed sync raster for target NTN bands on supporting 3MHz CHBW as following:

Applicable SS raster entries per operating band (FR1-NTN) for 3 MHz channel bandwidth

| NTN satellite operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 15 kHz | Case A | 37492 – <1> –37630 |
| n255 | 15 kHz | Case A | 34269 – <1> – 34424 |
| n254 | 15 kHz | Case A | 39060– <1> – 39129 |
| NOTE 1: SS Block pattern is defined in clause 4.1 in TS 38.213 [8]. |  |  |  |

3		Conclusion

In this contribution, we provide initial analysis on the system parameters impact to supporting 3MHz CHBW for FR1 NTN bands.

Proposal 1: Introduce 3MHz (15kHz) CHBW for NTN FR1 bands (n256, n255 and n254) and reuse Maximum transmission bandwidth configuration NRB and minimum guard band from TN specification for 3MHz (15kHz)

- Maximum transmission bandwidth configuration NRB :15RB

- Minimum Guard band: 142.5 kHz

- Applicable NTN bands: band n256, n255 and n254

Proposal 2: Reusing TN  3MHz CHBW sync raster design and extending to 3GHz for FR1 NTN bands.

GSCN parameters for the global frequency for 3 MHz channel bandwidth

| Range of frequencies (MHz) | SS block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 0 – 3000 | N * 600 kHz + M * 50 kHz + 300 kHz, N = 1:1665, M ϵ {1,3,5} (Note 1) | 26638+3N + (M-3)/2 | 26640 – 41630 |
| NOTE 1: Only applicable for 15 PRB transmission bandwidth configuration within 3 MHz channel bandwidth with punctured PBCH defined in TS 38.211 [6] clause 7.4.3.1. |  |  |  |

Proposal 3: The detailed sync raster for target NTN bands on supporting 3MHz CHBW as following:

Applicable SS raster entries per operating band (FR1-NTN) for 3 MHz channel bandwidth

| NTN satellite operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 15 kHz | Case A | 37492 – <1> –37630 |
| n255 | 15 kHz | Case A | 34269 – <1> – 34424 |
| n254 | 15 kHz | Case A | 39060– <1> – 39129 |
| NOTE 1: SS Block pattern is defined in clause 4.1 in TS 38.213 [8]. |  |  |  |

Reference

[1] RP-241281, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN, Samsung

[2] TS 38.101-1 v18.6.0

[3] TS 38.101-5 v18.6.0