# 3GPP TSG-RAN WG4 Meeting #112	R4-2412530

Maastricht, NETHERLANDS, 19th – 23th August, 2024

Agenda Item:	8.8.3.2

Source: 	Samsung

Title: 	UE RF requirements for Less than 5MHz supporting in NTN FR1 bands

Document for:	Discussion

1		Introduction

For Rel-19 Enhanced requirements and conductive test methodology for NR NTN and IoT NTN WI, supporting 3MHz CHBW for NTN operation is one of major objective with detailed WID as following:

Less than 5MHz for NTN - Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands - Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Focus on power class 3 (PC3) for Tx RF requirements - Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary signalling to support the above objectives

In this contribution, we analyse the expected specification impact on supporting 3MHz CHBW from UE RF requirements perspective.

2		Discussion

2.1 Section 6 Tx RF requirements

In below table we summarize the expected changes/update on TS 38.101-5 section 6 Tx RF requirements due to 3MHz CHBW introduction.

| Section | Expected changes/update |
| --- | --- |
| 6.2 Transmitted power 6.2.1 UE maximum output power 6.2.2 UE maximum output power reduction 6.2.3 UE additional maximum output power 6.2.4 Configured transmitted power | According to WID, target PC is PC3. For MPR, in TN specification 38.101-1, separate value specified for Pi/2 BPSK under 3MHz CHBW (note 3 in table 6.2.2-1). In NTN specification, MPR directly refer to TN specification. We believe same change applied for NTN on MPR table update. For regulation requirements and A-MPR (table), further work required for below NS values: - Band n254: NS_03N, NS_04N and NS_05N - Band n256: NS_24 - Band n255: NS_02N (no A-MPR required) |
| 6.3 Output power dynamics 6.3.1 Minimum output power 6.3.2 Transmit OFF power 6.3.3 Transmit ON/OFF time mask 6.3.4 Power control | 3MHz CHBW need to be added into section 6.3.1 and 6.3.2 without change on the requirements. |
| 6.4 Transmit signal quality | No update expected |
| 6.5 Output RF spectrum emission 6.5.1 Occupied bandwidth 6.5.2 Out of band emission 6.5.3 Spurious emission 6.5.4 Transmit intermodulation | For 6.5.1 Occupied bandwidth: - 3MHz CHBW need to added into table without change on requirement. For 6.5.2 out of band emission: - 3MHz CHBW need to be added into ACLR table without change on requirements - General emission mask needs to be update with 3MHz CHBW as similar changes for breaking point from TN TS 38.101-1 For 6.5.3 Spurious emission - FOOB need to be updated to add 3MHz CHBW with 6MHz boundary No update on 6.5.4 transmit intermodulation |

Proposal 1: For A-MPR and corresponding regulation requirements, further update and evaluation work required if 3MHz need to be applied:

  - Band n254: NS_03N, NS_04N and NS_05N

  - Band n256: NS_24

  - Band n255: NS_02N (no A-MPR required)

Proposal 2: 3MHz CHBW need to be added into section 6.3.1 and 6.3.2 without change on the requirements.

Proposal 3: For 6.5.1 Occupied bandwidth, 3MHz CHBW need to added into table without change on requirement.

Proposal 4: For 6.5.2 out of band emission:

  - 3MHz CHBW need to be added into ACLR table without change on requirements

  - General emission mask needs to be update with 3MHz CHBW as similar changes for breaking point from TN TS 38.101-1

Proposal 5: For 6.5.3 Spurious emission: FOOB need to be updated to add 3MHz CHBW with 6MHz boundary

2.2 Section 7 Rx RF requirements

In below table we summarize the expected changes/update on TS 38.101-5 section 6 Tx RF requirements due to 3MHz CHBW introduction.

| Section | Expected changes/update |
| --- | --- |
| 7.3 Reference sensitivity | New requirements for 3MHz need be specified for reference sensitivity on band n256, n255 and n254. - 2.2 dB scaling factor can be applied based on existing 5MHz requirements FRC can be refer to TN specification without update on TS 38.101-5. |
| 7.4 Maximum input level | 3MHz CHBW need to be added into table without change on the requirements. |
| 7.5 Adjacent channel selectivity | 3MHz CHBW need to be added into table without change on the requirements. - The interference signal configuration also need to be updated with CHBW 3MHz and offset. |
| 7.6 Blocking characteristics 7.6.2 In-band blocking 7.6.3 Out of band blocking 7.6.4 Narrow band blocking | For in-band blocking, similar change as TN specification, add 3MHz CHBW into table and interference frequency offset need to be updated. For out of band blocking, 3MHz CHBW need to be added into table without change on the requirements. For narrow band blocking, similar update as TN specification, 3MHz CHBW added into table with Pw as 18dBw. |
| 7.7 Spurious response | 3MHz CHBW need to be added into table without change on the requirements. |
| 7.8 Intermodulation characteristics | NTN specification directly refer to TN specification and update in TN specification for 3MHz CHBW also applicable. NO update on NTN specification expected. |
| 7.9 Spurious emissions | No update expected |
| FRC | NTN UE RF specification, FRC refer to TN specification and new 3MHz FRC already specified in TN specification which can be applied for NTN as well. No additional update on FRC expected. |

Proposal 6: For 7.3 Reference sensitivity, new requirements for 3MHz need be specified for reference sensitivity on band n256, n255 and n254:

  - 2.2 dB offset can be applied based on existing 5MHz requirements

Proposal 7: For 7.4 Maximum input level and 7.7 Spurious response: 3MHz CHBW need to be added into table without change on the requirements.

Proposal 8: For 7.5 ACS, 3MHz CHBW need to be added into table without change on the requirements.

  - The interference signal configuration also needs to be updated with CHBW 3MHz and offset.

Proposal 9: For 7.6 blocking requirements, following changes required:

  - For in-band blocking, add 3MHz CHBW into table and interference frequency offset need to be updated.

  - For out of band blocking, 3MHz CHBW need to be added into table without change on the requirements.

  - For narrow band blocking, 3MHz CHBW added into table with Pw as 18dBw.

3		Conclusion

In this contribution, brief summary on UE RF specification impact of introducing 3MHz CHBW was provided.

3.1 Section 6 Tx RF requirements

Proposal 1: For A-MPR and corresponding regulation requirements, further update and evaluation work required if 3MHz need to be applied:

  - Band n254: NS_03N, NS_04N and NS_05N

  - Band n256: NS_24

  - Band n255: NS_02N (no A-MPR required)

Proposal 2: 3MHz CHBW need to be added into section 6.3.1 and 6.3.2 without change on the requirements.

Proposal 3: For 6.5.1 Occupied bandwidth, 3MHz CHBW need to added into table without change on the requirement.

Proposal 4: For 6.5.2 out of band emission:

  - 3MHz CHBW need to be added into ACLR table without change on requirements

  - General emission mask needs to be update with 3MHz CHBW as similar changes for breaking point from TN TS 38.101-1

Proposal 5: For 6.5.3 Spurious emission: FOOB need to be updated to add 3MHz CHBW with 6MHz boundary.

3.2 Section 7 Rx RF requirements

Proposal 6: For 7.3 Reference sensitivity, new requirements for 3MHz need be specified for reference sensitivity on band n256, n255 and n254:

  - 2.2 dB offset can be applied based on existing 5MHz requirements

Proposal 7: For 7.4 Maximum input level and 7.7 Spurious response: 3MHz CHBW need to be added into table without change on the requirements.

Proposal 8: For 7.5 ACS, 3MHz CHBW need to be added into table without change on the requirements.

  - The interference signal configuration also needs to be updated with CHBW 3MHz and offset.

Proposal 9: For 7.6 blocking requirements, following changes required:

  - For in-band blocking, add 3MHz CHBW into table and interference frequency offset need to be updated.

  - For out of band blocking, 3MHz CHBW need to be added into table without change on the requirements.

  - For narrow band blocking, 3MHz CHBW added into table with Pw as 18dBw.

Reference

[1] RP-241281, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN, Samsung

[2] TS 38.101-1 v18.6.0

[3] TS 38.101-5 v18.6.0